3GPP TSG-RAN WG2 Meeting #130		R2-2504522

St.Julians, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on remaining issues on intermediate relay UE (re)selection and discovery

Agenda Item:		8.13.2

Document for:	Discussion and Decision

- Introduction

In RAN2 #129bis meeting, the discovery (mainly for model-B) and relay (re)selection (mainly for intermediate relay UE reselection behaviour) are further discussed and some agreements have been reached.

This contribution will further strive to solve the remaining issue on the following two aspects, including:

- Solve the FFS issue for discovery model-B, i.e., whether the intermediate relay UE always establishes itself as a remote UE first and thus no need to check PC5 AS condition;

- How to handle the notification message in intermediate relay UE (re)selection, e.g., when to send the notification and what is the behaviour of remote UE receiving this notification.

- Discussion

  - Discovery model-B

In RAN2 #129bis meeting, a bunch of agreements for discovery model-B have been reached[1]:

Agreements:

If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration.  If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages.

For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

For the FFS issue, the arguments from companies are that the intermediate relay UE may always need to establish its parent link as a remote UE first so the PC5 AS condition between intermediate relay UE and last relay UE does not need to be checked.

In SA2 specification[2], the multi-hop U2N discovery is specified in 6.3.2.5.2 (for model-A) and 6.3.2.5.3 (for model-B). For model-A, it is clear stated that ‘The 5G ProSe Intermediate UE-to-Network Relay shall establish a Layer-2 link with the parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay identified by the Announcer Info in the discovery entry before sending the 5G ProSe UE-to-Network Relay Discovery Announcement message.’. However, for model-B, there is no such requirement specified.

Observation 1: In SA2 TS 23.304, there is requirement for model-A that the intermediate relay UE should establish parent link before forwarding discovery message, but there is no similar description for model-B.

In our understanding, if the intermediate relay UE does not have any service, it is possible that the intermediate relay UE does not have any PC5 link established with last relay UE but is able to forward the solicitation messages. Then after the discovery model-B procedure finishes, the Layer-2 Link establishment procedure will be performed according to subclause ‘6.4.3.9.1 Layer-2 Link establishment and management’ in [2]. Therefore, we propose to confirm that the intermediate relay UE does not always establish itself as a remote UE first, and remove the FFS point.

Proposal 1: RAN2 to confirm that the intermediate relay UE is not required to always establish itself as a remote UE first in multi-hop model-B discovery, and remove the FFS point on AS condition checking.

  - Intermediate relay UE (re)selection

The (re)selection for intermediate relay UE has been discussed for two meetings, and the agreements are as follows:

#129 Agreements:

The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE.

Upon intermediate relay UE performing relay (re)selection, it can notify its child node.  If the cause of (re)selection already causes a notification, there is no double-trigger of the notification.

FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger)

FFS if the notification is required in all cases

FFS the content in the notification

FFS the child node behaviour after receiving this notification

FFS other condition/timing that the intermediate relay UE notifies its child node

#129bis Agreements:

Agreements:

Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected).

When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE).  FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment).  This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

FFS detailed information in the indication and child UE handling.

FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

With all the FFS issues, we would like to attempt to answer the following questions about the intermediate relay UE triggering (re)selection and help to form a whole picture of the whole procedure:

- Question 1: Whether the intermediate relay UE should notify the child node upon performing relay (re)selection?

- Question 2: If yes for Q1, when should the intermediate relay UE do the notification?

- Question 3: If yes for Q1, what should be included in the notification message?

- Question 4: What is the child node behaviour after receiving the notification?

We would like to use the Figure in SA2 specification[2] as the example:

Figure 6.4.3.8.3-2: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays with Root Relay change

According to SA2 specification[2], ‘During the transmition of re-selection indication to 5G ProSe Remote UE, after receiving the re-selection indication a 5G ProSe Intermediate UE-to-Network Relay in the path based on configuration may determine to relay the re-selection indication to 5G ProSe Remote UE, or perform relay reselection by itself as described above’.

Logically, to avoid so-called ‘double trigger’ in intermediate relay UE triggering re(selection), we share the same view in SA2 spec that the intermediate relay UE will either do the reselection itself first, or it should notify its child node at some time point. For the case mentioned in above FFS point ‘reselection under the same serving cell without changing the hop count’, we understand it is a valid case that the notification indication is not needed to be sent to child nodes.

For all other cases, e.g., the intermediate relay UE reselects to other cell, reselects to other intermediate/last relay UE with hop count changed, or there is no suitable candidate intermediate/last relay UE to be found, then the intermediate relay UE should transmit a notification message downstream. Therefore, we propose:

Proposal 2: When intermediate relay UE detects PC5 RLF or receives notification on other links (e.g., failure on Uu), it performs relay reselection and does not transmit a notification message downstream if the reselected path is under the same serving cell without changing the hop count. For all other cases, a notification message downstream will be transmitted.

If we agree on Proposal-2, there will not be ‘double trigger’, and it means that whenever the notification message is sent to remote UE, it should trigger the relay (re)selection at the remote UE, because now it means the parent link has been changed (e.g. to a different cell) and the remote UE should select its own path by itself. Therefore, in our understanding, there is no need for the remote UE to know the reason of PC5-RRC signaling from the first relay UE indicating relay (re)selection. The possible reason now can be that:

- Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

- Release of RRC connection between an intermediate relay UE and the network.

- PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

- (Re)selection is performed by intermediate relay UE but the intermediate relay UE e.g.  reselects to other cell, reselects to other intermediate/last relay UE with hop count changed, or there is no suitable candidate intermediate/last relay UE to be found.

Therefore, we can just have a simple notification message which is actually a relay (re)selection indication.

Moreover, there are some discussions about whether to use AS signalling or upper layer signalling, e.g. discovery message to serve as the notification message. In our understanding, the legacy discovery message cannot be reused because the intermediate relay UE may not find any suitable intermediate/last relay UE during the reselection. If other upper layer signalling is going to be considered, it is also not acceptable according to the fact that SA2 has already finished their work item.

Observation 2: Rely on upper layer signalling discovery for notification message is not workable when there is no suitable relay found during intermediate relay UE (re)selection.

Observation 3: Rely on upper layer signalling except discovery message for notification message introduce more interaction with SA2 and is not realistic as SA2 has finalized their specification and work item.

Therefore, it is preferable to use AS message NotificationMessageSidelink and the cause value in the message could reuse legacy cause value, e.g. PC5 RLF. As long as the remote UE receives the notification message, the remote UE should perform relay (re)selection.

Proposal 3: Reuse the AS signalling NotificationMessageSidelink and reuse one of the legacy cause values (e.g. PC5 RLF) to be included in it, for notification from intermediate relay UE to child nodes.

Proposal 4: Whenever receiving the notification message indicating relay (re)selection from first relay UE, the remote UE triggers the relay (re)selection.

- Conclusion

In this contribution, we discussed the discovery and intermediate relay UE (re)selection. We have the following Observations and Proposals:

For discovery:

Observation 1: In SA2 TS 23.304, there is requirement for model-A that the intermediate relay UE should establish parent link before forwarding discovery message, but there is no similar description for model-B.

Proposal 1: RAN2 to confirm that the intermediate relay UE is not required to always establish itself as a remote UE first in multi-hop model-B discovery, and remove the FFS point on AS condition checking.

For intermediate relay UE (re)selection:

Observation 2: Rely on upper layer signalling discovery for notification message is not workable when there is no suitable relay found during intermediate relay UE (re)selection.

Observation 3: Rely on upper layer signalling except discovery message for notification message introduce more interaction with SA2 and is not realistic as SA2 has finalized their specification and work item.

Proposal 2: When intermediate relay UE detects PC5 RLF or receives notification on other links (e.g., failure on Uu), it performs relay reselection and does not transmit a notification message downstream if the reselected path is under the same serving cell without changing the hop count. For all other cases, a notification message downstream will be transmitted.

Proposal 3: Reuse the AS signalling NotificationMessageSidelink and reuse one of the legacy cause values (e.g. PC5 RLF) to be included in it, for notification from intermediate relay UE to child nodes.

Proposal 4: Whenever receiving the notification message indicating relay (re)selection from first relay UE, the remote UE triggers the relay (re)selection.

- References

- RAN2 #129bis meeting chair notes;

- 3GPP TS 23.304 V19.3.0 (2025-03).