3GPP TSG RAN WG2 Meeting #130	 R2-2504531

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.4

Source: 	ITL

Title: 	Discussion on Conditional intra-CU LTM

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to conditional intra-CU LTM are as follows:

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In this contribution, we discussed the Conditional Intra-CU LTM.

Discussion

Cell and beam selection in L3 measurement based C-LTM

In this section, we show our view regarding candidate cell selection for Conditional LTM (C-LTM) when multiple candidate cells simultaneously satisfy the execution condition. In C-LTM procedures, it is possible that more than one candidate cell fulfills the configured execution condition at the same time. Given that C-LTM supports early uplink synchronization via pre-acquired TA, utilizing this information can lead to faster switching with reduced latency. Therefore, prioritizing cells for which the UE has valid TA is logical and efficient.

Therefore, we support prioritizing candidate cells with valid TA when multiple candidates fulfill the C-LTM execution condition. This approach enables RACH-less handover where applicable and optimizes the mobility performance by leveraging already synchronized uplink timing.

Key advantages of this approach is that enables low-latency C-LTM by utilizing pre-synchronized TA information.

In cases where no valid TA is available or multiple cells have valid TA, the UE should be allowed to select a candidate cell by implementation.

Proposal 1: For L3 measurement-based C-LTM, when multiple candidate cells fulfill the execution condition, the cell for which the UE has a valid Timing Advance should be prioritized; if no valid TA is available or multiple cells have valid TA, the UE may autonomously select the target cell based on implementation

In L3-based Conditional LTM, the execution condition is evaluated at the cell level based on consolidated measurements across multiple beams. Once the execution condition is met and a candidate cell is selected, the question arises of how to choose among the available beams within that cell to perform RACH-less handover.

Unlike L1-based events, where the triggering beam is directly known, the L3-based configuration may result in multiple beams satisfying the handover condition. Thus, a method to determine which beam to use for the RACH-less procedure is needed.

We support using a configured RSRP threshold (e.g., cg-RRC-RSRP-ThresholdSSB) as a filtering criterion to determine beam eligibility. Among the beams satisfying this threshold, we propose that the UE be allowed to autonomously select one for executing the RACH-less LTM procedure.

- Use of a configured RSRP threshold to identify beams eligible for RACH-less C-LTM. This threshold serves as a clear and consistent criterion for beam selection, aligned with mechanisms already applied in existing RACH-less procedures.

In particular, when multiple beams within the selected candidate cell satisfy the threshold, the UE may select one of them by implementation.

Proposal 2: In L3-based C-LTM, beam selection for RACH-less should be based on a configured RSRP threshold

Proposal 3: In L3-based C-LTM, if multiple beams within the selected candidate cell satisfy the configured RSRP threshold, the UE may select one of them by implementation

Conditional LTM failure recovery

In TS 38.300, for the Handover failure recovery, methods for not performing the RRC re-establishment procedure in certain cases are defined as follows:

- When DAPS handover fails, the UE falls back to the source cell configuration, resumes the connection with the source cell, and reports DAPS handover failure via the source without triggering RRC connection re-establishment if the source link has not been released.

- When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

- When LTM execution attempt triggered by LTM cell switch command MAC CE fails, the UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed.

In other words, after an LTM failure, if attemptLTM-Switch is configured and the selected cell is one of the LTM candidate cells, the UE can attempt the LTM cell switch again. And if attemptLTM-Switch is not configured and the selected cell is not one of the LTM candidate cells, the UE perform the RRC re-establishment procedure.

In RAN2#129, RAN2 agreed to support C-LTM fast recovery as follow:

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

According to TS 38.300, the UE may skip the RRC re-establishment procedure after HO or LTM failure under specific conditions. In Release 18, LTM failure recovery was limited to RACH-based LTM reattempts. In contrast, C-LTM in Release 19 introduces early TA for candidate cells, which can enable RACH-less C-LTM. Due to TA knowledge available in C-LTM candidate cells, RACH-less C-LTM can also be supported, improving recovery time and reducing signaling overhead.

Furthermore, it is our understanding that the cell selected by the UE after a C-LTM failure may be different from the originally attempted C-LTM candidate cell. In such a case, the TA associated with the newly selected cell may differ from the TA used in the original attempt. Therefore, the subsequent C-LTM attempt should not be considered a duplicate, but rather a new and valid execution attempt toward a different candidate cell.

Proposal 4: RAN2 is kindly asked to discuss introducing RACH-less based C-LTM fast recovery after C-LTM failure

Proposal 5: If the selected cell after C-LTM failure is not the originally attempted C-LTM candidate cell, the UE may attempt RACH-less C-LTM toward the newly selected cell, provided it is a configured candidate and valid TA is available

Conclusion

In this contribution, we discussed the conditional LTM for Intra-CU LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: For L3 measurement-based C-LTM, when multiple candidate cells fulfill the execution condition, the cell for which the UE has a valid Timing Advance should be prioritized; if no valid TA is available or multiple cells have valid TA, the UE may autonomously select the target cell based on implementation

Proposal 2: In L3-based C-LTM, beam selection for RACH-less should be based on a configured RSRP threshold

Proposal 3: In L3-based C-LTM, if multiple beams within the selected candidate cell satisfy the configured RSRP threshold, the UE may select one of them by implementation

Proposal 4: RAN2 is kindly asked to discuss introducing RACH-less based C-LTM attempt after C-LTM failure

Proposal 5: If the selected cell after C-LTM failure is not the originally attempted C-LTM candidate cell, the UE may attempt RACH-less C-LTM toward the newly selected cell, provided it is a configured candidate and valid TA is available

Reference

RAN2#129bis meeting report