# 3GPP TSG-WG2 Meeting #130                                        			            R2-2504555

St.Julians, Malta,  May 19th – 23rd , 2025

Source:                      Qualcomm Incorporated

Title:                          Open issues on LP-WUS operation in IDLE/Inactive state

Agenda Item:	8.4.2

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# Discussion

In RAN2#127 meeting, it has been agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

One issue is, if the entry/exit conditions for both of measurement types are configured, which one of should be used by the UE. In this case, it should allow the UE to select the measurement metrics the UE can support.

- It is up to UE implementation to choose which measurement type is used for entry/exit condition if both of SSB based measurement, OOK based LP-SS measurement types are configured.

From UE perspective, UE may have different capabilities to support SSB based measurement, OOK based LP-SS measurement. If the supported measurement type has not be configured entry/exit condition, it is up to UE implementation to determine the entry/exit condition.

- It is up to UE implementation to determine the entry/exit condition if there is no entry/exit condition configured for the supported measurement type.

## 2.3 eDRX support

In last RAN2 meeting, RAN2 discussed LP-WUS with eDRX and had the following agreement.

LP-WUS is supported with eDRX, FFS on exact impact if any

A UE configured with an eDRX cycle longer than 10.24s monitors the POs only within a Paging Time Window (PTW) configured to the UE. RAN4 specifies serving and neighbor cell measurements and cell-reselection criteria evaluation periods under the assumption that all measurements are performed within the PTW, even for the case when measurements are performed at a relaxed rate. With full offloading, things may look very different in RAN4 based on how frequently measurements are performed by the WUR. Furthermore, the power savings gain may not be substantial with long eDRX cycles, and in some cases may see negative power savings if the WUR is required to perform measurements more frequently. So, given that the LP-WUS operation with long eDRX cycles (>10.24s) comes with the increased complexity and workload across working groups and provide little to no power savings, we think it makes sense to limit the use of LP-WUS only to eDRX cycles up-to 10.24s. Also, RAN2 needs to send an LS to RAN4, since there is specification impact on RRM requirements with eDRX.

- LP-WUS is only used for eDRX with cycle no longer than 10.24s, and send LS to RAN4.

## 2.3 Emergency PDU session handling

In last RAN2 meeting, there is open issues on emergency PDU session as follows.

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In SA2 #167, 17-21 February 2025, meeting there were discussions based on [2] on whether it is necessary to disable paging mechanisms like PEI, eDRX for a UE with an active emergency PDU session. SA2 agreed to remove the emergency PDU session restrictions for Paging Early Indication and LP-WUS from Rel.19 version of the specifications, as PSAP callback is delivered as normal call in IMS PDU session. SA2 prepared CR S2-2502428 [3], for this agreement.

- RAN2 does not specify specific LP-WUS handling when the UE has an emergency PDU session.

# Conclusion

- LP-WUS mode control

- It is up to UE implementation to choose which measurement type is used for entry/exit condition if both of SSB based measurement, OOK based LP-SS measurement types conditions are configured.

- It is up to UE implementation to determine the entry/exit condition if there is no entry/exit condition configured for the supported measurement type.

eDRX support

- LP-WUS is only used for eDRX with cycle no longer than 10.24s, and send LS to RAN4.

Emergency PDU session handling

- RAN2 does not specify specific LP-WUS handling when the UE has an emergency PDU session.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2]    S2-2500133, Emergency call back and “delayed” paging, Qualcomm Incorporated, 3GPP TSG-SA         WG2#166-AdHoc-e

[3]   S2-2502428, 23.501 CR6083 (Rel-19, 'F'): Removing unnecessary restrictions for emergency PDU sessions	Qualcomm	Rel-19