3GPP TSG-RAN WG2 Meeting #130	R2-2504573

St.Julians, Malta, May. 19th– 23rd, 2025

Agenda Item:	8.2.2

Source:	OPPO

Title:	Discussion on paging procedure for Ambient IoT

Document for:	Discussion, Decision

Introduction

During RAN2#129~129bis, we have reached the following agreements on ways to capture Msg1 resource(s)

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

and paging ID length

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

In this discussion, we will delve further into paging and R2D trigger message, and eliminate ambiguities in current study.

Discussion

Subsequent paging for the same service

Transaction ID

At RAN2#129, it is agreed that

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

It is important to clarify that “generated by the reader based on the CN correlation ID” does not imply a one-to-one mapping between the transaction ID and the CN correlation ID. For example, different readers receiving the same CN correlation ID may generate different transaction IDs. This could introduce ambiguity regarding whether subsequent paging can use a transaction ID different from the initial paging. Given that the design of the transaction ID is service request-related, we believe using the same transaction ID within one paging round is natural. Therefore, we propose the following to remove this ambiguity:

Proposal 1: RAN2 confirms that subsequent paging for the same service has the same transaction ID as the initial paging.

Message type

When handling subsequent paging for the same service, an important consideration is to avoid duplicate responses from devices to the reader. There are three methods to prevent duplicate responses in subsequent paging:

- Way-1: Paging ID. Subsequent paging messages include the paging IDs of devices that have not successfully responded to the service request.

- Way-2: Message type. Subsequent paging messages have a different message type than the initial paging message, indicating that it is a follow-up message.

- Way-3: Transaction ID. Subsequent paging uses the same transaction ID as the initial paging. If a device receives the same transaction ID, it will not respond again.

For Way-1, this method only applies if paging IDs are included in the paging message. If no paging IDs are included (i.e., paging for all devices) in the initial paging message, reader would not know which device has missed the initial paging and could not attach its paging ID in the subsequent paging. Therefore, using only paging ID to differentiate is not enough.

For Way-2, if the initial and subsequent paging messages use the same transaction ID but different message types, it effectively becomes a variant of Way 3. Since Way 3 is sufficient to differentiate between initial and subsequent paging, there is no need to introduce differences in message types. If initial and subsequent paging use different transaction IDs and different message types, this could confuse devices. For instance, if a device has responded to Service Request 1 but missed the initial paging of Service Request 2, it might not recognize that a subsequent paging message belongs to Service Request 2 and would not respond accordingly.

Observation 1: Using the same transaction ID for both initial and subsequent paging ensures that devices can effectively avoid duplicate responses.

Given these considerations, it appears that using the same transaction ID is sufficient for devices to avoid duplicate responses. Therefore, we propose:

Proposal 2: Initial paging and subsequent paging share the same message type.

Missed NACK

At RAN2#129bis, we have agreed that

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

For CBRA, a NACK can be sent to devices after Msg3 or possibly after a D2R data message. There is a possibility that a device might miss the NACK. If this happens, the device may incorrectly assume that it has successfully completed the data transmission and omit subsequent paging for re-access. The reader would not realize the NACK was missed until it sends a subsequent paging and receives no response from the device. To address this issue, two methods have been proposed:

- Option 1: The subsequent paging message can include AS ID/ device ID of devices that missed NACK.

- Option 2: Reader can retransmit NACK to devices that failed to receive the initial NACK, followed by sending a second subsequent paging message for re-access.

For option 1, a detailed analysis is also provide in our paper [1]. When a NACK for Msg3 is missed, the device will not respond to the first subsequent paging message. The reader will then recognize that the device has not yet re-accessed and send a second subsequent paging message containing AS ID of the device. When a NACK for D2R data is missed, device ID can be used as the reader already obtained the information from Msg3.

For option 2, since the reader already knows the AS ID of the device that missed the NACK, it simply needs to resend an R2D message containing the NACK and then perform a subsequent paging. This method allows the message format for initial and subsequent paging to remain consistent. However, it will introduce additional signalling overhead compared to option 1.

Given the above discussion, we propose

Proposal 3: RAN2 to discuss which option is preferred for subsequent paging of devices that missed a NACK:

- Option 1: The subsequent paging message can include AS ID/ device ID of devices that missed NACK.

- Option 2: Reader can retransmit NACK to devices that failed to receive the initial NACK, followed by sending a second subsequent paging message for re-access.

For option 2, if either the retransmitted NACK or the subsequent paging message is missed again, both messages would need to be re-sent, which adds complexity to the re-access procedure. Therefore, we recommend option 1 as the preferred approach for addressing this issue.

Where is MSG1 resource

At RAN2#129bis, it is agreed that

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

During the discussion on CR 38.300, two different interpretations of these agreements emerged:

- Understanding 1: The A-IoT device randomly selects the access occasion number from the initial paging message and use this information to find its detailed MSG1 resources in R2D trigger message

- Understanding 2: The initial paging message includes the detailed MSG1 resources and the R2D trigger message indicates the selected MSG1 resource is available.

From our perspective, Understanding 1 is the correct interpretation. However, it is essential to clarify this point.

Proposal 4: RAN2 to clarify which understanding of the initial paging and R2D trigger messages is correct:

- Understanding 1: The A-IoT device randomly selects the access occasion number from the initial paging message and use this information to find its detailed MSG1 resources in R2D trigger message

- Understanding 2: The initial paging message includes the detailed MSG1 resources and the R2D trigger message indicates the selected MSG1 resource is available.

Indication of paging ID existence

At RAN2@129bis, it was agreed that paging ID length is not needed for the case where no ID is included.

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

This means that the header of paging messages without a paging ID would differ from those with a paging ID. To differentiate between these two types of paging messages, it is advisable to add a 1-bit indicator in the header to show whether a paging ID is included.

Proposal 5: Include 1 bit indicator in the header of A-IoT paging message to distinguish whether a paging ID is included.

Conclusion

In this contribution, we focus on paging procedure and have the following observations and proposals:

Subsequent paging for the same service: Transaction ID

Proposal 1: RAN2 confirms that subsequent paging for the same service has the same transaction ID as the initial paging.

Subsequent paging for the same service: Message type

Observation 1: Using the same transaction ID for both initial and subsequent paging ensures that devices can effectively avoid duplicate responses.

Proposal 2: Initial paging and subsequent paging share the same message type.

Subsequent paging for the same service: Missed NACK

Proposal 3: RAN2 to discuss which option is preferred for subsequent paging of devices that missed a NACK:

- Option 1: The subsequent paging message can include AS ID/ device ID of devices that missed NACK.

- Option 2: Reader can retransmit NACK to devices that failed to receive the initial NACK, followed by sending a second subsequent paging message for re-access.

Where is MSG1 resource

Proposal 4: RAN2 to clarify which understanding of the initial paging and R2D trigger messages is correct:

- Understanding 1: The A-IoT device randomly selects the access occasion number from the initial paging message and use this information to find its detailed MSG1 resources in R2D trigger message

- Understanding 2: The initial paging message includes the detailed MSG1 resources and the R2D trigger message indicates the selected MSG1 resource is available.

Indication of paging ID existence

Proposal 5: Include 1 bit indicator in the header of A-IoT paging message to distinguish whether a paging ID is included.

Reference

[1] R2-2503750, Discussion on AIoT data transmission related functionalities, OPPO