3GPP TSG-RAN WG2 Meeting #130			       R2-2504579

St.Julians, Malta, 19th May – 23th May, 2025

Agenda Item:	8.5.4 (Netw_Energy_NR-enh-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining issues on common signal and channel transmissions adaptation

Document for: Discussion and Decision

Introduction

In this contribution, we discuss the remaining issues of the common signal and channel transmissions adaptation.

Discussion

- 2.1 Adaptation of paging occasions in the time domain

Regarding the paging occasion adaptation, RAN2 has made the following agreements so far:

RAN2#129bis:

1.	For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

2.	For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

3.	Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

RAN2#129:

1.	For N, values smaller than T/32 are not supported.

2.	The maximum possible value for Ns is 8.

3.	Introduce a separate PEI configuration.

4.	Paging adaptations are configured semi-statically and updated via system information update notification.

5.	A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

Based on the agreements, we will discuss the remaining issues in detail.

Separate pei-ConfigBWP for paging adaptation

It is still FFS whether a separate pei-ConfigBWP for paging adaptation should be introduced. However, considering RAN2’s previous agreements related to PEI, as listed below, we think that the network should be able to support separate PEI transmission occasions for legacy PEI and paging adaptation PEI.

RAN2#129bis:

1.	For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

2.	For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

RAN2#129:

- Introduce a separate PEI configuration.

As the pei-ConfigBWP-r17 consists of pei-SearchSpace-r17 and firstPDCCH-MonitoringOccasionOfPEI-O-r17, separate PEI transmission occasions can be enabled by introducing separate pei-ConfigBWP for paging adaptation. The pei-SearchSpace specifies a search space set used to monitor the PDCCH for detection of DCI format 2_7 (i.e., the PEI message). The firstPDCCH-MonitoringOccasionOfPEI-O specifies the symbol-level offset from the start of the reference frame for PEI-O to the first PDCCH monitoring occasion on this BWP. In other words, pei-SearchSpace informs the UE of the candidate frequency and time domain resources for PEI transmission, while firstPDCCH-MonitoringOccasionOfPEI-O provides the specific time location of the PDCCH monitoring occasion for the PEI message reception. Since separate PEI transmission occasions can be enabled by configuring different pei-ConfigBWP configurations, we think that a separate pei-ConfigBWP for paging adaptation should be introduced.

Specifically, we think that both pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O should be introduced for paging adaptation in order to fully support the network’s flexible scheduling of PEI transmissions. Although it may seem that a firstPDCCH-MonitoringOccasionOfPEI-O is sufficient to enable separate PEI transmission, this only enables separation at the symbol level. A separate pei-SearchSpace allows the UE to receive PEI messages using different frequency domain resources and different time at the slot level. It also enables the UE to monitor PDCCH occasions of PEI with different periodicities, which is essential for paging adaptation, as the periodicity of PF associated with PEI for paging adaptation may differ from that of the Rel-17 paging configuration.

Proposal 1	Introduce a separate pei-ConfigBWP for paging adaptation, including both pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O.

RACH congestion occurrence due to consecutive POs

For Rel-19 paging adaptation, RAN2 agreed to increase the interval between PFs and increase the number of POs per PF to compensate for the reduced number of POs. As the number of POs associated with a single PF increase, some POs in a PF may be placed consecutively in time. In extreme cases, the network may place all the POs in a PF consecutively to further extend the network’s sleep time. As a result, RA triggered by paging may increase in specific ROs, potentially leading to RACH congestion.

In our understanding, simply increasing the number of ROs is not a feasible solution for RACH congestion caused by consecutive POs. This is because even if the network configures additional PRACH resources, if consecutive POs share the same nearest PRACH resource, there is a high probability that paged UEs will select the same PRACH resources. One possible solution is that the network provides an association between Rel-19 paged UEs and PRACH resources, allowing paged UEs to select different PRACH resources based on the association. By doing so, the network can distribute RA attempts triggered by paging across multiple PRACH resources, thereby reducing the risk of RACH congestion.

Proposal 2	Support association between Rel-19 paged UEs and PRACH resources to distribute RA attempts triggered by paging across multiple PRACH resources.

firstPDCCH-MonitoringOccasionOfPO for paging adaptations

Regarding the firstPDCCH-MonitoringOccasionOfPO configuration, the following enhancements have been proposed by other companies to reduce signaling overhead:

- Do not configure firstPDCCH-MonitoringOccasionOfPO for Rel-19 POs.

- Restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO.

- Configure evenly spaced POs by specifying only the starting position and interval, instead of explicitly signaling each PO occasion.

- Reduce the granularity of firstPDCCH-MonitoringOccasionOfPO by introducing a slot level offset instead of the current symbol-level offset.

However, these enhancements limit the network’s scheduling flexibility. Specifically, they reduce the network’s ability to flexibly place POs, which may impose constraints when the fine-grained or flexible PO offset configuration is required. In addition, since the IE firstPDCCH-MonitoringOccasionOfPO is optional, the network can simply choose not to configure it when signaling overhead is a concern or when the configuration is unnecessary. Based on these reasons, we prefer to configure firstPDCCH-MonitoringOccasionOfPO as in the legacy firstPDCCH-MonitoringOccasionOfPO.

Proposal 3	Configure firstPDCCH-MonitoringOccasionOfPO for paging adaptation in the same manner as in the legacy firstPDCCH-MonitoringOccasionOfPO.

- 2.2 Adaptation of PRACH in the time domain

Regarding configuration additional RACH resources, it is understood that the following parameters, which are configured in RACH-ConfigGeneric IE, can be separately configured for the additional RACH resources based on the related RAN1 agreements below

- PRACH configuration index

- msg1-FrequencyStart

- msg1-FDM

In addition, it is understood that the following parameters, which are configured in RACH-ConfigCommon IE, can be separately configured for the additional RACH resources based on the related RAN1 agreements below.

- number of SSB per RO

- CB-PreamblesPerSSB

R1-118b meeting Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details R1-119 meeting Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. R1-120 meeting Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported R1-120b meeting Agreement (Legacy RO) For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon. Agreement Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported - CB-PreamblesPerSSB

The configuration for additional RACH resources contains only essential parameters for increasing RACH resources in time and frequency domain. If the increased number of ROs is not consistent with the legacy parameter such as totalNumberOfRA-Preambles, it seems reasonable to separately configure the related parameters like CB-Preambles Per SSB and number of SSB per RO for consistency as discussed in RAN1.

Since the parameters for additional RACH resources can be separately configured and there exist the corresponding legacy parameters, the additional parameters can be put into the same IE configured for the corresponding legacy parameters. As discussed in RAN1, the additional RACH resources can be configured within the same RACH-ConfigCommon used to configure the corresponding legacy RACH resources. In detail, some parameters which are present in RACH-ConfigGeneric in RACH-ConfigCommon, they can be configured in RACH-ConfigGeneric in RACH-ConfigCommon.

Proposal 4	Additional RACH resources are configured in RACH-ConfigCommon IE, and only essential parameters such as PRACH configuration index, msg1-FrequencyStart, msg1-FDM, number of SSB per RO, and CB-PreamblesPerSSB are considered in the configuration.

Regarding RACH adaptation, RA-RNTI ambiguity issue has been raised. According to TS 38.321 MAC specification, the RA-RNTI is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

If additional RACH resources are configured and activated, it is not clear how to assign f_id to ROs from legacy RACH configuration and additional RACH configuration without confusion. In the above equation, f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8). The PRACH frequency resources are numbered in increasing order, starting from the lowest frequency according to TS 38.211.

For example, assuming legacy ROs and additional ROs overlap in time domain, a legacy UE and an NES UE may have the same frequency index value for different ROs (i.e., a legacy RO and a addition RO, respectively) if the frequency of the legacy RO is higher than the frequency of the additional ROs. In this case, for the legacy UE, legacy ROs are numbered in increasing order, starting from the lowest frequency. On the other hand, for the NES UE, both legacy ROs and additional ROs are numbered from the lowest frequency. Since the frequency of the additional ROs are lower than legacy ROs in this example, the legacy UE and NES UE have the same frequency index for different ROs, and they have the different frequency index for the same legacy ROs. Generally, depending on how to index ROs from legacy ROs and additional ROs, the different ROs may be indexed by the same frequency index value and vice versa. The ROs, which are different in frequency domain, may not be distinguished by the frequency index (f_id) and it may result in RA_RNTI ambiguity issue.

If the following three conditions are met, RA-RNTI ambiguity issue may be resolved without impact to the legacy UEs.

- ROs with different frequencies are indexed by different frequency index values (f_id).

- The existing RA-RNTI equation is used.

- Legacy UE and NES UE use the same f_id to indicate a legacy RO.

Please note that if the first bullet is satisfied, the second bullet is also satisfied.

RAN1 agreed that UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs). The range of the frequency index (0 ≤ f_id < 8) in the equation is enough for indexing 8 FDM-ed valid ROs.

Observation 1	RAN1 agreed that UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs). The range of f_id (0 ≤ f_id < 8) is enough for indexing 8 FDM-ed valid ROs.

Regarding f_id to ROs mapping, legacy UE will follow the legacy mechanism. If NES UE maps f_id to legacy ROs first according to the legacy mechanism and it maps f_id to additional ROs after f_id to legacy ROs mapping, NES UE assigns the same f_id to legacy ROs with legacy UE. NES UE assigns different f_id to additional ROs. Then, the above-mentioned three conditions are met and the RA-RNTI ambiguity issue can be resolved.

Proposal 5	Use the existing equation for RA-RNTI computation below for the case that legacy ROs and additional ROs overlap in time domain:
RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

Proposal 6	ROs are numbered from legacy ROs to additional ROs if legacy ROs and additional ROs overlap in time domain. i.e., the lower values of f_id are mapped to the legay ROs and the higher values of f_id are mapped to the additional ROs.

Conclusion

Regarding adaptation of paging occasions in the time domain

Proposal 1	Introduce a separate pei-ConfigBWP for paging adaptation, including both pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O.

Proposal 2	Support association between Rel-19 paged UEs and PRACH resources to distribute RA attempts triggered by paging across multiple PRACH resources.

Proposal 3	Configure firstPDCCH-MonitoringOccasionOfPO for paging adaptation in the same manner as in the legacy firstPDCCH-MonitoringOccasionOfPO.

Regarding adaptation of PRACH in the time domain

Proposal 4	Additional RACH resources are configured in RACH-ConfigCommon IE, and only essential parameters such as PRACH configuration index, msg1-FrequencyStart, msg1-FDM, number of SSB per RO, and CB-PreamblesPerSSB are considered in the configuration.

Observation 1	RAN1 agreed that UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs). The range of f_id (0 ≤ f_id < 8) is enough for indexing 8 FDM-ed valid ROs.

Proposal 5	Use the existing equation for RA-RNTI computation below for the case that legacy ROs and additional ROs overlap in time domain:
RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

Proposal 6	ROs are numbered from legacy ROs to additional ROs if legacy ROs and additional ROs overlap in time domain. i.e., the lower values of f_id are mapped to the legay ROs and the higher values of f_id are mapped to the additional ROs.

# 4. References