3GPP TSG-RAN2#130	R2-2504602

St. Julian, Malta, May 19th-23rd, 2025

Agenda item:		8.11.2

Source:		Lenovo

Title:		 Random Access in SBFD

Document for:		Discussion and Decision

- Introduction

The Work Item for Sub-band non-overlapping full duplex (SBFD) has been started in RAN2. The following agreements were made at the previous meetings [1]:

Agreements at RAN2 #129: - When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. - When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation and does not apply the SBFD RACH configuration. - For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. - If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. - FFS whether RO type selection is performed before or after the RA type selection. - FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported. Agreements at RAN2 #129bis: - NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. - Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. - RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

This contribution looks at some of the aspects related to Random Access for SBFD including configuration of RO type, integration of RO selection into the RACH procedure and the fallback behaviour.

- Discussion

During the CR Reviews, one open issue was identified whether Early UL Synchronization with an LTM Candidate Cell and/or RACH-based LTM should be supported on SBFD symbols. Some companies proposed that SBFD can be leveraged for Early UL Synchronization with an LTM Candidate Cell and RACH-based LTM in order to reduce the latency associated with handover. Since SBFD symbols are characterized by higher CLI, we think that the latency benefit may not always be achieved depending on the load in cell. Additionally, since LTM handover is not a latency sensitive use case, we do not see the motivation to have such optimization where the specification complexity may be high. Given that there is limited time remaining for the end of the release, it is suggested that Early UL Synchronization with an LTM Candidate Cell and RACH-based LTM are not supported on SBFD symbols.

(RRC-2) Early UL Synchronization with an LTM Candidate Cell and RACH-based LTM are not supported on SBFD symbols.

RAN2 has already agreed in previous meetings that the network can indicate a SBFD aware UE with a RO type for initial RA transmission in the case of CBRA. This means that a SBFD aware UE may always have to initiate RA based on the network indication regardless of its measured metrics. For example, a UE at cell edge with high pathloss may be forced to initiate RACH on a SBFD RO if such was indicated by the network. Some companies mentioned in the last meeting that the benefit of such indication would be to enable load balancing. However, in some cases, this may lead to (1) a reduced probability of RACH success, due to high pathloss and inter-gNB CLI, and, more importantly, (2) increase of inter-UE CLI at a nearby “victim” UE receiving a downlink signal/channel, due to the preamble uplink transmission. In our view, some criteria need to be additionally defined so that the load balancing can be efficiently achieved without impacting the UE’s success probability for RACH or causing high inter-UE CLI at a nearby downlink UE. One approach is to allow UE to select an additional RO type if the measured SSB RSRP is above an SSB RSRP threshold, which would only allow UEs with good/high link budget (or low pathloss) to perform RA on an additional RO.

If the network indicates additional RO for initial RA in the case of CBRA, SSB RSRP threshold is configured as an additional criterion.

To limit impact of inter-UE CLI, one approach would be to indicate/configure a different SSB RSRP threshold for each SSB or a group of SSBs. For example, a high SSB RSRP threshold can be indicated/configured for an SSB (or a group of SSBs) providing coverage for a hotspot area (e.g., a shopping mall or an indoor office). Such an approach will allow only a UE with a very good/high link budget (or very low pathloss) to select additional RO type. In such case, the UE would use a low PRACH power that incur/cause low inter-UE CLI. On the other hand, a low SSB RSRP threshold can be indicated/configured for an SSB (or a group of SSBs) providing coverage for a non-hotspot area, which would allow more UEs to select additional RO type, eventually achieving a desired load balancing between additional RO type and legacy RO type.

RAN2 to agree that a different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs to limit inter-UE CLI and achieve a better load balancing between additional RO type and legacy RO type.

- Conclusion

This document discusses some aspects of Random Access for SBFD. Following are the proposals and observations made in this document:

- Proposal 1: (RRC-2) Early UL Synchronization with an LTM Candidate Cell and RACH-based LTM are not supported on SBFD symbols.

- Proposal 2: If the network indicates additional RO for initial RA in the case of CBRA, SSB RSRP threshold is configured as an additional criterion.

- Proposal 3: RAN2 to agree that a different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs to limit inter-UE CLI and achieve a better load balancing between additional RO type and legacy RO type.

- References

[1] R2-2501332: MIMO, LP-WUS, SBFD Chair’s Notes