3GPP TSG-RAN WG2 Meeting #130	R2-2504619

St Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.7.4.1

Source:	Samsung

Title:	Finalising LCP design for XR Ph3

Document for:	Discussion & Decision

# Introduction

In the RAN2#128 meeting (November 2024), RAN2 agreed the following for LCP enhancements for Rel-19 XR:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

At the subsequent RAN2 meeting (RAN2#129, February 2025), the following was additionally agreed:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

And finally, at the most recent RAN2 meeting (RAN2#129-bis, April 2025), the following agreements were made:

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

In this paper, we discuss the key remaining issues of the operations related with the additional LCH priority, and the impact of agreements made. We look at the details of SR priority determination for intra-UE prioritization and its specification impact, UE behaviour when Bj is negative, enhancements to MAC PDU assembly, as well as handling of other priority traffic e.g. SRBs in light of the additional LCH priority for XR traffic.

# Discussion

## The intra-UE prioritization case and the additional LCH Priority: additional spec impact for the SR case

For legacy intra-UE prioritization, when more than one UL grant(s) and/or SR(s) overlap in time domain, a priority-based comparison among the overlapped UL grant(s) and/or SR(s) is conducted to determine which UL grant or SR should be transmitted. The priority-based comparison was based on legacy (default) LCH priority, i.e., the priority of UL grant is defined as the highest (legacy) priority of LCHs multiplexed or to be multiplexed in the corresponding MAC PDU, and the priority of SR is defined as the (legacy) priority of the LCH triggering the SR. RAN2 agreed that in Release-19 the intra-UE prioritization may also use the additional LCP priority for UL grant priority determination. For SR priority determination during intra-UE prioritization, we agreed that we would look at the spec impact before confirming this.

Priority of an SR is the priority of the LCH that triggered the BSR/DSR which could not be sent (and therefore triggered said SR). The question is whether something could happen to make this priority no longer relevant (when determining whether to send this SR or a grant that clashes with it). It is possible that – by the time of the MAC PDU transmission – data in this LCH has ‘expired’ and the SR no longer needs to be sent. It is also possible that data linked to the grant which clashes with the SR has expired. Therefore we propose for RAN2 to specify whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority at the time of transmission.

Proposal 1a. For the case of intra-UE overlapping resources prioritization linked to Scheduling Request triggered by logical channels that have LCH priority-adjusted data to be transmitted, RAN2 to specify (using the TP in R2-2503690 as baseline) whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority at the time of transmission.

One additional consideration is whether we treat non-BSR (and non-DSR) triggered SR differently.

Proposal 1b. For the case of intra-UE overlapping resources prioritization linked to Scheduling Request, RAN2 to discuss whether to capture the cause of SR triggering in the specifications i.e. make it clear that SR in question was triggered according to the BSR/DSR procedures (as the additional priority would only be relevant in this case).

## Case of an LCH with an upgraded priority with negative Bj

RAN2’s current Working Assumption is that no Bj enhancement is introduced. Currently, the LCHs are served in the first round of LCP procedure only if corresponding Bj>0. If corresponding Bj becomes negative, it will in any case be incremented by PBR × T before every instance of the LCP procedure. Therefore we do not foresee a case where Bj would stay negative for long enough to have any substantial impact; the only case could be where PBR is set very low (and it is unclear why the network would do that for XR data), or if T is very short i.e. if the LCH in question is selected for MAC PDU assembly very frequently – but if this happens, presumably this channel has high priority and will hence be served in 2nd rounds of these ostensibly very frequent LCP procedures. As a result, we do not envisage a use case for Bj becoming negative – and staying negative – for too long for XR data.

Proposal 2. RAN2 to confirm the Working Assumption of not allowing an LCH with an upgraded priority to be transmitted in the 1st round of LCP when Bj is negative i.e. stick to legacy behaviour.

## Data prioritization over comparatively less urgent MAC CEs

We now turn our attention to the potential use-case of prioritization of data over MAC CEs, which for LCHs with priority-adjusted data, is potentially very relevant as the in-time delivery is important. So far, dedicated configured grant or resource separated by LCP have been introduced over multiple releases of NR to aid in-time delivery. In Rel-18, multiple CG occasions, DSR reporting and discard mechanism were introduced to support in-time delivery with higher resource efficiency. However, most of the MAC CEs, e.g., BSR/DSR/PHR, etc., still have higher absolute priorities over any of XR data, and those MAC CEs can use any type of uplink grants. This could make unexpected interruption during LCP and MAC PDU generation, resulting in LCH priority-adjusted data being delayed due to such MAC CEs. Therefore, we see value in data prioritization over some less important/urgent MAC CEs, and propose the following:

Proposal 3. RAN2 to consider prioritizing LCH with LCH priority-adjusted data over comparatively less important/urgent MAC CEs.

As an illustration of some additional underlying issues, and using the BSR MAC CE as an example, a BSR is triggered and LCH#1 uses new/adjusted priority P_a as it contains SDUs close to expiry. By the time MAC PDU assembly starts, data arrives into LCH#2 which has priority P_b1 > P_a (and this was not the case when BSR was triggered, where LCH#2 had a priority P_b<P_a). If we now want to compare priority of the BSR MAC CE with that of data (in legacy systems we would prioritize the BSR MAC CE, but for XR data we may wish to prioritize data), it is unclear whether we should compare P_b and P_a, or P_b1 and P_a.

Observation 1: Re-calculating LCH priorities at time of MAC PDU assembly relative to priorities at the time of BSR triggering could cause issues in cases where we want to prioritize data over a BSR MAC CE.

Proposal 4. Assuming RAN2 agrees to prioritizing LCH with LCH priority-adjusted data over BSR MAC CEs where beneficial, when determining the priority of the BSR MAC CE relative to data in order to compare it with the priority of the data, the priority should be that of the logical channel that resulted in the most recent BSR trigger prior to MAC PDU assembly, and whose data cannot all be included in the MAC PDU.

## Handling of SRBs

In the past meetings, certain submissions discussed the perceived impact of the additional LCH priority on timely transmission of signaling data. In our view, the handling of this case is best left to network implementation:

Proposal 5. Handling of any concerns to do with XR traffic potentially delaying SRBs traffic is left to network implementation.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1a. For the case of intra-UE overlapping resources prioritization linked to Scheduling Request triggered by logical channels that have LCH priority-adjusted data to be transmitted, RAN2 to specify (using the TP in R2-2503690 as baseline) whether the priority of the LCH which triggered the SR used for the intra-UE prioritization is that from the point in time when the SR was triggered, or the current priority at the time of transmission.

Proposal 1b. For the case of intra-UE overlapping resources prioritization linked to Scheduling Request, RAN2 to discuss whether to capture the cause of SR triggering in the specifications i.e. make it clear that SR in question was triggered according to the BSR/DSR procedures (as the additional priority would only be relevant in this case).

Proposal 2. RAN2 to confirm the Working Assumption of not allowing an LCH with an upgraded priority to be transmitted in the 1st round of LCP when Bj is negative i.e. stick to legacy behaviour.

Proposal 3. RAN2 to consider prioritizing LCH with LCH priority-adjusted data over comparatively less important/urgent MAC CEs.

Proposal 4. Assuming RAN2 agrees to prioritizing LCH with LCH priority-adjusted data over BSR MAC CEs where beneficial, when determining the priority of the BSR MAC CE relative to data in order to compare it with the priority of the data, the priority should be that of the logical channel that resulted in the most recent BSR trigger prior to MAC PDU assembly, and whose data cannot all be included in the MAC PDU.

Proposal 5. Handling of any concerns to do with XR traffic potentially delaying SRBs traffic is left to network implementation.