3GPP TSG-RAN WG2 Meeting #130	R2-2504626

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:        ZTE Corporation

Title:           Consideration on the Remaining Issues of UEIBM

Agenda item:   8.12.3

Document for:  Discussion and Decision

- Introduction

According to the post email discussion [Post129bis][215][MIMO_Ph5] Running CR for TS 38.321 (Samsung), the following issues on the UEIBM need to be further discussed.

Issue 1: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), discuss whether to generate MAC PDU for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH. Issue 2: Discuss UE behaviour when UE-initiated report is triggered but there is no valid PUCCH/PUSCH resource to transmit UE-initiated report (e.g., PUCCH resource is released by RRC and type-1 CG is cleared in MAC due to TAT expired). Issue 3: Discuss whether the same rule of CSI report transmission is applied to UEI report (e.g., for activated/deactivated SCell/SCG, handling of measurement gaps, handling of FR2 UL gaps, BWP operation, cell DRX operation).

Meanwhile, there are also some remaining issues/EDs on this topic in the post email discussion “[Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson)” as below:

| - Topic | - Issue | - Remarks |
| --- | --- | --- |
| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | To be discussed in contributions, but we should wait once the UEI BMR parameters are more stable. |
|  | Editor’s note: The description of CSI-ReportConfig IE may have further updates. | This could be handled during a round of updates for 38.331 based on companies input. |
|  | Editor’s note: Once the full list of parameters is provided, CSI-ReportUE-IBM fields may be restructured to account for field dependencies in ASN.1 (e.g. by embedding valueOfQ in eventType and having a new reportConfigType). | This could be handled during a round of updates for 38.331 once the UEI BMR parameters are more stable. There were suggestions from companies during [Post129bis][214][ MIMO_Ph5] (Vivo Issue1 and ZTE- Issue 1). So even though we may only do these changes later, those suggestions could be considered as the baseline. Therefore, if one has different suggestions and/or more details to provide, it could be good to see those already now in company contributions. |
|  | Editor’s note: The fields ul-BWP-Id, servCellIndex and periodicityAndOffset may have further updates. | To be discussed in contributions, but we should wait once the UEI BMR parameters are more stable. |

In this paper we would like to share our views on these remaining issues.

- Discussion

Based the above remaining issues and EDs, there are 3 main topics:

- Topic 1: Whether the same rule of CSI report transmission is applied to UEI report (including the Issue 1 on UL skipping and Issue 3 for activated/deactivated SCell/SCG, handling of measurement gaps, handling of FR2 UL gaps, BWP operation, cell DRX operation)

- Topic 2: No PUCCH/PUSCH Resource for the UEIBR

- Topic 3: UEIBM RRC Parameters Structure

In the next chapter we’d like to discuss these 3 topics one by one.

2.1 Same Rule as Legacy CSI report or not

In this chapter would focus on the Topic 1 which include both the remaining Issue 1 on UL skipping and Issue 3 for activated/deactivated SCell/SCG, handling of measurement gaps, handling of FR2 UL gaps, BWP operation, and cell DRX operation. Considering that the UEI mode A CSI report is similar to the legacy aperiodic CSI Report and there are some special processing for the legacy aperiodic CSI report, we’d like to discuss the UEI mode A CSI report and UEI mode B CSI report separately.

UEI mode A CSI Report

For the UEI mode A CSI report, as one company has pointed in the post email discussion, RAN1 has agreed to Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report. From the MAC layer spec aspect, the UEI mode A CSI report is similar to the legacy aperiodic CSI report.

Observation 1: For the UEI mode A CSI report, RAN1 has agreed to reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report and thus the UEI mode A CSI report is similar to the legacy aperiodic CSI report.

For the aperiodic CSI report, some special processing is included as below in the MAC spec for the UL skipping, DRX and CSI reporting on the dormant BWP.

##### 5.4.3.1.3 Allocation of resources The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and … 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and …

## 5.7 Discontinuous Reception (DRX) Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

## 5.15.1 Downlink and Uplink 1> if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP: 2> stop the bwp-InactivityTimer of this Serving Cell, if running. 2> not monitor the PDCCH on the BWP; 2> not monitor the PDCCH for the BWP; 2> not receive DL-SCH on the BWP; 2> not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

Obviously, the same principle shall be adopted for the UEI mode A CSI report.

Proposal 1: The same principle as the aperiodic CSI report for the UL skipping, DRX and dormant BWP shall be adopted for the UEI mode A CSI report.

On whether and how to capture this proposal in the MAC spec, generally there are 3 options:

- Option 1: Add UEI mode A CSI report to every MAC spec chapter (e.g. 5.4/5/7/5/15) that includes aperiodic CSI report;

- Option 2: Add a General Note/Description to MAC spec, e.g. the UE’s behaviour for the UEI mode A CSI report is the same as the aperiodic CSI report;

- Option 3: No MAC spec impact, and take the UEI mode A CSI report as one kind of the aperiodic CSI report.

Proposal 2: Ran2 to discuss the following 3 options on how to capture the proposal 1 in MAC spec

- Option 1: Add UEI mode A CSI report to every MAC spec chapter (e.g. 5.4/5.7/5.15) that includes aperiodic CSI report;

- Option 2: Add a General Note/Description to MAC spec, e.g. the UE’s behaviour for the UEI mode A CSI report is the same as the aperiodic CSI report;

- Option 3: No MAC spec impact, and take the UEI mode A CSI report as one kind of the aperiodic CSI report.

Obviously, the option 2 is slightly preferred to keep the forward compatibility and reduce the spec impact. Furthermore, as described in the remaining issue 3, the CSI report was included in many chapters, such as:

- Chapter 5.9 on activated/deactivated SCell/SCG,

- Chapter 5.14 on measurement Gaps

- Chapter 5.15.1 on BWP operation

- Chapter 5.29 on activation/Deactivation of SCG

- Chapter 5.30 on FR2 UL gap

If we go to the option 1, we may also need to check the spec impact on the above chapters.

Proposal 2a: On how to capture the proposal 1 in MAC spec, select Option 2 as baseline.

UEI mode B CSI report

The UEI mode B CSI report is quite similar to the legacy semi-persistent CSI report on the PUSCH, thus the similar principle to the legacy semi-persistent CSI report on the PUSCH can be taken as baseline, then based on this baseline RAN2 can further discuss some exception cases.

Proposal 3: From MAC layer aspect, the similar principle to the legacy semi-persistent CSI report on the PUSCH can be taken as baseline for the UEI mode B CSI report and RAN2 can further discuss some exception cases.

Based on the proposal 3, we go through the above mentioned chapters in the Issue 3(e.g. 5.9, 5.14, 5.15.1, 5.29 5.30), there is one exceptional case on the dormant BWP.

1> if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP: 2> stop the bwp-InactivityTimer of this Serving Cell, if running. 2> not monitor the PDCCH on the BWP; 2> not monitor the PDCCH for the BWP; 2> not receive DL-SCH on the BWP; 2> not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

On this topic, in the last meeting, it has been agreed that

If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP.

However it’s not clear on whether it’s allowed the UE to perform mode-B beam reporting for this BWP. During the post email discussion, there are 2 different understandings:

- Understanding 1: The UE can perform mode-B beam reporting for this BWP.

- Understanding 2: The UE can’t perform mode-B beam reporting for this BWP.

If we take the legacy semi-persistent CSI report behaviour, it means the UE can report it for this dormant BWP, e.g. report it on the SpCell. However, for the UEI mode B CSI report for the dormant BWP, the question is how to determine the current beam. Even in the last RAN1 meeting a new parameter was introduced for some special cases, it’s still hard to determine the current beam for the dormant BWP when this new parameter is absent.

| CCofIndicatedTCI-r19 | New | This parameter is used to indicate a CC index of the serving cell on which the indicated TCI state used to determine the current beam RS is applied | ServCellIndex |
| --- | --- | --- | --- |

Based on the above analysis, the understanding 2 is preferred for the UEI mode B CSI report.

Proposal 3a: As an exceptional case, if the BWP in a SCell is a dormant BWP, the UE can’t perform UEI mode B CSI report for this BWP.

2.2 No PUCCH/PUSCH Resource

The second remaining issue is to discuss UE behaviour when UE-initiated report is triggered but there is no valid PUCCH/PUSCH resource to transmit UE-initiated report (e.g., PUCCH resource is released by RRC and type-1 CG is cleared in MAC due to TAT expired).

For this case the RACH procedure can be triggered, however we don’t think this would introduce any spec impact for that according to the current spec, when TAT is expired, the RACH would be triggered to get UL synchronized.

Proposal 4: When UE-initiated report is triggered but there is no valid PUCCH/PUSCH resource to transmit UE-initiated report, the RACH procedure can be triggered without MAC spec impact/or enhancement.

2.3 Asn.1 Structure

During the RRC running CR review, the below issues and the EDs were listed for the UEIBM topic.

| FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | To be discussed in contributions, but we should wait once the UEI BMR parameters are more stable. |
| --- | --- |
| Editor’s note: The description of CSI-ReportConfig IE may have further updates. | This could be handled during a round of updates for 38.331 based on companies input. |
| Editor’s note: Once the full list of parameters is provided, CSI-ReportUE-IBM fields may be restructured to account for field dependencies in ASN.1 (e.g. by embedding valueOfQ in eventType and having a new reportConfigType). | This could be handled during a round of updates for 38.331 once the UEI BMR parameters are more stable. There were suggestions from companies during [Post129bis][214][ MIMO_Ph5] (Vivo Issue1 and ZTE- Issue 1). So even though we may only do these changes later, those suggestions could be considered as the baseline. Therefore, if one has different suggestions and/or more details to provide, it could be good to see those already now in company contributions. |
| Editor’s note: The fields ul-BWP-Id, servCellIndex and periodicityAndOffset may have further updates. | To be discussed in contributions, but we should wait once the UEI BMR parameters are more stable. |

Furthermore, the RAN1 parameters tend to be stable with few FFS on the detail value range, thus we can start the discussion on the ASN.1 structure.

| reportTransmissionMode-r19 | New | This parameter is used to inform one report transmission mode on UCI based beam report procedure for UE-initated/event-driven beam reporting. - ModeA: UCI in a dynamically scheduled PUSCH - ModeB: UCI in a pre-configured UL resource(s) | {ModeA, ModeB} |
| --- | --- | --- | --- |
| eventThreshold-r19 | New | This parameter is used to inform a threshold value for trigger event detection regarding Event-2 or Event-7. | FFS |
| eventThreshold-Event1-r19 | New | This parameter is used to inform a threshold value for trigger event detection regarding Event-1. | FFS |
| enabledCurrentBeamReport-r19 | New | This parameter is to enable or disable whether current beam is always reported. - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported l - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported. | Up to RAN2 |
| nrofReportedRS-UEIBR-r19 | New | This parameter is used to inform the number of reported RS(s) in a report format for UE-initated/event-driven beam reporting. | 1,2,3,4 |
| newBeamResourceSet-r19 | New or existing | This parameter is used to configure the RS resource set for new beam(s) for Event 1, Event 2, or Event 7 | Up to RAN2 |
| eventType-r19 | New | This parameter is used to inform a event type for UE-initated/event-driven beam reporting. - Event-1:Quality of the current beam in L1-RSRP is worse than a certain threshold. - Event-2: Quality of at least one new beam, in L1-RSRP, becomes a threshold value better than the current beam. - Event-7: Quality of at least one new beam, in L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. | {Event1, Event2, Event7} |
| valueOfQ-r19 | New | This parameter is used to inform the value of Q for Event-7 - Event-7: Quality of at least one new beam， in L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. | 1, 2, 3, 4, 5, 6, 7, 8 |
| configuredPUSCHResourceOfModeB-r19 | New | This parameter is used to inform Type-1 CG PUSCH resource for second channel in mode-B | configuredGrantConfigIndex, UL bwp-Id, serving cell index |
| PUCCHResource -r19 | New | This parameter is used to configure the periodic PUCCH resource for first PUCCH - to request dynamically scheduled PUSCH to carry UE-initated/event-driven beam report for mode-A - to notify Type-1 CG PUSCH to carry UE-initated/event-driven beam report for mode-B. | periodicityAndOffset, PUCCH-ResourceID |
| numOfSymbols-ModeB-r19 | New | This parameter is used to configure time offset in number of symbols for determining available transmission occasion of second PUSCH in Mode-B from the first PUCCH | FFS |
| carrier | Existing | This parameter is used to indicate in which serving cell where the new beam measurement resources for UE-initiated/Event-driven beam report are to be found. | ServCellIndex |
| CCofIndicatedTCI-r19 | New | This parameter is used to indicate a CC index of the serving cell on which the indicated TCI state used to determine the current beam RS is applied | ServCellIndex |

Observation 2: The RAN1 parameters tend to be stable with few FFS on the detail value range, thus RAN2 can start the discussion on the ASN.1 structure.

The above parameters can be further divided into 2 types:

- Type 1: Common Parameters that would be configured by all of the Event types e.g. enabledCurrentBeamReport-r19/ nrofReportedRS-UEIBR-r19/ newBeamResourceSet-r19/ Carrier/ CCofIndicatedTCI-r19/ PUCCHResourcer19/ reportTransmissionMode-r19(with choice structure, for the mode B further indicate numOfSymbols-ModeB-r19/ configuredPUSCHResourceOfModeB-r19)

- Type 2: Event Specific, e.g. eventType-r19 with choice structure

- Event 1: eventThreshold-Event1-r19

- Event 2: eventThreshold-r19

- Event 7: eventThreshold-r19, valueOfQ-r19

For the Type 2 parameters, a choice structure can be used.

Proposal 5: Use the choice structure for the Event specific parameters, e.g.

- Event 1: eventThreshold-Event1-r19

- Event 2: eventThreshold-r19

- Event 7: eventThreshold-r19, valueOfQ-r19

For these parameters, based on the companies’ inputs, there are 3 options for the general structure as shown in the Fig 1:

- Option 1: Include the UEIBM parameters in the CSI-MeasConfig.

- Option 2: Include the UEIBM parameters in the CSI-ReportConfig with a CSI-ReportUE-IBM structure.

- Option 3: Include the UEIBM parameters in the CSI-ReportConfig directly (without a CSI-ReportUE-IBM structure).

Fig 1: UEIBM Parameters Structure

The advantage of the option 1 is that it’s quite clear and in the RAN2 spec, there is no need to further clarify which parameters (especially the mandatory present parameters in the legacy CSI-ReportConfig) are not used for the UEIBM. Thus the option 1 is preferred. Companies may argue that it’s RAN1’s decision, but we think it’s totally RAN2 issue on the ASN1 structure design and an LS can be sent to RAN1 to indicate this modification.

Proposal 6: Move the UEIBM parameters to the CSI-MeasConfig, and take the option 1 as baseline for the UEIBM ASN.1 parameters structure.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: For the UEI mode A CSI report, RAN1 has agreed to reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report and thus the UEI mode A CSI report is similar to the legacy aperiodic CSI report.

Proposal 1: The same principle as the aperiodic CSI report for the UL skipping, DRX and dormant BWP shall be adopted for the UEI mode A CSI report.

Proposal 2: Ran2 to discuss the following 3 options on how to capture the proposal 1 in MAC spec:

- Option 1: Add UEI mode A CSI report to every MAC spec chapter (e.g. 5.4/5.7/5.15) that includes aperiodic CSI report;

- Option 2: Add a General Note/Description to MAC spec, e.g. the UE’s behaviour for the UEI mode A CSI report is the same as the aperiodic CSI report;

- Option 3: No MAC spec impact, and take the UEI mode A CSI report as one kind of the aperiodic CSI report.

Proposal 2a: On how to capture the proposal 1 in MAC spec, select Option 2 as baseline.

Proposal 3: From MAC layer aspect, the similar principle to the legacy semi-persistent CSI report on the PUSCH can be taken as baseline for the UEI mode B CSI report and RAN2 can further discuss some exception cases.

Proposal 3a: As an exceptional case, if the BWP in a SCell is a dormant BWP, the UE can’t perform UEI mode B CSI report for this BWP.

Proposal 4: When UE-initiated report is triggered but there is no valid PUCCH/PUSCH resource to transmit UE-initiated report, the RACH procedure can be triggered without MAC spec impact/or enhancement.

Observation 2: The RAN1 parameters tend to be stable with few FFS on the detail value range, thus RAN2 can start the discussion on the ASN.1 structure.

Proposal 5: Use the choice structure for the Event specific parameters, e.g.

- Event 1: eventThreshold-Event1-r19

- Event 2: eventThreshold-r19

- Event 7: eventThreshold-r19, valueOfQ-r19

Proposal 6: Move the UEIBM parameters to the CSI-MeasConfig, and take the option 1 as baseline for the UEIBM ASN.1 parameters structure.