3GPP TSG-RAN WG2 Meeting #130	R2-2504627

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:        ZTE Corporation

Title:           Consideration on the Type 4 UE for Non-collocated Deployment

Agenda item:   8.20.1

Document for:  Discussion and Decision

- Introduction

A LS was received from RAN4 on the Type 4 UE for Non-collocated Deployment.

- Overall Description: RAN4 discussed the applicable UE requirements, new UE capability and new BS signalling for Type 4a/4b1 in previous meetings for which the following agreements were reached. RAN4 respectfully ask RAN2 to devise necessary UE capabilities and BS signalling accordingly. UE capability Type - For intra-band non-contiguous NR-CA, Type 4b is considered, Type 4a is not considered - For inter-band non-contiguous EN-DC with overlapping or partially overlapping bands, both Type 4a and Type 4b are considered UE requirements: - Reuse the power imbalance of Type 2 - Reuse the MRTD and MTTD requirements of Type 2 UE capability: - Define one capability for NR CA and one capability for EN-DC - For EN-DC, Type 4a and Type 4b capability can be differentiated by legacy capability, such as MIMO-CapabilityDL-r10 on E-UTRA carrier - If UE reports capability for Type 4a/4b, Type 2 capability shall be deemed as supported by default, and UE still needs to report Type 2 capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS) BS signalling: - When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed - When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed - When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2 NOTE: maxMIMO-layers intends for the legacy BS signaling NOTE 1: Type4a/4b is mentioned in the WID RP-242083. 2. Actions: To RAN2: ACTION: RAN4 respectfully ask RAN2 to devise necessary UE capabilities and BS signaling.

In this paper we share our views on this LS.

- Discussion

In this chapter, we first discuss the BS signaling, then discuss the UE capability.

2.1 BS signaling

For the BS signaling, as summarized in the LS, new BS signaling is needed for the case with maxMIMO-layers=4.

BS signalling: - When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed - When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed - When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2 NOTE: maxMIMO-layers intends for the legacy BS signaling

In the legacy R18 signaling (e.g. nonCollocatedTypeNR-CA-r18 for the CA case), there are 2 cases

- When maxMIMO-layers ＞2, it is collocated. (But during RAN4 discussion, companies don’t think it’s necessary to specify)

- When maxMIMO-layers<=2, if Rel-18 new BS signaling IE is present, it is collocated (Type-1); If Rel-18 new BS signaling IE is absent, it’s non-collocated (Type-2).

Now for the UE support R19 capability, there would be 3 cases:

- When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed

- When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed

- When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2

For the case 3, there are still 3 sub-cases:

- Subcase 1: The BS doesn’t support non-collocated deployment with maxMIMO-layers=4, e.g. the legacy BS. For this case, if the maxMIMO-layers=4, it’s collocated (Type-1);

- Subcase 2: The BS supports non-collocated deployment with maxMIMO-layers=4, and it configures it as non-collocated. For this case, if the maxMIMO-layers=4, it’s non-collocated (Type-2);

- Subcase 3: The BS supports non-collocated deployment with maxMIMO-layers=4, but it configures it as collocated. For this case, if the maxMIMO-layers=4, it’s collocated (Type-1);

Proposal 1: RAN2 to confirm that the signaling should support to indicate the below 3 sub-cases when maxMIMO-layers=4.

- Subcase 1: The BS doesn’t support non-collocated deployment with maxMIMO-layers=4, e.g. the legacy BS. For this case, if the maxMIMO-layers=4, it’s collocated (Type-1);

- Subcase 2: The BS supports non-collocated deployment with maxMIMO-layers=4, and it configures it as non-collocated. For this case, if the maxMIMO-layers=4, it’s non-collocated (Type-2);

- Subcase 3: The BS supports non-collocated deployment with maxMIMO-layers=4, but it configures it as collocated. For this case, if the maxMIMO-layers=4, it’s collocated (Type-1).

Based on these 3 sub-cases, first we need to introduce a parameter on whether the BS supports non-collocated deployment with maxMIMO-layers=4. To keep backward compatibility, when this parameter is absent, it means the BS doesn’t support non-collocated deployment with maxMIMO-layers=4.

Proposal 2: To distinguish the sub-case 1 from the subcase 2/3, a parameter (e.g. nonCollocatedTypeNR-CA-r19/ nonCollocatedTypeMR-DC-r19) is needed to indicate whether the BS supports non-collocated deployment with maxMIMO-layers=4.

Proposal 2a: When this parameter (e.g. nonCollocatedTypeNR-CA-r19/ nonCollocatedTypeMR-DC-r19) is absent, it means the BS doesn’t support non-collocated deployment with maxMIMO-layers=4 and thus it’s collocated (Type-1);

Furthermore, to further distinguish subcase 2 from the subcase 3, 2 values shall be set when this parameter (e.g. nonCollocatedTypeNR-CA-r19/ nonCollocatedTypeMR-DC-r19) is present, e.g. value 1 means subcase 2, while value 2 means subcase 3.

Proposal 3: To further distinguish subcase 2 from the subcase 3, 2 values shall be introduced when the parameter (e.g. nonCollocatedTypeNR-CA-r19/ nonCollocatedTypeMR-DC-r19) is present, e.g. value 1 means subcase 2, while value 2 means subcase 3.

2.2 UE Capability Signaling

For the UE capability, in the LS, it has been clearly indicated to define one capability for NR CA and one capability for EN-DC.

UE capability: - Define one capability for NR CA and one capability for EN-DC - For EN-DC, Type 4a and Type 4b capability can be differentiated by legacy capability, such as MIMO-CapabilityDL-r10 on E-UTRA carrier - If UE reports capability for Type 4a/4b, Type 2 capability shall be deemed as supported by default, and UE still needs to report Type 2 capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS)

For the NR-CA, a similar capability to the legacy intraBandNR-CA-non-collocated-r18 can be introduced for the type 4 UE. While for the EN-DC case, a similar capability to the legacy requirementTypeIndication-r18 r18 can be introduced.

Proposal 4: For the NR-CA, a similar capability to the legacy intraBandNR-CA-non-collocated-r18 can be introduced for the type 4 UE. While for the EN-DC case, a similar capability to the legacy requirementTypeIndication-r18 r18 can be introduced..

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: RAN2 to confirm that the signaling should support to indicate the below 3 sub-cases when maxMIMO-layers=4.

- Subcase 1: The BS doesn’t support non-collocated deployment with maxMIMO-layers=4, e.g. the legacy BS. For this case, if the maxMIMO-layers=4, it’s collocated (Type-1);

- Subcase 2: The BS supports non-collocated deployment with maxMIMO-layers=4, and it configures it as non-collocated. For this case, if the maxMIMO-layers=4, it’s non-collocated (Type-2);

- Subcase 3: The BS supports non-collocated deployment with maxMIMO-layers=4, but it configures it as collocated. For this case, if the maxMIMO-layers=4, it’s collocated (Type-1).

Proposal 2: To distinguish the subcase 1 from the subcase 2/3, a parameter (e.g. nonCollocatedTypeNR-CA-r19/ nonCollocatedTypeMR-DC-r19) is needed to indicate whether the BS supports non-collocated deployment with maxMIMO-layers=4.

Proposal 2a: When this parameter (e.g. nonCollocatedTypeNR-CA-r19/ nonCollocatedTypeMR-DC-r19) is absent, it means the BS doesn’t support non-collocated deployment with maxMIMO-layers=4 and thus it’s collocated (Type-1);

Proposal 3: To further distinguish subcase 2 from the subcase 3, 2 values shall be introduced when the parameter (e.g. nonCollocatedTypeNR-CA-r19/ nonCollocatedTypeMR-DC-r19) is present, e.g. value 1 means subcase 2, while value 2 means subcase 3.

Proposal 4: For the NR-CA, a similar capability to the legacy intraBandNR-CA-non-collocated-r18 can be introduced for the type 4 UE. While for the EN-DC case, a similar capability to the legacy requirementTypeIndication-r18 r18 can be introduced..

- Reference

[1] R4-2505161 LS to RAN2 on Type 4 for non-collocated deployment  3GPP