3GPP TSG-RAN WG2 #130	R2-2504636

St. Julian’s, Malta, 19th – 23rd May, 2025

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	LCM for UE-side models for beam management

Document for:	Discussion

# 1	Introduction

In this contribution we share our views on remaining open issues listed in [1] and further aspects.

# 2	Discussion

## 2.1 RRC open issues from [1]

This section discusses RRC open issues identified in [1], as an outcome of the email discussion [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson).

### Open issue RRC-1: Cause of inapplicability

RAN2#129bis agreed that:

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

In our view, reporting the cause of inapplicability is intended to assist the network to decide whether it should wait for some time before de-configuring an inapplicable configuration because the configuration is likely to become applicable soon, or whether it can de-configure it immediately because it is not expected that this configuration becomes applicable soon. It would thus be sufficient to define two possible cause values of inapplicability that serve this purpose. It is expected that a configuration becomes applicable soon in case there is a suitable model not yet available in the UE, but that the UE is ready to fetch or that the UE is preparing to apply. Hence, we can define a cause value “preparationOngoing” for this case. For the second case (not expected that the configuration becomes applicable soon), there may be several reasons, e.g. there is no suitable available model neither at the UE nor at the UE-side training entity, e.g. there is no AIML model trained yet for a certain functionality. Hence, we can define a generic cause value “prolongedInapplicability” for this case.

(RRC-1) In the applicability report (including inapplicability indication for a certain inference configuration), the UE may include the following values related to the unavailability of the AIML model: “preparationOngoing” (if the AIML model is available at the storing entity, and the UE is downloading or about to download it) or “prolongedInapplicability” (if no AIML model is available for downloading).

### Open issue RRC-2: Content of otherConfig for enabling applicability reports in UAI

The following agreements were made:

From RAN2#127: „“Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig.“ From RAN2#129: “3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.”

Related to these agreements, the issue was raised of whether the network needs to explicitly send a configuration in otherConfig, to enable the UE to send applicability reports via UAI, or whether sending full inference configurations in RRCReconfiguration (in CSI-ReportConfig) implicitly enables the UE to report applicability (including applicability updates) in UAI. In our view, any kind of reporting that the UE performs in UAI needs to be configured by the network, in order to follow the principle that was applied for the legacy configurations. We do not see a reason to change this principle, especially since applicability reporting via UAI is a separate function than applicability reporting via RRCReconfigurationComplete, which was also agreed for option A. Thus, the UE should be configured to send applicability reports in UAI via otherConfig.

Furthermore, if RAN2 confirms that the sets of inference related parameters for option B are sent in otherConfig (as agreed in RAN2#129bis), the configuration content in otherConfig should be extended with the parameters received from RAN1 for option B.

(RRC-2) The UE is explicitly configured in otherConfig to send applicability reports in UAI, including applicability reports for updates. Sending inference configurations in CSI-ReportConfig does not implicitly configure the UE to send applicability reports in UAI.

### Open issue RRC-3: UE data collection request

RAN2#129bis agreed the following regarding UE-side data collection for training:

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Thus, as mentioned in the open issue list [1], it is not yet clarified:

- how to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW

- where/what the NW provides as candidate configurations

- what the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.).

During previous discussions, concerns have been raised about allowing the UE to express a preference for specific radio measurement configurations in the request for data collection. It is also agreed that the preference can only be out of a list of candidate configurations provided by NW. To take this into account and also simplify these procedures, it is most straightforward that the candidate configurations for UE data collection are the same configurations that the NW configures for inference. For instance, if the UE receives an inference configuration and determines that it is inapplicable or would like to collect more data for it to improve its model, it can indicate this inference configuration as a preferred configuration for data collection. Thus, there is no need to configure separate candidate configurations for UE data collection and the UE can simply include IDs of the inference configurations (i.e. CSI-ReportConfigId and potentially a new ID for sets of inference related parameters for option B sent in otherConfig) in the request for data collection.

Furthermore, based on the agreements above, it is not mandatory that the UE includes a preferred candidate configuration in the request to the network. To address concerns that were raised in the past, we think this should be under network control, so otherConfig should include an explicit configuration to indicate to the UE whether it is allowed to include preferred candidate configurations in the request or not. Namely, the UE can be allowed to send only start/stop information, without a specific preferred candidate configuration.

(RRC-3) The candidate UE data collection configurations from the NW can be the inference configurations.

(RRC-3) The UE can indicate a preferred candidate UE data collection configuration by including a reference to the inference configuration(s) in the request for data collection.

(RRC-3) The NW controls via configuration in otherConfig whether the UE is allowed to include preferred candidate configurations in UE data collection request.

### Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

RAN2#129 agreed that:

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Further, RAN2#129bis agreed:

2 Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

The agreements imply that a periodic CSI report configuration that is initially reported as inapplicable will not be activated but will be maintained by the UE. This case (i.e. the UE being configured with a periodic CSI report configuration that is not active) is new compared to legacy procedures and it should be clarified how the UE should behave further, especially when the status of the periodic CSI report configuration changes from inapplicable to applicable.

Without any specification impact, this can be solved if the network simply de-configures the periodic CSI report configuration after the UE reports that this configuration is inapplicable.

However, if this solution is not desired because it limits the possibility to activate this configuration once it becomes applicable (e.g. as a result of performance monitoring), a standardized solution is needed. One possible standardized solution is that the UE autonomously activates the periodic CSI report configuration once it becomes applicable. However, this solution may not work in a straightforward way, because the NW may not be sending the needed reference signals, since it does not know yet that the configuration became applicable. Thus, the UE should not autonomously activate such a configuration, but just report to the NW that it became applicable and it is then up to the NW to activate it. To activate this configuration, the MAC CE used for activating semi-persistent CSI report configurations can be enhanced.

Hence, related to the handling of the periodic CSI report configuration, the following should be discussed.

(RRC-4) Related to the handling of the periodic CSI report configuration, RAN2 to discussion the following options:

  - Upon applicability status change from inapplicable to applicable, the UE continues to operate according to the inapplicable status (no specification impact expected)

  - If a standardized solution is needed, upon applicability status change from inapplicable to applicable, the UE does not autonomously activate a periodic CSI report configuration, i.e. the NW can activate a periodic CSI report configuration that changed from inapplicable to applicable via the MAC CE used for activating semi-persistent CSI report configuration. If the MAC CE is used for activating a periodic CSI report configuration, it should also be supported that the MAC CE is used for de-activating a periodic CSI report configuration that changed from applicable to inapplicable.

### Open issue RRC-5: Reporting behaviour of inapplicable periodic beam prediction configuration

It was raised that it is ambiguous what the UE should report to the NW when its periodic beam prediction configuration becomes inapplicable. Firstly, in our view this issue holds not only for periodic CSI reporting configurations, but also for semi-persistent and aperiodic CSI reporting configurations that become inapplicable.

Further, RAN2#128 agreed that:

2. When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable

In the spirit of the agreement above based on which the UE doesn’t autonomously deactivate, RAN2 should not agree that the UE autonomously stops reporting predicted values based on the periodic CSI report configuration, even though for periodic CSI reports there is no “activation/deactivation” concept per se, unlike for semi-persistent and aperiodic reporting. The reason is that uplink resources have already been allocated for reporting the predicted values and the NW expects at L1 some values in a certain format. If the UE autonomously stops transmitting them, this may have a negative impact on decoding in RAN1. Furthermore, if the UE keeps reporting some values at L1 (e.g. inaccurate prediction or actual measurements instead of predictions), this also falls within the RAN1 domain, since it may have an impact on the format of the expected L1 reports. Thus, we think it is up to RAN1 to solve this issue.

(RRC-5) It is up to RAN1 whether the UE stops reporting any values or whether it reports other values instead of predictions, when a CSI report configuration changes from applicable to inapplicable.

### Open issue RRC-6: Handling of inference, applicability reporting and UE data collection preference configurations when UE goes to RRC_IDLE/INACTIVE state and during RRCReestablishment

The UE configurations for inference, applicability reporting (configuration in otherConfig to enable applicability reporting via UAI), UE data collection (radio measurement configuration for UE data collection) and UE data collection preference (configuration in otherConfig to allow UE start/stop request and a preferred measurement configuration for UE data collection) are specific to each cell. Furthermore, the NW may change or de-configure/de-activate the inference configuration even if the UE stays in the same cell, if e.g. the UE moves within the cell, which then may also require a change in the related applicability report configuration, the UE data collection configuration, UE data collection preference configuration, and UE data collection configuration. The same holds also for the UE performance monitoring configuration, which is referencing an inference configuration. Thus, we do not see the need for the UE to keep these configurations upon entering RRC_IDLE/INACTIVE and upon initiating the RRCReestablishment procedure, since these configurations may not be useful afterwards anyway.

(RRC-6) UE configurations related to inference, applicability reporting, performance monitoring and UE data collection (for measurements and for providing assistance information) are released upon entering RRC_IDLE/INACTIVE and upon initiating the RRCReestablishment procedure.

### Open issue RRC-7: Applicability reporting for option B in RRCReconfigurationComplete

RAN2#129bis agreed that:

RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

It is straightforward to allow applicability changes for option B to be reported via UAI, as also agreed for option A. However, assuming that the sets of inference related parameters for option B are sent in otherConfig, as in the agreement above, it is natural that the initial applicability report for option B is also sent in UAI and not in RRCReconfigurationComplete, since otherConfig typically configures the UE for reporting via UAI. An exception could be considered (to allow the UE to report the initial applicability for option B in RRCReconfigurationComplete, even though option B parameters are sent in otherConfig), if the UE replies with the same simple applicability information as for option A. However, in our view this is not yet clarified either.

Overall, these procedural aspects depend, among others, on whether the parameters for option B are indeed sent in otherConfig, which will only be confirmed after RAN1 provides the option B inference related parameters, as per the agreement above. Therefore, in our view, any agreements about such procedures for option B should anyway be revisited after receiving option B inference related parameters.

(RRC-7) Applicability reports for option B (initial applicability and applicability changes) are sent in UAI, assuming that option B parameters are send in otherConfig. This assumption can be confirmed after receiving option B inference related parameters from RAN1.

### Open issue RRC-8: Coexistence between option A and option B

The issue was raised on how option A and option B can coexist. For this issue, as for open issue RRC-7 above, in our view, any agreements about such procedures for option B should anyway be revisited after receiving option B inference related parameters from RAN1.

At a high level, we think option A and option B can coexist in the sense that both can be configured by the network at the same time, i.e. some configurations are full inference configurations and some other configurations may be sets of inference related parameters. We do not see a reason to limit this. However, the exact impact on signalling and procedures for further interaction between these options should be discussed after receiving option B parameters from RAN1.

(RRC-8) Both option A can option B can be configured at the same time. This assumption can be confirmed after receiving option B inference related parameters from RAN1. FFS further details on the interaction between option A and option B.

## 2.2 Other aspects

### 2.2.1 Security aspects for applicability reporting

RAN2 has agreed to support option A (full inference configuration), consistent with RAN1 agreements, where the UE sends the applicability report in RRCReconfigurationComplete and the updated applicability in UAI. Let us discuss whether the applicability report from the UE can be sent before or after security activation. We note that this is a new type of report from the UE, so there is no specific legacy rule for it. In our view, a malicious applicability report can result in a severe performance degradation. For example, wrongly reporting an inference configuration as applicable may result in the NW activating that inference configuration, which is in fact inapplicable at the UE. Consequently, the UE will report wrong predictions or may have an unexpected behavior, causing a severe performance degradation. Thus, the applicability report should be sent only after security activation. This can be solved from the NW side, by sending the RRCReconfiguration with the inference configuration only after security activation, which ensures that the UE replies with the applicability report only after security activation.

The RRCReconfiguration with inference configuration is sent by the NW after security activation, so the applicability report is sent by the UE also after security activation.

### 2.2.2	Functionality activation/deactivation

RAN2#127bis agreed that:

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

Further, RAN2#128 agreed that:

2. When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Given the two agreements above, it is critical for the network to be informed about functionalities/configurations that become inapplicable as soon as possible, so that the network can deactivate those functionalities/configurations. Thus, it should be mandated (and not optional) that the UE reports when an applicable functionality/configuration becomes inapplicable. This aspect was also discussed in RAN2#128 and it was suggested that we revise the agreement from RAN2#127bis as follows: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

Revise agreement 3 from RAN2#127bis as: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	(RRC-1) In the applicability report (including inapplicability indication for a certain inference configuration), the UE may include the following values related to the unavailability of the AIML model: “preparationOngoing” (if the AIML model is available at the storing entity, and the UE is downloading or about to download it) or “prolongedInapplicability” (if no AIML model is available for downloading).

Proposal 2	(RRC-2) The UE is explicitly configured in otherConfig to send applicability reports in UAI, including applicability reports for updates. Sending inference configurations in CSI-ReportConfig does not implicitly configure the UE to send applicability reports in UAI.

Proposal 3	(RRC-3) The candidate UE data collection configurations from the NW can be the inference configurations.

Proposal 4	(RRC-3) The UE can indicate a preferred candidate UE data collection configuration by including a reference to the inference configuration(s) in the request for data collection.

Proposal 5	(RRC-3) The NW controls via configuration in otherConfig whether the UE is allowed to include preferred candidate configurations in UE data collection request.

Proposal 6	(RRC-4) Related to the handling of the periodic CSI report configuration, RAN2 to discussion the following options:

a.	Upon applicability status change from inapplicable to applicable, the UE continues to operate according to the inapplicable status (no specification impact expected)

b.	If a standardized solution is needed, upon applicability status change from inapplicable to applicable, the UE does not autonomously activate a periodic CSI report configuration, i.e. the NW can activate a periodic CSI report configuration that changed from inapplicable to applicable via the MAC CE used for activating semi-persistent CSI report configuration. If the MAC CE is used for activating a periodic CSI report configuration, it should also be supported that the MAC CE is used for de-activating a periodic CSI report configuration that changed from applicable to inapplicable.

Proposal 7	(RRC-5) It is up to RAN1 whether the UE stops reporting any values or whether it reports other values instead of predictions, when a CSI report configuration changes from applicable to inapplicable.

Proposal 8	(RRC-6) UE configurations related to inference, applicability reporting, performance monitoring and UE data collection (for measurements and for providing assistance information) are released upon entering RRC_IDLE/INACTIVE and upon initiating the RRCReestablishment procedure.

Proposal 9	(RRC-7) Applicability reports for option B (initial applicability and applicability changes) are sent in UAI, assuming that option B parameters are send in otherConfig. This assumption can be confirmed after receiving option B inference related parameters from RAN1.

Proposal 10	(RRC-8) Both option A can option B can be configured at the same time. This assumption can be confirmed after receiving option B inference related parameters from RAN1. FFS further details on the interaction between option A and option B.

Proposal 11	The RRCReconfiguration with inference configuration is sent by the NW after security activation, so the applicability report is sent by the UE also after security activation.

Proposal 12	Revise agreement 3 from RAN2#127bis as: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

# References

R2-2504350, “RRC open issues AIML PHY”, 3GPP TSG RAN WG2#130, St. Julian’s, Malta, 19-23 May 2025.

R1-2410898 / R2-2500011, “Reply LS on applicable functionality reporting for beam management UE-sided model”, 3GPP TSG RAN WG1#119, Orlando, US, November 18th – 22nd, 2024.

R2-2501517, “Summary of offline discussion [AT129][026][AI Phy] Signaling for BM”