3GPP TSG-RAN WG2 #130	R2-2504639

St. Julian, Malta, May 19th – 23rd, 2025

Agenda item:	8.6.3 (NR_Mob_Ph4-Core)

Source:	LG Electronics Inc.

Title: 	Resolving MAC open issues for LTM event triggered MR

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss MAC open issues identified and captured in [post129bis][117][MOB] email discussion. We also raise some more issues (marked as MAC-LGE# below) to discuss.

Discussion

MAC-LGE1: For given UL grant, multiple LTM MR MAC CEs for the same LTM report config can be included in one MAC PDU?

It may happen that different beams associated with an LTM report config satisfies entering condition with TTT at adjacent times. In this case, it is not crystal clear if each bean needs to trigger MR by each beam. It is also unclear if UE should generate only one LTM MR MAC CE even if multiple MRs has been triggered for the LTM report config.

Note that in L3 event triggered MR, whenever a cell satisfies entering condition with TTT, RRC generates MR message and submits it to lower layers. But, in LTM event triggered MR, considering that there is time gap between MR triggering and generation of MR MAC CE, it is not clear if the similar principle with beam granularity applies or not. Since this issue was not explicitly discussed, it would be good to discuss.

In our view, there are three options in case different beams satisfy entering condition with TTT at adjacent times for the same LTM report config:

- Option1 (multiple MRs triggered, and multiple LTM MR MAC CEs generated): Whenever a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered for the LTM report config. If UL grant is received while multiple MRs are triggered for the LTM report config, each triggered MR generates a separate LTM MR MAC CE, i.e., multiple LTM MR MAC CEs are generated. This option is similar to L3 event triggered MR.

- Option2 (multiple MRs triggered, but single LTM MR MAC CE generated): Whenever a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered for the LTM report config. Then, if UL grant is received while multiple MRs are triggered for the LTM report config, only one LTM MR MAC CE is generated.

- Option3: (single MR is triggered, and hence single LTM MR MAC CE generated) If a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered if MR has not been already triggered for the LTM report config. Then since at most MR can be triggered for an LTM report config, if UL grant is received, UE generates one MR MAC CE.

In option1, when UE receives UL grant, UE may have MRs triggered for the same LTM report config and hence generate multiple LTM MR CEs, as shown in Figure 1. These multiple LTM MR MAC CEs have the same contents except for the type00 beam for each LTM MR MAC CE, implying that UL grant would be wasted. Furthermore, UE suffers from managing status of each triggered MR for all triggered MRs (whether to cancel or not for each triggered MR, and which beam is a triggering beam for each MR/MAC CE), which increases UE complexity.

Figure 1. Option1 – For beams associated with the same LTM report config, each beam triggers a separate MR and each triggered MR generates a separate LTM MR MAC CE

In option2 is applied, there is no waste of UL grant as caused in option1, but it still suffers from increased UE complexity to manage status of each triggered MR for all triggered MRs (whether to cancel or not for each triggered MR, and which beam is a triggering beam for each MR/MAC CE). The example is shown in Figure 2. The current MAC running CR seems to be formulated based on this approach.

Figure 2. Option2 – For beams associated with the same LTM report config, each beam triggers a separate MR but only one LTM MR MAC CE is generated

Option3 has the issue of neither UL grant waste nor increased UE complexity, which makes the option3 attractive. The example is shown in Figure 3. If option3 is taken, we need to introduce a restriction on initiating measurement reporting in 5.x.3.1 such that UE initiates measurement reporting if measurement reporting procedure is not already triggered for the ltm-CSI-ReportConfigId.

Figure 3. Option3 – For beams associated with the same LTM report config, each beam triggers a MR if MR has not been already triggered. Then only one LTM MR MAC CE is generated.

Proposal 1: To clarify which option should be used to specify LTM event-triggered reporting:

- Option1 (multiple MRs triggered, and multiple LTM MR MAC CEs generated): Whenever a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered for the LTM report config. If UL grant is received while multiple MRs are triggered for the LTM report config, each triggered MR generates a separate LTM MR MAC CE, i.e., multiple LTM MR MAC CEs are generated. This option is similar to L3 event triggered MR.

- Option2 (multiple MRs triggered, but single LTM MR MAC CE generated): Whenever a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered for the LTM report config. Then, if UL grant is received while multiple MRs are triggered for the LTM report config, only one LTM MR MAC CE is generated.

- Option3: (single MR is triggered, and hence single LTM MR MAC CE generated) If a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered if MR has not been already triggered for the LTM report config. Then since at most MR can be triggered for an LTM report config, if UL grant is received, UE generates one MR MAC CE.

Our preference is to choose one of option2 and 3.

MAC-LGE2: If UE receives UL grant that can accommodate up to M LTM MR MAC CEs while N (>M) MRs have been triggered and all the N MRs are associated with different LTM report configs, how to select M out of N?

We can consider three options.

- Option1: FIFO based on triggering timing of L1 measurement report

- Option2: LIFO based on triggering timing of L1 measurement report

- Option3: Up to UE implementation.

In our view, option1 is most desirable because the order of triggered events itself is meaningful information for network side and FIFO principle based on option1 can enable delivery of this information to network.

Figure 4. Three MRs associated with different LTM report configs are triggered. When UE receives UL grant, UE generates two MR MAC CEs out of three triggered MRs based on UL grant size, where the UE selects two first triggered MRs to generate the two MR MAC CEs.

Figure 4 illustrates option1. Three MRs, MR#1, MR#2, and MR#3 are triggered, where the LTM report configs associated with the triggered MRs are all different, and MR#1, MR#2, and MR#3 are triggered in order. Then UE receives UL grant which can only accommodate 2 LTM MR MAC CEs. Then, UE generates two MR MAC CEs corresponding to MR#1 and MR#2 and transmits the two MR MAC CEs in MAC PDU.

Proposal 2: When UE generates M LTM MR MAC CEs to be included in MAC PDU while N (>M) MR have been triggered and pending, UE selects M LTM events to report by applying FIFO principle based on timing of triggering the concerned MR.

MAC-LGE3: Modification of LTM MR MAC CE structure

We discuss the structure of the current LTM MR MAC CE in the running CR. Figure6 is the current structure captured in the running CR.

Figure 6. Structure of the current LTM MR MAC CE in the running CR

In our view, there are two issues which motivate a modified structure.

- 1) Oct1 consists of ReportID and Type1 fields. While ReportID is common for all beams included in the MAC CE, Type1 field is only relevant to the best beam included in Oct2 and Oct3 and irrelevant to other Oct N with N>3. Then, in case this structure is used for LTM MR MAC CE triggered by LTM2, where the essential information is only reportID and serving cell beam information (RSRPserving), if the MAC CE does not include best candidate cell beam information (Oct2 and 3) but only include serving cell information (RSRPserving), the usage of Type1 becomes unclear. To avoid such unclarity, it would be better to relocate Type1 from Oct1 to Oct2.

- 2) For serving cell beam info, R bit is the first bit of the relevant bit string (Oct 2M+2). For candidate cell beam info, however, R bit is not the first bit of the relevant bit strings (Oct2M, Oct 2M+1 for M=1, 2, ..). It would be better if the relative R bit position in the bitstring for serving cell beam is the same as the relative R bit position in the bitstring for candidate cell beam. Desirably, R bit could be placed as MSB in the first Oct for each beam.

Based on the discussion, the modified format of regular/truncated LTM MR MAC CE is shown in Figure 7.

Figure 7. Modified LTM MR MAC CE format

Proposal 3: The first octet (Oct1) only includes information common for all the included beams, and R bit for each beam is placed as MSB of the first octet for the beam. The resulting LTM MR MAC CE format is illustrated in Figure 7.

MAC-5 and MAC-LGE4: LTM MR MAC CE triggered by LTM2

For LTM MR MAC CE triggered by LTM2, we think there are several issues to discuss, including not only MAC-5 issue but also MAC-LGE4 described below.

- MAC-5 issue: For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates

- MAC-LGE4: The detailed structure of LTM MR MAC CE

  - which/how the fields are set for serving cell beam and candidate cell beam

  - whether to introduce a new LTM MR MAC CE format specific to MR triggered by LTM2

Let us discuss MAC-5 issue first. For LTM MR MAC CE triggered by LTM2, the essential information is serving cell beam information. The necessity of further including LTM candidate cell beam into the MAC CE is not clear;

- if LTM3 were configured and there were any candidate cell beam that is offset better than the serving cell beam for the duration of TTT, the candidate cell beam could be reported as triggering beam for LTM3.

- Even if LTM MR MAC CE triggered by LTM2 reports one or more candidate cell beams with good quality to network, it would be risky for network to trigger cell switch based on the reported candidate cell beams, because the reported candidate cell beam quality is not evaluated against TTT (i.e. the reported candidate cell beam might be temporarily good).

Even though the necessity of including candidate cell beam into LTM MR MAC CE triggered by LTM2 is very unclear to us, if network can control UE on whether to include candidate cell beams in the MAC CE via RRC, we see no problem in including candidate cell beam info into LTM MR MAC CE triggered by LTM2, assuming that network will use the reported candidate cell beam(s) in a reasonably way.

Proposal 4: For LTM MR MAC CE triggered by LTM2, beam measurement results of LTM candidates are included, if configured by network via RRC. For each LTM candidate cell, if included, only one best beam is included, if available, and its beam type is set to 11. RSRI1 and RSRP1 are set to the beam information of the best beam among the included LTM candidate beams.

Assuming that RSRI1 and RSRP1 are set to the beam information of the best beam among the included LTM candidate beams, there are two possible options on where to set serving cell beam information as triggering beam. The first option is to use the Octet including RSRPserving field and the other option is to use the octets including RSRIM and DIFFRSRPM. Our preference is to use RSRPserving field, because RSRI needs not be reported for serving cell beam (network always knows it).

Proposal 5: For LTM MR MAC CE triggered by LTM2, serving cell beam measurement result is included in RSRPserving field.

Let us move onto the discussion on the structure of LTM MR MAC CE triggered by LTM2, assuming that two candidate cell beams are included by applying proposal 4 and 5 based on the modified LTM MR MAC CE format in proposal 3. Then, depending on the position of the octet containing RSRPserving, the LTM MR MAC CE would look like the one in Figure 8(a) or 8(b).

(a) 							(b)

Figure 8. The structure of LTM MR MAC CE triggered by LTM2. (a) candidate cell beams are included in Oct2 to Oct6 and serving cell beam is included in the last octet K, (b) candidate cell beams are included in Oct3 to Oct K-1 and serving cell beam is included in the 2nd octet.

We can observe that if the left structure in Figure 8 is taken, it means that the LTM MR MAC CE format for LTM 3/4/5 shown in Figure 7 is reused for LTM MR MAC CE format for LTM2. Then for truncated LTM MR MAC CE triggered by LTM2, there may be the case that the first octet and the last octet are included but the other octets are not included at all. If this complicates MAC CE generation process, this structure would be less desirable.

If the right structure in Figure 8 is taken, when generating LTM MR MAC CE triggered by LTM2, octets are included in the order of the octet number. But this structure is different LTM MR MAC CE structure for LTM 3/4/5 shown in Figure 7. That is, we need to have two separate LTM MR MAC CE structures, one for LTM3/4/5 and the other for LTM2. To avoid two separate LTM MR MAC CE structures, we could consider using the structure in Figure 8(b) for LTM MR MAC CE triggered by all LTM events.

Both alternatives are feasible, and we have no strong preference.

Proposal 6: For LTM MR MAC CE triggered by LTM2, RAN2 to decide either one of the following options for the location of RSRPserving. If Option2 is selected, discuss if this structure is also applicable for LTM MR MAC CE triggered by LTM3/4/5 universally.

- Option1: RSRPserving is placed in the last octet

- Option2: RSRPserving is placed in the 2nd octet

For LTM MR MAC CE triggered LTM2, serving cell beam is the only essential information and candidate cell beams can be reported on a best-effort basis. Since there is only one serving cell beam and the serving cell beam is always included in LTM MR MAC CE triggered by LTM2, once UE transmits truncated MR MAC CE triggered by LTM2, it is not necessary to keep the MR state as triggered merely to report candidate cell beams later if another grant is received.

Proposal 7: After transmitting truncated LTM MR MAC CE triggered by LTM2, UE cancels the triggered MR.

MAC-6: Truncated LTM MR MAC CE details. E.g.

Except the triggered beam, whether any beam should be prioritised to be included in the truncated L1 MR MAC CE? In case the UL grant could include at least 2 beams in truncated L1 MR MAC CE, which beam should be included as the first beam in the truncated L1 MR MAC CE? Based on following agreement: For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

We see it valuable to minimize any disparity between regular LTM MR MAC CE and truncated LTM MR MAC CE in terms of the format/structure. That is, same rule of generating LTM MR MAC CE can be applied to both regular LTM MR MAC CE and truncated LTM MR MAC CE, based on the following proposal.

Proposal 8: For both regular LTM MR MAC CE and truncated LTM MR MAC CE triggered by LTM 3/4/5, the beams are included in the following order

- best quality beam of any type, triggering beam (type00 or type01), type10 beam, and then type11 beam.

- For the beams of the same type, beam of higher quality is included first.

MAC-7: Whether one UL TB could include more than one truncated MAC CE or only one?

For the case more than 1 triggering events are pending, if UL grant is sufficient for one regular L1 MR MAC CE or for two truncated L1 MR MAC CEs but insufficient for two regular L1 MR MAC CEs, the UE behaviour should be discussed.

This issue was initially raised in [R2-2502950]. To discuss the issue, let us assume that UE receives UL grant while N triggered LTM events are pending. Further assume that the UL grant is sufficient for including some combinations of L regular LTM MR MAC CEs and M truncated MR MAC CEs, where L, M, and N are non-negative integer with L+M≤N, 0≤L<N, 0≤M≤N but the UL grant is insufficient for including N regular LTM MR MAC CEs. For example, while UE has two LTM events are pending, UE receives UL grant sufficient for 1 regular MAC CE+1 truncated MAC CE or for 2 truncated MAC CEs, but insufficient for 2 regular MAC CEs. For such cases, three possible options can be considered:

- Option1: Maximize the number of regular LTM MR MAC CEs (only one regular MAC CE in the example)

  - Pros: Since regular LTM MR MAC CE includes complete beam information, network can make a reliable decision based on the received LTM MR MAC CE.

  - Cons: Compared to option2, since a smaller number of triggered events are selected and reported in one TB, unselected triggered events and corresponding beams are reported later.

- Option2: Maximize the number of truncated LTM MR MAC CEs (two truncated MAC CEs in the example)

  - Pros: Compared to option1, a larger number of triggered events and corresponding triggered beams are selected and reported earlier in one TB.

  - Cons: After transmission of multiple truncated LTM MR MAC CEs, regular LTM MR MAC CEs for these should be transmitted again.

- Option3: Network configures, via reportConfig for LTM, which options between option1 and 2 should be applied

- Option4: Up to UE implementation.

In our view, option1 and option2 have their own pros and cons, hence mandating a particular option between the two is not strongly justified. For this reason, we can consider option3 and option4, and we slightly prefer option4 given the assumption that smart UE implementation will find its best way. But, if network vendors prefer to have a consistent UE behavior, we are fine with option3.

Proposal 9: In case N triggered LTM events are pending, if UL grant is sufficient for including some combinations of L regular LTM MR MAC CEs and M truncated MR MAC CEs with L+M≤N, 0≤L<N, 0≤M≤N, but insufficient for including N regular LTM MR MAC CEs, it is up to UE implementation which combination to include.

if UL grant is sufficient for one regular LTM MR MAC CE or for two truncated LTM MR MAC CEs but insufficient for two regular LTM MR MAC CEs, UE is allowed to construct MAC PDU either by including one regular LTM MR MAC CEs or by including two truncated LTM MR MAC CEs, based on its implementation choice.

MAC-15: Whether the triggered MR should be cancelled if all the triggered beam(s), i.e. Type#00 and Type#01, have been reported, no matter whether truncated MR MAC CE or regular MR MAC CE is used before

The cancelation of the MR upon reporting all triggered beams (type#00, type01) seems to be motivated by the below agreement.

Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

The key question is what is meany by “event-triggered beam(s)” as highlighted in yellow. It is obvious that type00/type01 beam is meant and type11 beam is not meant. Ambiguity is whether type10 beam is also meant or not.

Technically, it is a design choice whether to count type10 beam in addition to type00/01 beam as essential beams, because either way would work. Our preference is that type10 beam is also meany by “event-triggered beam(s)”. The reasoning of our preference is that if type01/10 beams(s) are reported without any type10 beam(s) or with an incomplete type10 beam list and then the triggered MR is cancelled, network may misunderstand the complete/latest beamsTriggeredList (or network has to store the previously reported beamsTriggeredList for each LTM report config and compare the received type00/01 beam with the stored beamsTriggeredList in order to construct the complete/latest beamsTriggeredList).

Proposal 10: After transmitting truncated MR MAC CE, if there is at least one type00, type01 or type10 beam, triggered MR is not cancelled.

MAC-16: Whether the triggered MR should be cancelled in the below two cases:

- Case 1: When a beam (or multiple beam) meets the entry condition for TTT (Type#00), a MR will be triggered. Before getting the UL grant, if this beam (or all these beams) meets the leaving condition, then, whether this MR is still triggered or should be cancelled?

- Case 2: When a beam (or multiple beam) in the reporting list meets the leaving condition for TTT (Type#01), a MR will be triggered. Before getting the UL grant, if this beam (or all these beams) meets the entry condition again, then, whether this MR is still triggered or should be cancelled?

In our view, it would be consistent and sufficient to cancel the triggered MR in both cases. In case1, if the beam satisfies the entering condition again before UL grant is received, a new MR is simply triggered again. Then when UL grant is received, LTM MR MAC CE including the beam as type00 is generated and transmitted. Likewise, in case2, if the beam satisfies the leaving condition again before UL grant it received, a new MR is simply triggered again. Then if UL grant is received, LTM MR MAC CE including the beam as type01 is generated and transmitted.

One small downside of cancelling the triggered MR in both cases could be that if the MR is cancelled after SR is already transmitted and then the concerned beam meets entering condition again in case1 (or meets leaving condition again in case2) before receiving UL grant, SR may be transmitted again because a new MR is triggered. If the triggered MR were not cancelled, even if the beam meets entering condition again in case1 (or leaving condition again in case2) before receiving UL grant, the SR is not transmitted again because a new MR is not triggered. We, however, think the downside is small because satisfaction of entering > leaving >entering with TTT (or leaving > entering > leaving with TTT) for a beam during the time period [MR is initially triggered, UL grant is received] does not happen frequently.

Proposal 11: If a beam which triggered the MR due to entering (or leaving) condition with TTT satisfies leaving (or entering) condition with TTT before receiving UL grant, cancel the triggered MR.

Conclusion

MAC-LGE1: For given UL grant, multiple LTM MR MAC CEs for the same LTM report config can be included in one MAC PDU?

Proposal 1: To clarify which option should be used to specify LTM event-triggered reporting

- Option1 (multiple MRs triggered, and multiple LTM MR MAC CEs generated): Whenever a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered for the LTM report config. If UL grant is received while multiple MRs are triggered for the LTM report config, each triggered MR generates a separate LTM MR MAC CE, i.e., multiple LTM MR MAC CEs are generated. This option is similar to L3 event triggered MR.

- Option2 (multiple MRs triggered, but single LTM MR MAC CE generated): Whenever a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered for the LTM report config. Then, if UL grant is received while multiple MRs are triggered for the LTM report config, only one LTM MR MAC CE is generated. The current MAC running CR seems to implement this option.

- Option3: (single MR is triggered, and hence single LTM MR MAC CE generated) If a new beam satisfies entering condition with TTT for an LTM report config, a new MR is triggered if MR has not been already triggered for the LTM report config. Then since at most MR can be triggered for an LTM report config, if UL grant is received, UE generates one MR MAC CE.

MAC-LGE2: If UE receives UL grant that can accommodate up to M LTM MR MAC CEs while N (>M) MRs have been triggered and all the N MRs are associated with different LTM report configs, how to select M out of N?

Proposal 2: When UE generates M LTM MR MAC CEs to be included in MAC PDU while N (>M) MR have been triggered and pending, UE selects M LTM events to report by applying FIFO principle based on timing of triggering the concerned MR.

MAC-LGE3: Modification of LTM MR MAC CE structure

Proposal 3: The first octet (Oct1) only includes information common for all the included beams, and R bit for each beam is placed as MSB of the first octet for the beam. The resulting LTM MR MAC CE format is illustrated in Figure 7.

Figure 7. Modified LTM MR MAC CE format

MAC-5 and MAC-LGE4: LTM MR MAC CE triggered by LTM2

Proposal 4: For LTM MR MAC CE triggered by LTM2, beam measurement results of LTM candidates are included, if configured by network via RRC. For each LTM candidate cell, if included, only one best beam is included, if available, and its beam type is set to 11. RSRI1 and RSRP1 are set to the beam information of the best beam among the included LTM candidate beams.

Proposal 5: For LTM MR MAC CE triggered by LTM2, serving cell beam measurement result is included in RSRPserving field.

Proposal 6: For LTM MR MAC CE triggered by LTM2, RAN2 to decide either one of the following options for the location of RSRPserving. If Option2 is selected, discuss if this structure is also applicable for LTM MR MAC CE triggered by LTM3/4/5 universally.

- Option1: RSRPserving is placed in the last octet

- Option2: RSRPserving is placed in the 2nd octet

Proposal 7: After transmitting truncated LTM MR MAC CE triggered by LTM2, UE cancels the triggered MR.

MAC-6: Truncated LTM MR MAC CE details. E.g.

Proposal 8: For both regular LTM MR MAC CE and truncated LTM MR MAC CE triggered by LTM 3/4/5, the beams are included in the following order

- best quality beam of any type, triggering beam (type00 or type01), type10 beam, and then type11 beam.

- For the beams of the same type, beam of higher quality is included first.

MAC-7: Whether one UL TB could include more than one truncated MAC CE or only one?

Proposal 9: In case N triggered LTM events are pending, if UL grant is sufficient for including some combinations of L regular LTM MR MAC CEs and M truncated MR MAC CEs with L+M≤N, 0≤L<N, 0≤M≤N, but insufficient for including N regular LTM MR MAC CEs, it is up to UE implementation which combination to include.

MAC-15: Whether the triggered MR should be cancelled if all the triggered beam(s), i.e. Type#00 and Type#01, have been reported, no matter whether truncated MR MAC CE or regular MR MAC CE is used before

Proposal 10: After transmitting truncated MR MAC CE, if there is at least one type00, type01 or type10 beam, triggered MR is not cancelled.

MAC-16: Whether the triggered MR should be cancelled in the below two cases:

Proposal 11: If a beam which triggered the MR due to entering (or leaving) condition with TTT satisfies leaving (or entering) condition with TTT before receiving UL grant, cancel the triggered MR.