# 3GPP TSG-RAN WG2 #130	R2-2504643

# Malta, MT, 19 – 23 May 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on the LP-WUS deactivation with RRC dedicated message

Document for:	Discussion and decision

Agenda Item:	8.4.2

## Introduction

On LP-WUS activation/deactivation it was discussed in RAN2 #129bis[1] that the two solution RRC dedicated signal vs NAS.

------

R2-2502659	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

Noted

Proposal 2: Support dedicated LP-WUS monitoring (de-)activation per UE via RRC release message.

R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

Noted

Proposal 3: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signalling; the absence of indication means UE is allowed to use LP-WUS functionality

One important use case for the operator is that the emergency call back. RAN2 sent an LS [2] to confirm SA2's understanding of the operation of emergency call during eDRX, and SA2 sent reply LS [3]. This LS exchange concerns eDRX, which allows the NW to indicate whether to use eDRX to the UE via RRC. It is assumed to align the solution for LP-WUS with eDRX

## Discussion

Regarding to the Reply LS on emergency call back and paging state, SA2 provide the view as below:

SA2 analysed the identified issue above and would like to provide the following feedback:

- Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node.

- In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10.

Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN.

We believe that LP-WUS activation/deactivation handling is tightly related to gNB implementation to control the Idle/Connected. The decision of whether or not to transition the UE to Idle mode is determined by the implementation of the gNB. In the implementation of the gNB, for example, the decision to transition to Idle mode may be based on considerations such as the resource utilization status of the gNB or the availability of other frequency bands. When deactivating LP-WUS under NAS control, these considerations cannot be taken into account, which may result in the unnecessary deactivation of LP-WUS.

Observation 1: NAS control cannot take the RAN implementation for the Idle/connected into account.

Observation 2: To enable the gNB to implement an appropriate LP-WUS deactivation with taking RAN implementation into account, it should be possible to deactivate LP-WUS via the dedicated RRC signal i.e. RRC Connection Release.

Proposal 1: gNB control the UE to deactivate UE LP-WUS capability with dedicated signal

## Summary and proposal

Observation 1: NAS control cannot take the RAN implementation for LP-WUS deactivation into account.

Observation 2: To enable the gNB to implement an appropriate LP-WUS deactivation with taking RAN implementation into account, it should be possible to deactivate LP-WUS via the dedicated RRC signal i.e. RRC Connection Release.

Proposal 1: gNB control the UE to deactivate UE LP-WUS capability with dedicated signal

## References

[1]	Draft_RAN2_129bis_Meeting_Report_v3

[2]	R2-2411041 LS on emergency services and eDRX

[3] 	R2-2501753/ S2-2502427 Reply LS on emergency call back and paging