3GPP TSG-RAN WG2 #130	R2-2504654

# St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.9.2

Source:	Ericsson

Title:	Support for store and forward in IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

Based on the conclusions from SA2 study report [1], the following objectives were defined in RAN for store and forward in the related work item [2]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

In this contribution, we aim to address the most pressing points.

# 2	Discussion

## 2.1	Satellite IDs in the NAS provided list

During RAN2#128, there was extensive discussion on the interpretation of the satellite ID list provided to the UE via NAS and the related UE behaviour. Some companies found the SA2 study inconclusive regarding whether the satellites in the list either support or operate in S&F mode when serving the UE. Consequently, it was agreed to send an LS to SA2 [3] to confirm the following understanding:

RAN2 would like to request SA2 to provide feedback on whether the following RAN2 understanding can be confirmed:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

RAN2 has received an LS response [4] which confirms RAN2’s understanding and further clarifies the intention with the S&F NAS wait timer and the satellite monitoring list [5]. In summary, its use highly depends on the scenario and SA2 has decided to leave it up to UE implementation. Although it is not advisable due to an expected worse performance, a UE is not prevented to attempt to access other satellites that are not in the monitoring lists.

NOTE A:	The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.

NOTE B:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

In previous meetings, some companies have suggested that a similar UE behaviour as in discontinuous coverage could be adopted for store and forward. In TS 36.304, “if the UE has determined that it is out of coverage [...]”, “the UE need not perform any idle mode tasks related to NTN”. However, as shown in the excerpt above, the intention of SA2 with the monitoring list is informative rather than normative. In contrast to discontinuous coverage case, where there is no any other alternative for NTN connection, S&F does not justify that the UE stops executing its idle mode tasks. Certainly, if a UE is does not expect to receive MT data from a specific satellite in S&F mode, other enhancements such to paging monitoring or extended power modes could still be discussed.

A UE continues executing its NTN idle mode tasks related to S&F operation (e.g., cell (re)selection, paging monitoring) even when it is not served by a satellite in the NAS monitoring list.

## 2.2	Neighbour and upcoming satellites

Similar to the NAS satellite monitoring list, it would be beneficial for the UE in AS level to have access to the operation mode of not only its serving satellite but also current neighbouring satellites. For neighbouring satellite, this information may be included in SIB33, along with their respective satellite ID. This additional information can enhance the efficiency of various AS procedures, such as cell re-selection. For example, depending on the applications layer necessities, a UE that is located close to the cell border may prioritize the reselection to a cell which does not operate in S&F mode.

- A UE may benefit from knowing the S&F operation status of neighbor satellites to steer mobility procedures.

One solution to assist mobility mechanisms which aligned with existing specifications is to consider the satellite/cell operation mode (S&F/non-S&F) in the cell reselection criteria. In this way, the UE may permanently or temporally change the cell ranking criteria taking into consideration the remaining time in S&F operation mode of the serving and/or neighbour cells.

The network informs the UE of the operation mode (S&F/non-S&F) of neighbour cells/satellites.

A new condition is introduced in cell reselection to consider the satellite operation mode (S&F/non-S&F).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A UE may benefit from knowing the S&F operation status of neighbor satellites to steer mobility procedures.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE continues executing its NTN idle mode tasks related to S&F operation (e.g., cell (re)selection, paging monitoring) even when it is not served by a satellite in the NAS monitoring list.

Proposal 2	The network informs the UE of the operation mode (S&F/non-S&F) of neighbour cells/satellites.

Proposal 3	A new condition is introduced in cell reselection to consider the satellite operation mode (S&F/non-S&F).

# 4	References

TR 3GPP TR 23.700-29 v19.0.0

RP-241624 - WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

R2-2411197, LS on Satellite IDs for store-and-forward operation, CATT, RAN2#128, Orlando, USA, November 2024.

S2-2502450

S2-2502450