3GPP TSG-RAN WG2 Meeting #130	R2-2504671
Malta, May 19th – 23rd, 2025

Agenda Item:	9.1

Source: 		Vice Chairman (Samsung)

Title: 			Report from session on R18 SL, R18/19 MOB and R19 NES

Document for:	Approval

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

## NBC Changes for Rel-18 corrections

R2-2503408	Handling of the default SRB configuration upon LTM execution	CATT	CR	Rel-18	38.331	18.5.1	5285	2	F	NR_Mob_enh2-Core	R2-2503008

R2-2504725	Correction on the maximum number of SSB rsources for L1 measurement without gaps in LTM	vivo, Ericsson, Xiaomi, ZTE Corporation	CR	Rel-18	38.331	18.5.1	5302	4	F	NR_Mob_enh2-Core

R2-2504143	Handling of PUCCH resources for L1 LTM reports at TAT expiry	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5332	3	F	NR_Mob_enh2-Core	R2-2503009

R2-2504567	Corrections on eEMR/IMR and LTM L2 Reset/UE-based TA measurement	Samsung, Ericsson	CR	Rel-18	38.331	18.5.1	5334	3	F	NR_Mob_enh2-Core	R2-2504549

R2-2504887	Handling of radio and RLC bearers at LTM cell switch execution in NR-DC	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5383	-	F	NR_Mob_enh2-Core

## List and Status of Offline/Email Discussions

POST Email discussion:

[POST130][113][SL] (ZTE)

Scope: Further check and complete CRs on introduction of additionalSpectrumEmission.

Intended outcome: CR in R2-2504707/R2-2504708/R2-2504709/R2-2504710.

Deadline: Short email discussion.

[POST130][118][NES] (Apple)

Scope: Prepare a LS to RAN4 to inform RAN2 agreements on RRM measurements.

Intended outcome: LS in R2-2504728.

Deadline: Short email discussion.

[POST130][116][MOB] (CATT)

Scope: Prepare LS to RAN3 to inform SP CSI-RS agreement.

Intended outcome: LS in R2-2504727.

Deadline: Short email discussion.

[POST130][107][NES] (Ericsson)

Scope: Update 38.331 running CR (also including this meeting’s agreements and latest other RAN WGs’ inputs) and remaining essential RRC open issues (including to continue discussion and make conclusion on P7, P12, P13, and P14 from R2-2504704).

Intended outcome: 38.331 running CR and remaining essential RRC open issues (including discussion summary on P7, P12, P13 and P14 from R2-2504704).

Deadline: Long email discussion.

[POST130][108][NES] (Apple)

Scope: Update 38.304 running CR (also including this meeting’s agreements and latest other RAN WGs’ inputs) and remaining essential idle/inactive open issues.

Intended outcome: 38.304 running CR and remaining essential idle/inactive open issues.

Deadline: Long email discussion.

[POST130][109][NES] (IDC)

Scope: Update 38.321 running CR (also including this meeting’s agreements and latest other RAN WGs’ inputs) and remaining essential MAC open issues.

Intended outcome: 38.321 running CR and remaining essential MAC open issues.

Deadline: Long email discussion.

[POST130][119][NES] (Huawei)

Scope: Update 38.300 running CR.

Intended outcome: 38.300 running CR.

Deadline: Long email discussion.

[POST130][110][MOB] (Ericsson)

Scope: Update 38.331 running CR (also including this meeting’s agreements and latest other WGs’ inputs) and remaining essential RRC open issues. Note RRC parts for L1 event-triggered MR will be merged into this CR after this meeting.

Intended outcome: 38.331 running CR and remaining essential RRC open issues.

Deadline: Long email discussion.

[POST130][111][MOB] (Vivo)

Scope: Update 38.321 running CR (also including this meeting’s agreements and latest other WGs’ inputs) and remaining essential MAC open issues.

Intended outcome: 38.321 running CR and remaining essential MAC open issues.

Deadline: Long email discussion.

[POST130][120][MOB] (CATT)

Scope: Update UE capability running CRs if needed (also including the latest other WGs’ inputs). And identify and discuss remaining UE capability related open issues.

Intended outcome: 38.306 and 38.331 running CR and discussion summary (if needed).

Deadline: Long email discussion.

[POST130][117][MOB] (Nokia)

Scope: Discuss issues of coexistence between L1 event triggered MR and mTRP (source cell).

- Whether to allow coexistence between event triggered MR and mTRP (source cell)?

- If companies agree to allow it, how to define the current beam?

Intended outcome: Summary discussion.

Deadline: Long email discussion.

AT Email discussion:

[AT130][101][MOB] (Vivo)

Scope: Discuss and attempt to make conclusion on MAC-5/13/14/15/16/17.

Intended outcome: Discussion summary in R2-2504701.

Deadline: Wednesday (9:40-10:40, main room) -> Comeback in CB session -> Completed.

[AT130][102][MOB] (Ericsson)

Scope: Discuss and attempt to make conclusion on RRC-2/3/6/7/9/10.

Intended outcome: Discussion summary in R2-2504702.

Deadline: Tuesday (17:00-18:00, main room) -> Comeback in CB session -> Completed.

[AT130][115][MOB] (MediaTek)

Scope: Offline discussion on RAN4 LS (R2-2503325) related corrections and attempt to propose consolidated one.

Intended outcome: CR in R2-2504726.

Deadline: Comeback in CB session -> Completed.

[AT130][103][NES] (Ericsson)

Scope: Prepare response LS to RAN1

Intended outcome: Response LS in R2-2504703.

Deadline: Wednesday 19:00 -> Email approval -> Completed.

[AT130][104][NES] (Ericsson)

Scope: Discuss and attempt to make conclusions on the following issues:

- FFS on SIBX

  - FFS: if list of cells is ARFCN&PCI or only PCI.

- FFS on DownlinkConfigCommonSIB

  - FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

  - FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

- P6 in R2-2504037

Intended outcome: Discussion summary in R2-2504704.

Deadline: Comeback in CB session. 30min f2f offline discussion -> Completed.

[AT130][105][NES] (Apple)

Scope: Discuss and attempt to make conclusions on the following issues:

- FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

- FFS whether to explicitly capture the failure case of OD-SIB1 window expiry in 38.304.

Intended outcome: Discussion summary in R2-2504705.

Deadline: Comeback in CB session. 30min f2f offline discussion -> Completed.

[AT130][106][NES] (IDC)

Scope: Discuss and attempt to make conclusions on the following issues:

- Issue 2: Whether there is a need to capture selection/availability of additional RACH resources for RACH adaptation.

Intended outcome: Discussion summary in R2-2504706.

Deadline: Comeback in CB session. 30min f2f offline discussion -> Completed.

[AT130][112][NES] (OPPO)

Scope: Discuss whether there is further specification impact to support feature and feature combination, and how to solve the issue if any.

Intended outcome: Discussion summary in R2-2504718

Deadline: Comeback in CB session. 30min f2f offline discussion -> Completed.

[AT130][114][NES] (Nokia)

Scope: Prepare the LS to RAN1 to inform RAN2 agreement on RA-RNTI.

Intended outcome: LS in R2-2504724.

Deadline: Wednesday 18:00 -> Email approval -> Completed.

## Approved outgoing LSs

R2-2504703	LS reply on DCI-based PRACH adaptation	LS out	Rel-19	Netw_Energy_NR_enh-Core	To:RAN1

R2-2504724	LS on RA-RNTI for PRACH adaptation	LS out	Rel-19	Netw_Energy_NR_enh-Core	To:RAN1

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

R2-2504428	MAC correction on Release-17 Sidelink enhancement	LG Electronics Inc.	CR	Rel-17	38.321	17.12.0	2086	-	F	NR_SL_enh-Core

R2-2504432	MAC correction on Release-17 Sidelink enhancement	LG Electronics Inc.	CR	Rel-18	38.321	18.5.0	2089	-	A	NR_SL_enh-Core

R2-2504428 and R2-2504432 are agreed.

### 7.0.2	Rel-18 corrections

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.  Coordinate with rapporteurs and chair if input above limit is required

Tdoc limitation: 7

#### 7.0.2.0	In-principle agreed CRs

R2-2503718	Correction on MCSt Restriction for Resource Selection	Apple	CR	Rel-18	38.321	18.5.0	2060	1	F	NR_SL_enh2-Core	R2-2502185

Agreed.

R2-2504008	Correction on sidelink RRC specification	ZTE Corporation, Sanechips, Ericsson	CR	Rel-18	38.331	18.5.1	5330	2	F	NR_SL_enh2-Core	R2-2503010

Agreed.

R2-2504425	MAC correction on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.5.0	2057	1	F	NR_SL_enh2-Core	R2-2502062

Agreed.

R2-2503408	Handling of the default SRB configuration upon LTM execution	CATT	CR	Rel-18	38.331	18.5.1	5285	2	F	NR_Mob_enh2-Core	R2-2503008

Agreed.

R2-2503628	Correction on UE capability for L1-RSRP measurement in LTM	vivo, Ericsson, Xiaomi, ZTE Corporation	CR	Rel-18	38.306	18.5.0	1249	2	F	NR_Mob_enh2-Core	R2-2503016

Agreed.

R2-2503629	Correction on the maximum number of SSB rsources for L1 measurement without gaps in LTM	vivo, Ericsson, Xiaomi, ZTE Corporation	CR	Rel-18	38.331	18.5.1	5302	3	F	NR_Mob_enh2-Core	R2-2503021

The changes in maxL1-MeasNoGapSCG should be moved to maxSSBsL1-MeasNoGapSCG.

Will be revised in R2-2504725.

Will be comeback in CB session.

[Vivo]: Indicates the changes in maxL1-MeasNoGapSCG should be moved to maxSSBsL1-MeasNoGapSCG. [Huawei]: Some changes are not about LTM.

R2-2504725	Correction on the maximum number of SSB rsources for L1 measurement without gaps in LTM	vivo, Ericsson, Xiaomi, ZTE Corporation	CR	Rel-18	38.331	18.5.1	5302	4	F	NR_Mob_enh2-Core

Agreed.

R2-2504122	Capability Coordination for LTM	Ericsson, Nokia	CR	Rel-18	38.331	18.5.1	5363	-	F	NR_Mob_enh2-Core	Withdrawn

R2-2504127	Capability coordination for LTM	Ericsson, Nokia	CR	Rel-18	38.331	18.5.1	5305	2	F	NR_Mob_enh2-Core	R2-2503018

R2-2504695	Capability coordination for LTM	Ericsson, Nokia	CR	Rel-18	38.331	18.5.1	5305	3	F	NR_Mob_enh2-Core	R2-2504127

[Ericsson]: R2-2504695 has small updates (cover page and editorial corrections). [Huawei]: “11.4” should be added into Clauses affected.

“11.4” should be added into Clauses affected.

Agreed with the change above in R2-2504729.

R2-2504143	Handling of PUCCH resources for L1 LTM reports at TAT expiry	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5332	3	F	NR_Mob_enh2-Core	R2-2503009

Agreed.

R2-2504534	Corrections on eEMR/IMR and LTM L2 Reset/UE-based TA measurement	Samsung, Ericsson	CR	Rel-18	38.331	18.5.1	5380	-	F	NR_Mob_enh2-Core	Withdrawn

R2-2504549	Corrections on eEMR/IMR and LTM L2 Reset/UE-based TA measurement	Samsung, Ericsson	CR	Rel-18	38.331	18.5.1	5334	2	F	NR_Mob_enh2-Core	R2-2503020

=> Revised in R2-2504567

R2-2504567	Corrections on eEMR/IMR and LTM L2 Reset/UE-based TA measurement	Samsung, Ericsson	CR	Rel-18	38.331	18.5.1	5334	3	F	NR_Mob_enh2-Core	R2-2504549

Agreed.

#### 7.0.2.20	NR Sidelink evolution

(NR_SL_enh2-Core; leading WG: RAN1; REL-18; WID: RP-230077)

R2-2503332	Reply LS to RAN2 on AdditionalSpectrumEmission (R4-2505217; contact: ZTE)	RAN4	LS in	Rel-18	NR_SL_enh2-Core	To:RAN2	Cc:RAN1

Noted.

R2-2504458	Discussion on implementation of AdditionalSpectrumEmission	ZTE Corporation, Sanechips	discussion	Rel-18	NR_SL_enh2-Core

Proposal 1.	The AdditionalSpectrumEmission is associated to the SL frequency included in SL-FreqConfig for Dedicated signaling and SL-FreqConfigCommon for SIB and Pre-configuration.

Proposal 2.	AdditionalSpectrumEmission having 0-7 for R16/R17 sidelink, and AdditionalSpectrumEmission-r18 having 0-39 for R18 sidelink.

Proposal 3.	It is suggested to discuss following two options:

Option1: Capture AdditionalSpectrumEmission in legacy Sidelink IEs, see example in annex 5.2.1

Option2: Capture AdditionalSpectrumEmission in lateNonCriticalExtension, see example in annex 5.2.2

Proposal 4.	RAN2 is suggested to discuss whether we need to use CR with early implementation and which CR (R17 or R18) is baseline for early implementation.

[ZTE]: Prefer implementing it from R17 with early implementation. [OPPO, Huawei, Ericsson]: Prefer implementing it from R16 as requested from RAN4.

Will implement it from Rel-16.

[OPPO]: Think option 2 can work and it is considered as NBC. [ZTE]: How to handle preconfiguration with option2? Consider it is not possible to apply option2 into preconfiguration.

If BC change is possible from R16 for SIB, dedicated configuration, preoconfiguration, we’ll go towards option2, otherwise we’ll go towards option1.

[CB session]:

[ZTE]: Suggest we agree with option2, endorse the provided CRs as baseline, and have short email discussion for the detailed check.

Option 2 (capture AdditionalSpectrumEmission in lateNonCriticalExtension, see example in annex 5.2.2) is agreed.

R2-2504904	Introduction of additionalSpectrumEmission in SL	ZTE Corporation, Sanechips, OPPO, Xiaomi, Ericsson, LG, Apple, Huawei, HiSilicon	CR	Rel-15	38.331	15.28.0	5390	-	F	NR_SL_enh2-Core

R2-2504719	Introduction of additionalSpectrumEmission in SL	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.19.0	5384	-	A	NR_SL_enh2-Core

R2-2504720	Introduction of additionalSpectrumEmission in SL	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.12.0	5385	-	A	NR_SL_enh2-Core

R2-2504721	Introduction of additionalSpectrumEmission in SL	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.5.1	5386	-	A	NR_SL_enh2-Core

CRs in R2-2504904, R2-2504719, R2-2504720, and R2-2504721 are endorsed.

[POST130][113][SL] (ZTE)

Scope: Further check and complete CRs on introduction of additionalSpectrumEmission.

Intended outcome: CR in R2-2504707/R2-2504708/R2-2504709/R2-2504710.

Deadline: Short email discussion

R2-2504433	Correction on MCSt for resource selection	LG, Ericsson	CR	Rel-18	38.321	18.5.0	2090	-	F	NR_SL_enh2-Core

[OPPO]: Is the sentence “(if reselected single slot candidate resources based on the above procedure are consecutive)” something beyond RAN2 agreement, or still aligned with RAN2 agreement? [LG]: It clarifies the single slot-based resource reselection mechanism, but ok to remove this sentence if it brings some confusion. [Apple, Vivo]: Ok with the removal. [Xiaomi]: Think it is good to have clear clarification.

Remove “(if reselected single slot candidate resources based on the above procedure are consecutive)”

Agreed with the change above in R2-2504722.

R2-2504438	Correction on TX carrier (re-)selection	LG Electronics Inc.	CR	Rel-18	38.321	18.5.0	2091	-	F	NR_SL_enh2-Core

Agreed.

R2-2504539	Correction on SL-U Power Class Capability	Apple, OPPO	CR	Rel-18	38.306	18.5.0	1298	-	F	NR_SL_enh2-Core	Late

[Huawei]: In table, “-r18” should be removed from sl-PowerClassUnlicensed-r18. Cover page needs to be also updated accordingly.

In Table A.4-1, “-r18” should be removed from sl-PowerClassUnlicensed-r18.

Cover page is updated accordingly.

Agreed with the change above in R2-2504723.

R2-2503722	Dummy sl-PowerClassUnlicensed capability	Apple	CR	Rel-18	38.331	18.5.1	5345	-	F	NR_SL_enh2-Core	Withdrawn

#### 7.0.2.22	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

R2-2503325	Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18 (R4-2504940; contact: MediaTek)	RAN4	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

Noted.

R2-2504123	Clarification of pdcch-RACH-AffectedBandsList-r18 and related capabilities	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1	pdcch-RACH-AffectedBandsList-r18 field description is updated to clarify that:

•	the feature set entry where the UE includes this capability corresponds to the serving cell that may be impacted during PDCCH-ordered RACH transmission;

•	the feature set entry includes a list where each entry in the list corresponds to a target band for RACH transmission;

•	PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells.

Proposal 2	pdcch-RACH-AffectedBandsList field name is updated to pdcch-RACH-TargetBandsList.

Proposal 3	pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 field descriptions are updated to clarify that:

•	the feature set entry where the UE includes this capability corresponds to the serving cell that may be impacted during PDCCH-ordered RACH transmission;

•	the feature set entry includes a list where each entry in the list corresponds to a target band for RACH transmission;

Proposal 4	pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 field descriptions are updated to clarify that when an entry in the list is set to notSupported, the UE does not support the feature for that target band. If the entire field is missing, the UE does not support the feature in any target band.

Proposal 5	The capability values for pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 must be set consistently.

Proposal 6	RAN2 to agree on the CR provided in Annex.

[Ericsson]: Indicates there was some offline discussion. We can comeback with more consolidate TP. [Huawei, Apple]: Think “PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells.” is not really correct. And there is some difference between switching time and preparation time so we should not have same descriptions.

[AT130][115][MOB] (MediaTek)

Scope: Offline discussion on RAN4 LS (R2-2503325) related corrections and attempt to propose consolidated one.

Intended outcome: CR in R2-2504726.

Deadline: Comeback in CB session -> Completed.

R2-2504726	Clarification on PDCCH ordered RACH capabilities for LTM	MediaTek Inc.,Ericsson, Nokia, Huawei	CR	Rel-18	38.306	18.5.0	1274	1	F	NR_Mob_enh2-Core

Agreed.

R2-2503471	Clarification on source band definition and UE reporting unsupported target band in pdcch-RACH-AffectedBandsList-r18	MediaTek Inc.	CR	Rel-18	38.306	18.5.0	1274	-	F	NR_Mob_enh2-Core

Covered by discussion above.

R2-2503738	Correction to LTM cell switch failure handling	Google	CR	Rel-18	38.331	18.5.1	5346	-	F	NR_Mob_enh2-Core

[Ericsson]: This could be an issue only if the timer T304 expires before the RACH procedure fails, which does not sound a very common case, considering that here we are talking about CFRA. Further, some to double check is how UE treat information received within a MAC CE. The understanding is that such information are typically “one-shot” information, and thus they are not stored by the UE (or re-used later). Finally, we have agreed that for the case of LTM recovery only CBRA is used and therefore the issue described should never happen. [Xiaomi]: Support the proposal. [NEC]: CFRA resource will be released during RRC reestablishment. [Google]: Understand CFRA resource is not released during MAC reset. [MediaTek, Apple, CATT]: This information is included in MAC CE, it is clear that the UE does not maintain that forever. [Qualcomm]: Agree with Ericsson. [CATT]: Think it is not really LTM specific issue.

Not pursued.

R2-2503743	Data loss issue on fast LTM recovery	Fujitsu	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: RAN2 to confirm the data loss issue caused by fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure needs to be solved.

Proposal 2: If RAN2 confirms that the solution in Observation 2 is reasonable, RAN2 to decide either add a NOTE to TS 38.331 or TS 38.300.

[Ericsson]: It seems that this is a corner case, and one of the proposals is anyway to leave this to the network implementation. So probably nothing is needed from this point of view. Also, this does not seems a correction but rather an enhancement to the current LTM procedure. [Huawei]: Think it is not common case and no optimization is needed. [MediaTek, Xiaomi]: Do not support the proposal.

Not pursued.

R2-2504136	Corrections to Rel-18 LTM (interruptions during RACH and LTM Recovery to the source cell)	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 2: RAN2 to confirm that LTM Fast recovery procedure is not applicable if the selected cell for recovery is same as current serving cell.

[Ericsson]: So far, we did not forbid the case where the UE select the source cell when performing the fast LTM recovery procedure and I don’t think that this causes confusion at the network, since the network knows which candidates has configured. One it may argue why a UE selects a cell which is the previous failed cell, but I think that we can rely on the UE to be smart enough. [Qualcomm]: Do not agree with proposal. [Huawei]: Do not understand what network issue really is. [ZTE]: Candidate configuration id is included in RRC connection reconfiguration and network is aware of which configuration used by the UE. Do not see an issue.

Noted.

R2-2504144	Handling of radio and RLC bearers in LTM with NR-DC	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Discuss how to handle RBs for LTM in NR-DC

a)	option 1: modify the UE behaviour: at MCG LTM cell switch execution, the UE may only release MCG RLC bearers (unless the SCG is explicitly released) and radio bearers using the master key, and at SCG LTM cell switch execution, the UE may only release SCG RLC bearers and radio bearers using the secondary key

b)	option 2: modify MN-SN signalling for:

-	the SN to ask the MN to always provide the full MN RB + MCG configuration to the SN for SCG LTM, and the MN to provide them to the SN

-	the MN to ask the SN to always provide the full SN RB + SCG configuration to the MN for MCG LTM, and the SN to provide them to the MN

and assume that the MN/SN includes/updates them in the LTM reference or candidate configurations.

Proposal 2: If option 1 is selected, agree the draft CR in annex.

[MediaTek, CATT, Samsung, Ericsson, Nokia]: Option 1 was actually original intention in Rel-18 discussion. Support option 1. [Qualcomm]: Think it is NBC change.

Option1 is agreed.

Detailed wordings/changes can be further discussed among interested companies. Revised in R2-2504887.

Comeback in CB session.

Proposal 3 (modified): The network ensures that keystream reuse cannot happen.

[MediaTek, Ericsson, ZTE]: Support the proposal 3. [ZTE]: Think the first DRB specific condition is not needed. Also the last sentence can be also removed since it is one of network solutions.

The network ensures that keystream reuse cannot happen.

R2-2504887	Handling of radio and RLC bearers at LTM cell switch execution in NR-DC	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5383	-	F	NR_Mob_enh2-Core

Agreed.

R2-2504124	Summary of RRC proposals for feMob	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core	Late

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST129b][111], [112], [113], [114], [121], summary of identified stage-3 open issues that need online discussion and rapporteur’s suggestions if needed).

R2-2503305	Reply LS on paging enhancement in R19 NES (C1-252548; contact: Huawei)	CT1	LS in	Rel-19	Netw_Energy_NR_enh-Core	To:RAN2	Cc:RAN3, RAN1

Noted.

R2-2503312	LS on DCI-based PRACH adaptation (R1-2503086; contact: Ericsson)	RAN1	LS in	Rel-19	Netw_Energy_NR_enh	To:RAN2

RAN2 confirms it is feasible.

R2-2504429	[DRAFT] Reply LS on DCI-based PRACH adaptation	Ericsson	LS out	Rel-19	Netw_Energy_NR_enh-Core	To:RAN1

[Nokia]: Wonder if bit 5 was already defined in other WIs.

Response LS will simply answer that RAN2 confirms it is feasible.

RRC CR rapporteur takes whether bit 5 or other bit into account with the consideration of other WIs’ progress.

[AT130][103][NES] (Ericsson)

Scope: Prepare response LS to RAN1

Intended outcome: Response LS in R2-2504703.

Deadline: Wednesday 19:00 -> Email approval -> Completed.

R2-2504703	LS reply on DCI-based PRACH adaptation	LS out	Rel-19	Netw_Energy_NR_enh-Core	To:RAN1

Approved.

R2-2503319	Reply LS on energy saving indication from CN to RAN (R3-252404; contact: Huawei)	RAN3	LS in	Rel-19	EnergySys	To:SA2	Cc:RAN2

Noted.

R2-2503504	Introduction of Network Energy Savings Enhancements running CR	Huawei, HiSilicon	draftCR	Rel-19	38.300	18.5.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2503607	Running RRC CR for enhancements for network energy efficiency 	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2503608	Report of [POST129b][111][NES] (Ericsson)	Ericsson	discussion	Rel-19

5.2.1

Noted.

[AT130][104][NES] (Ericsson)

Scope: Discuss and attempt to make conclusions on the following issues:

- FFS on SIBX

  - FFS: if list of cells is ARFCN&PCI or only PCI.

- FFS on DownlinkConfigCommonSIB

  - FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

  - FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

- P6 in R2-2504037

Intended outcome: Discussion summary in R2-2504704.

Deadline: Comeback in CB session. 30min f2f offline discussion -> Completed.

R2-2504704	[AT130][104] [NES] (Ericsson)	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Include ARFCN inside the list with the PCI to have a NES-CellId list associated to a OD-SIB1 config.

P1 is not proceeded.

Proposal 2: RAN2 to consider the case when network send a short message and whether the current RRC procedure where UE checks for SIB1 before requesting it is enough.

Keep RRC CR as it is for the short message and UE behaviour.

Proposal 3: Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging

Proposal 4: The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

Proposal 5: The maximum offset value is extended to 32 radio frames.

Proposal 6: The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

Proposal 3, 4, 5 and 6 are agreed.

Proposal 8: Discuss if a new pei-ConfigBWP IE is introduced for Rel-19 UEs that support adaptive paging, i.e., if a new PEI search space is needed.

A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging.

Proposal 9: Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging.

Proposal 10: The maximum number of PEI occasion per paging frame is extended to 8.

Proposal 11: The maximum offset value is extended to 32 radio frames.

Proposal 9, 10, and 11 are agreed.

Not discussed:

Proposal 7	Discuss whether the maximum offset value for firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to 32 radio frames

v.	at symbol level

vi.	at slot level

vii.	by restricting the maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO field parameter for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N)

viii.	by restricting the configuration to evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them.

Proposal 12	Discuss how to extend the maximum offset value for firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter is extended to 32 radio frames.

Proposal 13	For Rel-19 PEI configuration, introduce the following parameters: -po-NumPerPEI-r19, -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19,  -firstPDCCH-MonitoringOccasionOfPEI-O-R19

Proposal 14	For Rel-19 PEI configuration, use the same value ranges as for legacy PEI for the following parameters: -po-NumPerPEI-r19,  -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19

R2-2503708	Running 38.304 CR for network energy saving	Apple (Rapporteur)	draftCR	Rel-19	38.304	18.4.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2503709	Summary report of [POST129b][113][NES] 38.304 CR (Apple)	Apple (Rapporteur)	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal: It is suggested to discuss and conclude the following two open issues on running CR of TS 38.304 in RAN2#130:

•	Issue 1: FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

•	Issue 2: FFS whether to explicitly capture the failure case of OD-SIB1 window expiry in 38.304.

Noted.

[AT130][105][NES] (Apple)

Scope: Discuss and attempt to make conclusions on the following issues:

- FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

- FFS whether to explicitly capture the failure case of OD-SIB1 window expiry in 38.304.

Intended outcome: Discussion summary in R2-2504705.

Deadline: Comeback in CB session. 30min f2f offline discussion -> Completed.

R2-2504705	Summary report of [AT130][105][NES] (Apple)	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 (compromised solution): Rel-19 excluded cell list can be configured as an empty list. No new UE behavior is introduced (i.e. the UE supporting OD-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present).

Proposal 2 (compromised solution): Same as the other 2 agreed cases of OD-SIB1 acquisition failure, capture the failure case of OD-SIB1 window expiry in TS 38.304. RAN2 confirm no other cases need to be considered and specified in TS 38.304.

Proposal 1 and 2 are agreed.

R2-2503752	Summary of [POST129b][121] Initial discussion on UE capabilities in NES	ZTE Corporation, Sanechips	discussion	Netw_Energy_NR_enh-Core

Proposal 1a: The capability for paging adaption to be included in UE-RadioPagingInfo is a separate capability from other NES features and no need to define a common capability for all NES features.

Proposal 1b: Use a “container” using OCTET STRING with content generated by UE for paging adaption capability.

Proposal 2: The paging adaption capability is per UE.

Proposal 3: Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band.

Proposal 4: Wait for RAN4 input to see whether capability of L3 RRM with multiple SMTC for SSB adaptation is covered in the feature list or not. If not, RAN2 will define it.

The capability for paging adaption to be included in UE-RadioPagingInfo is a separate capability from other NES features and no need to define a common capability for all NES features.

Use a “container” using OCTET STRING with content generated by UE for paging adaption capability.

The paging adaption capability is per UE.

Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band.

R2-2503753	Draft 38.331 CR for NES UE capability	ZTE Corporation, Sanechips	draftCR	Rel-19	38.331	18.5.1	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2503754	Draft 38.306 CR for NES UE capability	ZTE Corporation, Sanechips	draftCR	Rel-19	38.306	18.5.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2504282	Running 38.321 CR for network energy saving	InterDigital	draftCR	Rel-19	38.321	18.5.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2504281	Summary of Comments to 38.321 CR for NES	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

The draft running CR has been updated according to the input in section 2. In addition, the following open issues are added; companies are encouraged to address these open issues in their tdoc submissions.

-	Issue 1: How to define single MAC CE format for both explicit and implicit OD-SSB deactivation

-	Issue 2: Whether there is a need to capture selection/availability of additional RACH resources for RACH adaptation.

Noted.

[AT130][106][NES] (IDC)

Scope: Discuss and attempt to make conclusions on the following issues:

- Issue 2: Whether there is a need to capture selection/availability of additional RACH resources for RACH adaptation.

Intended outcome: Discussion summary in R2-2504706.

Deadline: Comeback in CB session. 30min f2f offline discussion -> Completed.

R2-2504706	[AT130][106][NES] Open issues for 38.321 CR (InterDigital)	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Rely on PHY layer specs to capture availability conditions related to usage of additional PRACH resources. It can be revisited if needed after checking PHY running specifications.

Proposal 2: A note in TS 38.321 is added to clarify that UE selects only amongst preambles configured for the next valid RO(s) for the selected SSB, when CB-PreamblesPerSSB is configured for additional ROs. Wording of the note can be refined during the running CR review discussion.

Proposal 3: The following two proposals were discussed and not pursued:

-	(P9 R2-2503712) If UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

-	(P8 R2-2503945) RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

Proposal 1, 2 and 3 are agreed.

[POST130][107][NES] (Ericsson)

Scope: Update 38.331 running CR (also including this meeting’s agreements and latest other RAN WGs’ inputs) and remaining essential RRC open issues (including to continue discussion and make conclusion on P7, P12, P13, and P14 from R2-2504704).

Intended outcome: 38.331 running CR and remaining essential RRC open issues (including discussion summary on P7, P12, P13 and P14 from R2-2504704).

Deadline: Long email discussion.

[POST130][108][NES] (Apple)

Scope: Update 38.304 running CR (also including this meeting’s agreements and latest other RAN WGs’ inputs) and remaining essential idle/inactive open issues.

Intended outcome: 38.304 running CR and remaining essential idle/inactive open issues.

Deadline: Long email discussion.

[POST130][109][NES] (IDC)

Scope: Update 38.321 running CR (also including this meeting’s agreements and latest other RAN WGs’ inputs) and remaining essential MAC open issues.

Intended outcome: 38.321 running CR and remaining essential MAC open issues.

Deadline: Long email discussion.

[POST130][119][NES] (Huawei)

Scope: Update 38.300 running CR.

Intended outcome: 38.300 running CR.

Deadline: Long email discussion.

For other running CRs, running CR updates can be directly provided by CR rapporteur next meeting if needed.

Agreements on stage-3 details

- Proposal (include ARFCN inside the list with the PCI to have a NES-CellId list associated to a OD-SIB1 config) is not proceeded.

- Keep RRC CR as it is for the short message and UE behaviour.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

- The maximum offset value is extended to 32 radio frames.

- The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

- A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum number of PEI occasion per paging frame is extended to 8.

- The maximum offset value is extended to 32 radio frames.

- Rel-19 excluded cell list can be configured as an empty list. No new UE behavior is introduced (i.e. the UE supporting OD-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present).

- Same as the other 2 agreed cases of OD-SIB1 acquisition failure, capture the failure case of OD-SIB1 window expiry in TS 38.304. RAN2 confirm no other cases need to be considered and specified in TS 38.304.

- The capability for paging adaption to be included in UE-RadioPagingInfo is a separate capability from other NES features and no need to define a common capability for all NES features.

- Use a “container” using OCTET STRING with content generated by UE for paging adaption capability.

- The paging adaption capability is per UE.

- Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band.

- Rely on PHY layer specs to capture availability conditions related to usage of additional PRACH resources. It can be revisited if needed after checking PHY running specifications.

- A note in TS 38.321 is added to clarify that UE selects only amongst preambles configured for the next valid RO(s) for the selected SSB, when CB-PreamblesPerSSB is configured for additional ROs. Wording of the note can be refined during the running CR review discussion.

- The following two proposals were discussed and not pursued:

- (P9 R2-2503712) If UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

- (P8 R2-2503945) RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

### 8.5.2	On-demand SSB SCell operation

Remaining essential open issues, including details of OD-SSB MAC CE (e.g. how to support implicit deactivation with single MAC CE format, etc., with the consideration of RAN1 progress), open issues for L3 RRM framework for case 1 and case 2, stage-3 identified open issues if needed, etc.

1. RRC configuration

R2-2503573	Discussion on on-demand SSB	Xiaomi	discussion	Netw_Energy_NR_enh-Core

Proposal 5: (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

(RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

2. MAC CE: General

R2-2504280	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2 (modified): The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index.

The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index.

[ZTE]: Size of configuration index is still FFS. [IDC]: Already captured it as editor’s note in TP.

Proposal 3: Adopt the text proposal to TS 38.321 in Annex A for the OD-SSB MAC CE.

TP will be baseline for the running CR. Detailed wordings can be further discussed.

3. MAC CE: What does “Ci=0” mean in implicit deactivation?

R2-2503392	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

•	OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

[LG]: With Samsung proposal, it will make the UE behaviour simple. [Xiaomi]: Understand it is not clear whether early deactivation is agreed in RAN1 so it would be better to agree as RAN2 assumes blabla. [ZTE]: We can decide it if RAN1 cannot make decision. How to use MAC CE is up to RAN2.

When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

- •	OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

R2-2503710	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: To support implicit deactivation of OD-SSB, UE re-interprets the bitmap field per SCell (i.e. Ci with SCellIndex i) in the following way:

•	If Ci=1, the UE regards OD-SSB is activated in the concerned SCell, applies the corresponding OD-SSB configuration indicated in the MAC-CE (i.e. set/reset od-ssb-nrofBurst) and resets counter to 0.

•	If Ci=0, the UE regards the OD-SSB configuration in the concerned SCell is not changed (i.e. od-ssb-nrofBurst is same as the one indicated by preceding MAC-CE) and continues the counter.

4. MAC CE: How to indicate “no change” in implicit deactivation?

R2-2504280	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the SCell Activation/Deactivation MAC CE:

R2-2503392	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: When UE receives MAC CE with Cj bit set to 1 for an SCell for which OD-SSB is currently activated:

•	if configuration index is different from the configuration index of the configuration currently applied for the activated OD-SSB, UE applies the new configuration based on new configuration index received in MAC CE. Otherwise, do nothing.

R2-2503407	Remaining issues on OD-SSB SCell operation	LG Electronics France	discussion	Rel-19	38.331	Netw_Energy_NR_enh-Core

Proposal 2: Introduce reserved value into additional octet to indicate that UE maintains previous OD-SSB configuration. FFS how to define reserved value.

[Huawei, Apple, OPPO, Xiaomi, Vivo]: Support IDC. [Nokia, CATT, Lenovo]: Support Samsung. [Ericsson, NTTDCM]: Support LG. [Apple]: Understand IDC proposal is similar to legacy behaviour. Other proposals are considered as more optimization. [LG, Ericsson]: With IDC proposal, it cannot really support maintain operation. [Vivo]: Network can configure, with the different configuration index, to have only different n burst transmissions. Then with IDC proposal, “maintain” can be also achieved. [Session chair]: That can work, but in the case, we need more configuration index ranges. [IDC]: As compromise, we’re ok with LG’s proposal. We have anyway reserved fields.

- IDC proposal: OPPO, ZTE, Apple, Xiaomi, IDC, Qualcomm, Vivo, Huawei, Samsung, Lenovo, CMCC

- Samsung proposal: Rakuten, Nokia, CATT

- LG proposal: LG, Ericsson, NTTDCM, Fujitsu

For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

5. L3 RRM framework: Case 1

R2-2503710	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4 (modified): In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

•	The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation

•	The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

[Xiaomi]: Understand we already agreed the principle. And wonder why it is restricted to MAC CE based activation/deactivation only? [Vivo]: Agree with the proposal in general, however do not think we need to mandate the UE when to stop measurement. [Nokia]: In general, ok with the proposal. However, wonder if we really need to capture it in RAN2 spec. Seems more related to RAN4. [Apple]: In legacy RRC, the UE’s measurement behaviour is specified. [Huawei]: We also have other open issue, e.g. whether the serving cell’s measurement will be done based on SMTC or OD-SSB transmission pattern. [OPPO]: Understand this proposal has no direct relation to the issue of SMTC / OD-SSB transmission pattern.

In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

- •	The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation

- •	The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

6. L3 RRM framework: Case 2a (AO-SSB and OD-SSB in the same frequency)

R2-2504540	Discussion on On-demand SSB	KDDI Corporation	discussion	Rel-19

Proposal 2 (modified): When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

Proposal 3 (modified): The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated.

[OPPO]: RAN4 already defined fast mode window for SCell activation, which is associated with OD-SSB transmission periodicity (not associated with SMTC). [Apple]: In RRC, the UE measurement behaviour is mainly specified for the activated SCell, and refer RAN4 spec for deactivated SCell. Support the proposal for the activated SCell and leave the deactivated SCell to RAN4. [Nokia, ZTE]: Support the modified proposal considering similar RAN2 agreement on SSB adaptation.

(modified) The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated.

Decision above does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

Will send a LS to RAN4 to check if they have any concern.

[POST130][118][NES] (Apple)

Scope: Prepare a LS to RAN4 to inform RAN2 agreements on RRM measurements.

Intended outcome: LS in R2-2504728.

Deadline: Short email discussion.

[Huawei]: The current conclusion sounds to provide misunderstanding to RAN4 that AO-SSB is not measured. Would like to have clear clarification to avoid any confusion to RAN4.

R2-2503376	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: For OD-SSB Case#2, for same-frequency scenario between AO-SSB and OD-SSB, R2 firstly conclude the support of L3 measurement configuration based on a single MO. And wait for further conclusion from R4 on whether to support C2.

R2-2503944	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: For both Case 1 (no always-on SSB) and Case 2 (always-on SSB), the measurements of OD-SSB is based on OD-SSB pattern rather than SMTC. For deactivated SCell, after the fast window, OD-SSB is measured based on measCycleSCell in combination with OD-SSB pattern.

7. L3 RRM framework: Case 2b (AO-SSB and OD-SSB in the different frequency)

R2-2504386	Discussion on On-demand SSB for Scell	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

Proposal 7: Multiple OD-SSBs with the different frequencies for a given SCell is not supported.

RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

R2-2503944	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: On how to utilize the new servingCellMO, RAN2 to down-select from the following:

•	Option 1 (modified): When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

[OPPO]: Ok with the serving cell measurement, but not sure if it is correct for intra-F neighbouring cells. It seems to propose neighbouring cell measurement is also dynamically changed according to the serving cell situation.

When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

•	Option 2: Regardless of the activation status of OD-SSB, UE uses both servingCellMO-OD and servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell and intra-f neighbour cells. FFS how the network determines the serving cell measurement results come from which MO.

8. MAC CE triggered MR?

R2-2503609	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: RAN2 to support MAC CE based trigger of serving cell OD-SSB measurement report.

R2-2503376	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 12: R2 not pursue OD-SSB A/D MAC-CE triggered MR for OD-SSB.

[Nokia]: In Rel-18, MAC CE based MR triggering was introduced for unknown SCell. Not sure if we also need to introduce MAC CE based MR triggering to this case. [Xiaomi]: Agree with Nokia, understand for case 2a, we can rely on legacy MAC CE triggered MR. For case 2, we may need it but think it is a kind of optimization. Not to support MAC CE based triggered MR. [Apple]: Agree with Nokia and Xiaomi. Also, in R18, it was decided by RAN4. [OPPO]: Agree with Apple.

RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

Agreements on OD-SSB

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

- The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed.

- When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

• OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

- For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

R2-2503376	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503392	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503407	Remaining issues on OD-SSB SCell operation	LG Electronics France	discussion	Rel-19	38.331	Netw_Energy_NR_enh-Core

R2-2503414	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503573	Discussion on on-demand SSB	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2503609	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503710	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503755	Remaining issues of on-demand SSB SCell operation	ZTE Corporation, Sanechips	discussion	Netw_Energy_NR_enh-Core

R2-2503805	Remaining issues on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503824	Remaining issues of OD-SSB	Rakuten Mobile, Inc	discussion	Rel-19

R2-2503827	Discussion on on-demand SSB SCell operation	Sharp	discussion

R2-2503893	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503916	Issues on the procedure of on-demand SSB SCell operation	Lenovo	discussion	Rel-19

R2-2503944	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503988	Remaining issues on On-demand SSB for SCell	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504036	Remaining issues of on-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504061	Remaining details on OD-SSB MAC CE	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504242	Discussion on On-demand SSB SCell Operation	Qualcomm Incorporated	discussion

R2-2504280	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504386	Discussion on On-demand SSB for Scell	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504417	On-demand SSB SCell operation	Panasonic	discussion	Rel-19

R2-2504418	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.	discussion	Rel-19

R2-2504540	Discussion on On-demand SSB	KDDI Corporation	discussion	Rel-19

### 8.5.3	On-demand SIB1

Remaining essential open issues, including whether to include cell access information in UL-WUS configuration, stage-3 identified open issues if needed, etc.

1. Access information of NES cell

R2-2503894	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: Allow in the SIB1 request configuration to provide following information:

•	CellAccessRelatedInfo (includes PLMN/NPN and cell reservations)

-	if desired we can remove RANAC/CellIdentity/iab-support/gnb-ID-Length/mobile-IAB-Support/ snpn-AccessInfoList as those are not needed for camping. Then it makes sense to have different version of the IE

•	cellBarredATG

•	cellBarred-eRedCap1Rx

•	Barred-eRedCap2Rx

•	cellBarredNES

•	cellBarredNTN and corresponding VSAT barring bits

•	cellBarred-RedCap1Rx

•	cellBarred-RedCap2Rx

R2-2503636	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: Do not support including the cell access related information into the UL-WUS configuration.

R2-2503711	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: The following feature specific barring bits common to all PLMN/NPN (totally 14 bit) are optionally included in UL WUS configuraiton:

•	cellReservedForOtherUse, cellReservedForFutureUse, cellBarredATG, cellBarred2RxXR, cellBarred-eRedCap1Rx, cellBarred-eRedCap2Rx, cellBarredNES, cellBarredNTN, cellBarredFixedVSAT, cellBarredMobileVSAT, cellBarred-RedCap1Rx, cellBarred-RedCap2Rx, halfDuplexRedCapAllowed, ncr-Support

[ZTE, CATT]: Share concern with Ericsson. Understand cell barred information is more dynamically changed compared to LP-WUS. [Apple, Xiaomi]: Do not think it is dynamically changed due to load balance and also, it is defined as optional, if so, network can skip configuring this information. [Nokia]: Understand both arguments make sense, however think most of barring information is semi-static. [OPPO]: Want not to support this optimization. [Huawei]: As analysed in Apple contribution, 14bits overheads (per cell or per WUS configuration) are acceptable, and also it will be defined as optional. [Session chair]: We now have insight of signalling overhead, let’s see companies’ views on the inclusion of the minimum feature specific cell barred information.

  - Option-1: Include feature specific barring common to all PLMN/NPN: Qualcomm, Vivo, Fujitsu, Huawei, Apple, Xiaomi, Google, Rakuten, NEC, ASUSTeK (10)

  - Option-2: Not to include any access related information: ZTE, Nokia, Lenovo, Samsung, CATT, Ericsson, OPPO, ITRI, KDDI (9)

[Qualcomm]: We still have concern for RedCap/eRedCap UEs since they are very low power and low complexity UE. [Ericsson]: Think it sounds not natural RedCap/eRedCap UE supports OD-SIB1.

Not to include access related information of NES into WUS configuration.

2. Broadcasting status for OD-SIB1 request

R2-2504493	Broadcasting status for OD-SIB1 request	ASUSTeK	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal: RAN2 decides one of the following alternatives:

Alt. 1: RAN2 sticks to conduct a broadcasting status check by checking K_SSB in MIB of that cell before requesting SIB1of that cell.

Alt.2: RAN2 reverts the following agreement:

Agreements on OD-SIB1: If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Alt. 3 (modified): RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

[Fujitsu]: RAN1 discussed and decided whether/how to check broadcast status is up to UE implementation. [Ericsson]: Prefer keeping original agreement since it can avoid frequent OD-SIB1 requests in some cases. [Samsung]: In the current running CR, there are two conditions. 1st check is about Kssb and 2nd check is about whether SIB1 is actually transmitted or not. We don’t need the 2nd check. When RAN2 made decision on broadcast status check, we assumed based on Kssb. However, RAN1 discussed and decided whether and how to do that, and decided it is left to the UE implementation. Note checking broadcast status is anyway for small optimization.

Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed.

RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

3. SIBxx/UL WUS validity

R2-2503391	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Discuss and agree the following for SIB1 request to cell selected for connection re-establishment

-	UE may send SIB1 request to selected cell (say cell B), if the SIB X last acquired from PCell (say Cell A) includes SIB1 request configuration of Cell B and this SIB X is valid in current modification period.

[Nokia]: As idle/inactive, the simplest option would be to use the latest SIBX acquired from the serving cell for RLF case. [Lenovo]: Agree with Nokia. Although the WUS configuration can be changed/updated between T311 starts and T311 expires, it sounds the chance is slim. [Nokia]: We can have similar situation for idle/inactive, e.g. idle/inactive UE goes to OOS and back to IC. [Samsung]: If the UE uses invalid RACH resources, it may bring the interference to the system. [Vivo]: Note Nokia’s suggestion is same as what we agreed last meeting.

Noted.

R2-2503636	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: The UE considers that the UL WUS configuration it obtained from Cell A is valid within the NES Cell that the UE reselected right after Cell A until the NES Cell itself provides an update concerning the UL WUS configuration, i.e., irrespective of the validity of the Cell A’s SIBxx that contains the UL WUS configuration.

[OPPO]: After cell reselection to NES cell, value tag or area scope can be changed. Then the UE should reacquire SIBX in NES cell. [Samsung]: We agreed WUS configuration is applied to the first cell reselection only. Upon camping on NES cell as the result of the first cell reselection, the UE needs to acquire its own SIBx again. Of course, if same area scope and value tag (as previously acquired SIBx) are indicated, the UE can skip acquiring it. [Ericsson]: Let’s see the proposed TP. What is wrong? [Samsung, Nokia]: If we remove “until receiving SIBxx from this cell”, it is ok.

TP1 is agreed with the removal of “until receiving SIBxx from this cell”.

4. Handling of RRC connected UE

R2-2503839	Remaining issues on OD-SIB1 operation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6 (modified): Add the UE behaviour in 5.2.2.3.5:

The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

Agreed.

R2-2503351	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1a: If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

Agreed.

5. Need to emphasize it is normal uplink?

R2-2503391	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1a: SIB 1 request is supported for both SUL and NUL.

Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration.

[ZTE]: Probably no much impact on RAN2, but it was excluded in RAN1 decision. [Huawei]: Want to follow RAN1 discussion and decision. [Samsung]: Understand RAN1 actually has never discussed specific SUL aspect. [ZTE]: Understand RAN1 clearly excluded SUL. [IDC]: Support the proposal. [Xiaomi]: Share the view with Samsung. [LG]: Would like to have more time to check.

Revisit it next meeting.

6. RAN1 Parameters in SIBxx

R2-2503756	Remaining issues of on-demand SIB1	ZTE Corporation, Sanechips	discussion	Netw_Energy_NR_enh-Core

Proposal 1: Include the following parameters from the RAN1 parameter list in SIBxx:

-	frequencyBandList

-	Prach-ConfigurationIndex

-	msg1-FDM

-	msg1-FrequencyStart

-	zeroCorrelationZoneConfig

-	preambleReceivedTargetPower

-	powerRampingStep

-	ra-ResponseWindow

-	searchSpaceZero

-	controlResourceSetZero

-	ra-SearchSpace

-	n-TimingAdvanceOffset

-	ssb-PeriodicityServingCell

-	od-sib1-WindowDuration

-	od-sib1-windowStartOffset

Agreed.

[Ericsson]: Note that they are already captured in the latest revised RRC running CR.

Agreements on OD-SIB1

- Not to include access related information of NES into WUS configuration.

- Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

- Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

- If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

R2-2503351	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503391	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503415	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503467	Remaining issues for On-demand SIB1 request and UE behaviour	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503485	Providing Cell Reselection Assistance information	Lenovo	discussion	Netw_Energy_NR_enh-Core

R2-2503502	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503636	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503711	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503756	Remaining issues of on-demand SIB1	ZTE Corporation, Sanechips	discussion	Netw_Energy_NR_enh-Core

R2-2503797	Open issues in the running CRs for OD-SIB1	Google	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503806	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503823	Remaining issues of OD-SIB1	Rakuten Mobile, Inc	discussion	Rel-19

R2-2503839	Remaining issues on OD-SIB1 operation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503894	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503955	Discussion on SIBxx validity check during T311	ITRI, Acer Incorporated	discussion	Netw_Energy_NR_enh-Core

R2-2503999	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504037	Remaining issues on OD-SIB1	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504054	Remaining details for on-demand SIB1	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504243	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

R2-2504387	Discussion on On-demand SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504419	Discussion on on-demand SIB1	NTT DOCOMO, INC.	discussion	Rel-19

R2-2504470	Discussion on on-demand SIB1	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504493	Broadcasting status for OD-SIB1 request	ASUSTeK	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504533	Discussion on On-demand SIB1	KDDI Corporation	discussion	Rel-19

R2-2504538	Discussion on Ondemand-SIB1	KDDI Corporation (TTC)	discussion	Rel-19	Late

R2-2504604	Reducing On-demand SIB1 overhead	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2504606	Remaining issues on OD-SIB1 Request	SHARP Corporation	discussion	Rel-19

R2-2504278	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core	Withdrawn

### 8.5.4	Adaptation of common signal/channel transmissions

Remaining essential open issues, including post email discussion [122], additional RO configuration, RA-RNTI calculation, with consideration of RAN1 progress, stage-3 identified open issues if needed, etc.

1. RACH: 2-step RA applicability

R2-2503553	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2 (modified): Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19 unless RAN1 decides supporting 2-step RACH this meeting.

[Nokia, ZTE, Ericsson]: Think there is no RAN2 impact, so it should leave it to RAN1. [Apple, OPPO, Qualcomm, Samsung]: Think there is RAN2 impact, e.g. fallback between 4step RACH and 2step RACH. Support the proposal. [OPPO]: We already see complication on additional RACH configuration and understand it needs some further impact on configuration aspect. [Samsung]: For 2-step RACH, in addition to additional RO, we also need separate PUSCH. Note all PUSCH design has been done in RAN1.

Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19 unless RAN1 decides supporting 2-step RACH this meeting.

2. RACH: Additional RO configuration

R2-2503712	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

•	Alt-1: addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

•	Alt-2: addl-RACH-Config-Adaptation is placed under rach-ConfigGeneric

[Nokia, Ericsson]: Prefer Alt-2 since rach-ConfigGeneric is applied to all cases. [LG, OPPO]: It doesn’t make sense to include ssb-perRACH-OccasionAndCB-PreamblesPerSSB under rach-ConfigGeneric. [Apple]: RAN2 already agreed in some cases (e.g. IAB), RACH adaptation is not applied, but with alt-2, it can be applied to them as well if we follow the current spec. [Ericsson]: Understand the raised issues for Alt-2, however consider it’s not complicated to fix it. This decision may impact CFRA discussion. [OPPO]: We need this decision first, otherwise it will be difficult to make progress on CFRA. [Samsung]: Do not see any blocking issue with this agreement even for CFRA. If needed, additional RO configuration can be included in CFRA.

- Alt-1: LG, Samsung, OPPO, Apple, Qualcomm, CATT, Vivo, Huawei

- Alt-2: Nokia, Ericsson, NTTDCM, ZTE

addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

Proposal 6: Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1.

Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1.

Proposal 7: addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

R2-2504579	Remaining issues on common signal and channel transmissions adaptation	LG Electronics Inc.	discussion	Rel-19

Proposal 4 (modified): Additional RO configuration includes PRACH configuration index, msg1-FrequencyStart, msg1-FDM, number of SSB per RO, and CB-PreamblesPerSSB are considered in the configuration.

[Apple]: In our contribution, we just copy what RAN1 agreed so we can directly agree with table.

Agree that the following information is included in additional RO configuration.

| Parameter name | Description | Value range |
| --- | --- | --- |
| addl-RACH-Config-Adaptation | Configuration of additional RACH resources for DCI based adaptation of RACH in time domain. | within [the same] RACH-ConfigCommon in SIB1 |
| PRACH-Config Index | RACH configuration index for additional RACH resources for DCI based adaptation of RACH in time domain. | Per addl-RACH-Config-Adaptation |
| msg1-FrequencyStart | Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0, at least for 4-step RACH. | Per addl-RACH-Config-Adaptation |
| prach-SubsetMask-Index-Adaptation | Single PRACH mask to identify the subset of the additional PRACH resources applicable at least for adaptation for DCI 1_0 with P-RNTI. | FFS |
| msg1-FDM | The number of PRACH transmission occasions FDMed in one time instance, at least for 4-step RACH. | Per addl-RACH-Config-Adaptation |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | Configuration of number of SSB per RO and CB preambles per SSB. | Per addl-RACH-Config-Adaptation |

3. RACH: Handling of RRC connected UE

R2-2503393	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1 (modified): if adaptation for additional PRACH resources is supported in active UL BWP, network can configure that paging search space in active DL BWP if network want to activate additional RO to the corresponding UEs.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

[CATT, Apple, ZTE, Vivo, Lenovo, LG, Qualcomm]: Support option 1. [Nokia, Xiaomi]: Support option2 assuming there is indication “activation/deactivation” in SIB also. [Huawei, Ericsson]: Don’t prefer option2, but we do not have to agree with option1 because if the UE is not configured with paging search space, that UE just doesn’t enjoy additional RO. [Ericsson]: Understand for option2, it is UE dedicated RRC message so it doesn’t have to be coupled with whether it is included in SIB1 or not. [IDC]: We are ok with some modified option 1, e.g. network can configure that paging search space in active DL BWP. Also for option2, we don’t prefer at least MAC or group common DCI. [ZTE]: Ok with the modified option1.

If adaptation for additional PRACH resources is supported in active UL BWP, network can configure that paging search space in active DL BWP if network want to activate additional RO to the corresponding UEs.

R2-2504427	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5 (modified): RRC connected UE monitors the DCI-based adaptation indication at least once every once every SI modification period.

[Apple]: Can we capture that it is legacy UE behaviour and no additional UE behaviour is needed? [Nokia]: Do not understand why additional RO needs to be associated with a public warning capable UE. [Samsung]: With the proposal, it is not clear whether short message will follow ETWS short message or SI change notification short message.

RRC connected UE monitors the DCI-based adaptation indication at least once every once every SI modification period.

4. RACH: Whether/how to handle UEs missed the short message?

R2-2503454	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 12: R2 discusses the options to handle the UE who camps on the cell after the first modification period where the additional RACH resources are being activated (i.e., camp at MP#1 or MP#2 in the figure):

Option-1: Not handle this issue;

Option-2: Handle this issue, e.g., reuse the new validity duration parameter to indicate the remaining validity duration of the additional RACH resources.

[Vivo, LG, CATT, Lenovo, Fujitsu, Qualcomm, IDC]: Support option1. Nothing is broken, just that UE doesn’t enjoy additional RO. [Xiaomi, Nokia]: Support option2. [Qualcomm]: Unless the camped UE immediately try RRC connection, additional RO is not really required.

RAN2 will not do anything to handle the UEs that missed the short message.

5. RACH: RA-RNTI collision issue

R2-2503574	Discussion on PRACH adaptation	Xiaomi	discussion	Netw_Energy_NR_enh-Core

Proposal 1: RAN2 agree to introduce a separate offset related to the RACH configuration index when calculating the RA-RNTI.

R2-2503895	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 11 (modified): No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

R2-2504055	RA-RNTI issue and RACH adaptations	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: To avoid RNTI collision/confusion, simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of additional PRACH resources for RACH adaptation for NES -capable UEs.

[ZTE, CATT, Samsung, LG, Apple, Lenovo]: Prefer Nokia proposal. [Sony]: Assume there is MAC impact with Nokia proposal. [Apple]: Do not agree with Sony. [Lenovo]: Agree with Apple. [Ericsson]: Do we need to send a LS to RAN1?

No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

RAN2 understand there is no MAC impact.

Send an LS to RAN1 to inform RAN2 agreement.

[AT130][114][NES] (Nokia)

Scope: Prepare the LS to RAN1 to inform RAN2 agreement on RA-RNTI.

Intended outcome: LS in R2-2504724.

Deadline: Wednesday 18:00 -> Email approval -> Completed.

R2-2504724	LS on RA-RNTI for PRACH adaptation	LS out	Rel-19	Netw_Energy_NR_enh-Core	To:RAN1

Approved.

6. RACH: Co-existence with SBFD?

R2-2503454	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8 (modified): R2 de-prioritizes the discussion of NES (including all NES scopes) and SBFD co-existence.

[Apple, LG]: Support the proposal and think the 2nd sentence is not needed. [LG]: Assume there is no issue for NES and SBFD co-existence. [LG]: Think same conclusion is applied to co-existence between OD-SIB1 and SBFD.

RAN2 de-prioritizes the discussion of NES (including all NES scopes) and SBFD co-existence.

7. RACH: Differentiation of BI field?

R2-2504494	Discussion on PRACH adaptation	ASUSTeK	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: BI field corresponding to the additional RO should have a different value compared with BI field corresponding to the legacy RO.

[IDC, Apple]: Don’t think this enhancement is needed.

BI is commonly applied to both legacy RO and additional RO.

8. SSB: L3 measurement configuration framework

R2-2503454	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 15: For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO.

For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO. No RAN2 spec impact is foreseen.

R2-2503945	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 10 (modified): The additional SMTC configured on SSB for SSB adaptation is used for serving cell measurements upon receiving DCI 2_9.

[Huawei]: In legacy, same SMTC configuration is applied to both serving cell and intra-F neighbouring cells measurements. [Nokia]: Prefer removing neighbouring cells measurement perspective. [Xiaomi, CATT]: If we remove intra-F neighbouring cells, nothing is new since we already agreed with the serving cell aspect.

Noted.

9. RACH: Email discussion report on CFRA applicability

R2-2503374	Summary of [POST129bis][122][NES] CFRA Applicability (OPPO)	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: R2 not pursue spec impact specifically for the usage of R19 additional RACH resources for CFRA initiated by

1)	PDCCH order for additional RACH;

2)	L3 HO command.

3)	LTM CSC MAC-CE;

4)	PDCCH order for LTM early UL sync;

RACH adaptation is not applied to the following cases (for non-serving cell case):

- 1)	PDCCH order for additional RACH;

- 2)	L3 HO command.

- 3)	LTM CSC MAC-CE;

- 4)	PDCCH order for LTM early UL sync;

Proposal 2: R2 not pursue spec impact specifically for the usage for R19 additional RACH resources for CFRA initiated by BFR. And taking that as a premise, R2 discuss whether UE does not base on DCI 1_0 of P-RNTI to know the availability of RACH resource indicated in rach-configGeneric for BFR.

[Nokia]: Do not see any blocking issue to support BFR. [Nokia]: Understand if BFR doesn’t include dedicated rach-configGeneric, it will use rach-configGeneric in rach-configCommon. Then, all required procedure would be same as CBRA, i.e. based on DCI of P-RNTI. [Samsung]: Think one spec impact is when dedicated CFRA preamble/SSB is configured, it’s not clear whether it is applied to legacy RO or additional RO. Understand it is applied to legacy RO currently. [Nokia]: Think we can add additional CFRA preamble/SSB that is applied to the additional RO. [OPPO]: We are not convinced for this use case since network cannot know when beam failure happens. [Qualcomm]: During offline discussion, as compromise, we were ok with supporting BFR case only if there is no spec impact. Now it seems there is actually spec impact. We don’t like to support this use case. [Huawei, LG]: Agree with OPPO and Qualcomm. Note network can reconfigure it anytime if network wants. [Ericsson]: Think spec impact is very marginal.

- Companies supporting RACH adaptation to BFR: Nokia, Ericsson

- Companies not supporting RACH adaptation to BFR: Apple, LG, Xiaomi, Huawei, Qualcomm, Samsung, CATT, Vivo

RACH adaptation is not applied to BFR.

Proposal 3: R2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

RAN2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

R2-2503712	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 11: To ensure efficient discussion, proponents of CFRA adaptation is suggested to clarify their understanding on below aspects:

•	Which additional RACH resource is applied in CFRA adaptation (e.g. whether RACH resource configured via RACH-ConfigCommon in SIB1 or dedicated CFRA resource).

•	Whether need to introduce dedicated SSB-to-RO mapping config for additional CFRA resource.

•	Which Mask is applied in CFRA adaptation (e.g. whether new Mask configured in SIB1 or legacy Mask configured in RRC).

[AT130][112][NES] (OPPO)

Scope: Discuss whether there is further specification impact to support feature and feature combination, and how to solve the issue if any.

Intended outcome: Discussion summary in R2-2504718

Deadline: Comeback in CB session. 30min f2f offline discussion -> Completed.

R2-2504718	[AT130][112][NES] (OPPO)	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 (modified): A same featureCombinationPreamblesList-r17 in RACH-ConfigCommon applies to both legacy and additional RACH, and allow mixing of MSG1 repetition feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if RACH-ConfigCommon associated with the selected random access resource set includes additional RO(s) and the selected random resource set is associated with MSG1 repetition, UE does not use the additional RO(s) configured in this RACH-ConfigCommon.

[Huawei]: Still prefer option-1c. [OPPO]: Understand even with option-1c, we may still need spec impact to clarify network restriction. [Xiaomi, Vivo]: Agree with Huawei. Would like to have more time between option-1b and option-1c. [Samsung]: Note option-1b will provide network restriction, i.e. separate rach-ConfigCommon should be configured for MSG 1 repetition and non-cases. [Nokia]: Want to clarify if this issue is also applied to MSG3 repetition? [Samsung]: Confirm no.

RAN2 will select one of the following options. (CE here means MSG1 repetition).

- Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

- Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17)

Agreements on common channel adaptation

- Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19 unless RAN1 decides supporting 2-step RACH this meeting.

- addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

- Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1. addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

- Agree that the following information is included in additional RO configuration (see the agreed table in the discussion of R2-2504579)

- If adaptation for additional PRACH resources is supported in active UL BWP, network can configure that paging search space in active DL BWP if network want to activate additional RO to the corresponding UEs.

- RRC connected UE monitors the DCI-based adaptation indication at least once every once every SI modification period.

- RAN2 will not do anything to handle the UEs that missed the short message.

- No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. RAN2 understand there is no MAC impact.

- RAN2 de-prioritizes the discussion of NES (including all NES scopes) and SBFD co-existence.

- BI is commonly applied to both legacy RO and additional RO.

- For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO. No RAN2 spec impact is foreseen.

- RACH adaptation is not applied to the following cases (for non-serving cell case): 1) PDCCH order for additional RACH, 2) L3 HO command, 3) LTM CSC MAC-CE, 4) PDCCH order for LTM early UL sync.

- RACH adaptation is not applied to BFR.

- RAN2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

- RAN2 will select one of the following options. (CE here means MSG1 repetition).

Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17).

R2-2503374	Summary of [POST129bis][122][NES] CFRA Applicability (OPPO)	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503393	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503416	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503454	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503468	Discussion on Adaptation of common signal channel transmissions	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503553	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503574	Discussion on PRACH adaptation	Xiaomi	discussion	Netw_Energy_NR_enh-Core

R2-2503712	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503757	Remaining issues of common signal/channel adaption	ZTE Corporation, Sanechips	discussion	Netw_Energy_NR_enh-Core

R2-2503828	Discussion on PRACH adaptation	Sharp	discussion

R2-2503895	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503945	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2503968	Issue for additional PRACH resources with multiple initial BWPs	Spreadtrum, UNISOC	discussion	Rel-19

R2-2504038	Discussion on adaptation of common signal transmissions	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504055	RA-RNTI issue and RACH adaptations	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504244	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated	discussion

R2-2504279	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504388	Discussion on Common signalling adaptation	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504420	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.	discussion	Rel-19

R2-2504427	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504429	[DRAFT] Reply LS on DCI-based PRACH adaptation	Ericsson	LS out	Rel-19	Netw_Energy_NR_enh-Core	To:RAN1

R2-2504471	Discussion on adaptation of common signal/channel transmissions	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504494	Discussion on PRACH adaptation	ASUSTeK	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2504579	Remaining issues on common signal and channel transmissions adaptation	LG Electronics Inc.	discussion	Rel-19

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-250339)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST129b][115], [116], [117], [118], [119], [120], summary of identified stage-3 open issues that need online discussion and rapporteur’s suggestions if needed).

R2-2503310	Reply LS on number of beam measurements in the measurement report MAC CE (R1-2503077; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2

MAC CR rapporteur will take it into account.

Noted.

[Lenovo]: Wonder if the current beam is included, how many beams in total? Is it 16 or 17? [Fujitsu]: Understand 16. [Lenovo]: Understand 17.

16 beams in total.

R2-2503311	LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells (R1-2503079; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2, RAN3

MAC CR rapporteur will take it into account.

Noted.

[CB session]:

[Vivo]: We haven’t decided whether a same MAC CE is used for both RRM measurement and early CSI acquisition purpose. Would like RAN2 to make decision to prepare MAC running CR. [Ericsson]: Before RAN2 makes decision, we would like to see more RAN1 inputs (e.g. related RRC configuration). MAC CR rapporteur can take RAN1 inputs into account in the preparation of MAC running CR (directly including discussion on this issue).

MAC CR rapporteur can discuss this issue (whether a same MAC CE is used for both RRM measurement and early CSI acquisition purpose) with the consideration of further RAN1 inputs, in running CR preparation.

R2-2503320	Reply LS for Reply LS on security handling for inter-CU LTM in non-DC cases (R3-252449; contact: CATT)	RAN3	LS in	Rel-19	NR_Mob_Ph4-Core	To:SA3	Cc:RAN2

Noted.

R2-2503412	Draft 306 running CR for UE capability for Mob Ph4	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Will be baseline for further discussion.

R2-2503413	Draft 331 running CR for UE capability for Mob Ph4	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Will be baseline for further discussion.

R2-2503616	Running MAC CR for Mob Ph4	vivo (Rapporteur)	draftCR	Rel-19	38.321	18.5.0	B	NR_Mob_Ph4-Core

Endorsed.

R2-2503766	Discussion summary and list of MAC open issue for Mob Ph4	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Open issue MAC-1 (essential): Whether the beam specific offset for serving cell Obs is needed for LTM event evaluation

Proposal 1: (4 vs. 4) RAN2 to discuss whether the beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is needed for the LTM event evaluation.

The beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is needed for the LTM event evaluation.

Proposal 2: (4/6) RAN2 to discuss whether the Obs offset is configured per-cell, while applied at the level of indidual beam, if it is agreed to be introduced.

Obs offset is configured per-cell, while applied at the level of indidual beam.

Proposal 3: RAN2 to discuss the Obs offset is applied to which LTM event(s), e.g. LTM2, 3, 5, if it is agreed to be introduced.

Obs offset is applied to LTM3.

[Vivo]: In L3 legacy MR, it is only applied to A3.

Open issue MAC-3 (essential): The initial status for Semi-persistent CSI-RS resource set for candidate cell

Proposal 4: (9/9) The Semi-persistent CSI-RS resource set for candidate cell MAC CE is initially deactiated upon (re-)configuration by upper layer.

The Semi-persistent CSI-RS resource set for candidate cell MAC CE is initially deactiated upon (re-)configuration by upper layer.

Open issue MAC-4 (essential): Whether keep L1-SINR quantity for L1 event triggered MR as FFS.

Proposal 5: (9/9) RAN2 assumes there is no L1-SINR quantity for L1 event triggered MR.

RAN2 assumes there is no L1-SINR quantity for L1 event triggered MR.

Other open issues:

Proposal 6: RAN2 to consider the above open issues related to MAC for mobility enhancement: MAC-5 to MAC-19.

[AT130][101][MOB] (Vivo)

Scope: Discuss and attempt to make conclusion on MAC-5/13/14/15/16/17.

Intended outcome: Discussion summary in R2-2504701.

Deadline: Wednesday (9:40-10:40, main room) -> Comeback in CB session -> Completed.

R2-2504701	Discussion report on [AT130][101][MOB] MAC open issues (vivo)	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal: RAN2 to agree the below offline agreement:

Offline agreement for Event triggered L1 measurement:

	It is up to the NW configuration whether the UE can include measurements for LTM candidates in the MR MAC CE triggered by LTM2, i.e. based on the current configuration in RRC on the number of beams for LTM2 included in the MAC CE.

	For MR triggered by LTM2, if measurements for LTM candidates is configured to be included in the MR MAC CE, which LTM candidate beam should be included:

- Only RS(s) associated with the report ID for LTM2.

	The triggered MR is not cancelled when transmitting truncated MAC CE. The triggered MR is cancelled if regular MAC CE is reported, as in current MAC running CR.

	When a beam (or multiple beam) meets the entry condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the leaving condition, then, the corresponding MR should be cancelled.

	When a beam (or multiple beam) meets the leaving condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the entry condition, then, the corresponding MR should be cancelled.

	When MR is triggered, if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, UE will trigger RACH procedure, as in legacy.

All proposals above are agreed.

Offline agreement for conditional intra-CU LTM:

	Support the co-existence of CLTM and (e)RedCap.

	Support the co-existence of CLTM and CovEnh.

For MAC-14, Rapporteur suggests not to discuss this issue in this meeting, and leave it open (it will not impact the WI completion). It could be considered based on companies’ contributions in the future meetings.

All proposals above are agreed.

R2-2503785	Introduction of event-triggered meas report for RRC spec	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

Endorsed.

R2-2504125	Running RRC CR for inter-CU LTM and CLTM	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

Endorsed.

R2-2503786	Summary of [POST128][116][MOB] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504126	RRC open issues list for mobility	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

[AT130][102][MOB] (Ericsson)

Scope: Discuss and attempt to make conclusion on RRC-2/3/6/7/9/10.

Intended outcome: Discussion summary in R2-2504702.

Deadline: Tuesday (17:00-18:00, main room) -> Comeback in CB session -> Completed.

R2-2504702	Summary of [AT130][102][MOB]	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: Resolve RRC-1 during the RRC email discussion.

Proposal 2: (RRC-2) The appliedLTM-CandidateId is included in both MN and SN RRCReconfigurationComplete message (Rel18 specification should be okay).

Proposal 3: For RRC-3, keep current text in the running RRC CR.

Proposal 4: Resolve RRC-4 during the RRC email discussion.

Proposal 5: For RRC-5, wait for RAN1.

Proposal 6: (RRC-6) In inter-MN LTM failure (LTM cell switch failure) the UE fall back to the previous source PCell configuration before selecting a cell (same as legacy procedure).

Proposal 7: (RRC-7) The legacy sk-counter is used for inter-MN LTM (this means sk-counter value can be re-used).

Proposal 8: (RRC-9) Use the term “LTM” for parts which are common to LTM and CLTM. Use the term “CLTM” only for parts which apply only to CLTM (but not LTM). This will be used in all RAN2 specifications.

Proposal 9: (RRC-10) Follow RAN1 agreement which indicate that at least one candidate RS (within the LTM CSI resource configuration) shall be configured for event LTM2.

All proposals above are agreed.

R2-2503732	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.5.0	B	NR_Mob_Ph4-Core

Endorsed.

R2-2503733	Report of [POST129b][119][Mob] Stage-2 CR 38.300 (Apple)	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to discuss and confirm if it is valid to have the UE configured with DC, but the CLTM configuration does not have any CLTM candidate cells with DC configured (i.e., explicit configuration for each of the CLTM candidates to inform the UE to release DC).

[Apple]: Will have unofficial offline with interested companies. [Ericsson]: Support the proposal and understand there is no additional stage-3 spec impact to allow it.

Comeback in CB session.

Postpone it to the next meeting.

[CB session]:

[Apple]: We can postpone it to the next meeting. Apple plan to prepare some TP.

Proposal 2: Reuse the current measurement related description for LTM for TS38.300 and use TS38.331 for detailed specification of the LTM measurement additions.

Reuse the current measurement related description for LTM for TS38.300 and use TS38.331 for detailed specification of the LTM measurement additions.

Can consider adding some minimum clarification to address the concern.

[ZTE]: Think the current stage 2 running CR is not correct. To avoid re-discussion from scratch, alternatively, we can add some minimum clarification on top of the current version.

R2-2503679	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	draftCR	Rel-19	37.340	18.5.0	B	NR_Mob_Ph4-Core

Endorsed.

R2-2503680	List of 37.340 open issues for R19 mobility	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: Regarding MCG LTM with SN addition, RAN2 to discuss and confirm whether there is no restriction on only supporting MCG LTM with SN addition within the same SN.

[Ericsson]: SN addition is SN addition and we don’t have different procedure to the same and different SN. [Apple]: The original intention was to allow sk-counter update due to MN change. However, if we allow SN change simultaneously, it may bring additional issue. [Vivo]: No need of restriction in the stage 2. [Xiaomi]: At least, we should clarify that inter-MN change with SN change shouldn’t be supported. [China Telecom]: This restriction (i.e. inter-MN change with SN change shouldn’t be supported) was already captured in the running CR.

No restriction on only supporting MCG LTM with SN addition within the same SN.

Proposal 2: Regarding the remaining FFS related to RAN3 in the running CR, RAN2 keeps the FFS for now and waits for RAN3’s progress.

[POST130][110][MOB] (Ericsson)

Scope: Update 38.331 running CR (also including this meeting’s agreements and latest other WGs’ inputs) and remaining essential RRC open issues. Note RRC parts for L1 event-triggered MR will be merged into this CR after this meeting.

Intended outcome: 38.331 running CR and remaining essential RRC open issues.

Deadline: Long email discussion.

[POST130][111][MOB] (Vivo)

Scope: Update 38.321 running CR (also including this meeting’s agreements and latest other WGs’ inputs) and remaining essential MAC open issues.

Intended outcome: 38.321 running CR and remaining essential MAC open issues.

Deadline: Long email discussion.

[POST130][120][MOB] (CATT)

Scope: Update UE capability running CRs if needed (also including the latest other WGs’ inputs). And identify and discuss remaining UE capability related open issues.

Intended outcome: 38.306 and 38.331 running CR and discussion summary (if needed).

Deadline: Long email discussion.

For stage 2 and other running CRs, running CR updates can be directly provided by CR rapporteur next meeting if needed.

[Huawei]: What about RRC running CR on L1 event-triggered MR? [Apple]: Propose to start merging RRC CRs after this meeting.

Agreements on stage-3 details

- 16 beams in total including the current beam in MR MAC CE.

- The beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is needed for the LTM event evaluation. Obs offset is configured per-cell, while applied at the level of indidual beam. Obs offset is applied to LTM3.

- The Semi-persistent CSI-RS resource set for candidate cell MAC CE is initially deactiated upon (re-)configuration by upper layer.

- RAN2 assumes there is no L1-SINR quantity for L1 event triggered MR.

- It is up to the NW configuration whether the UE can include measurements for LTM candidates in the MR MAC CE triggered by LTM2, i.e. based on the current configuration in RRC on the number of beams for LTM2 included in the MAC CE.

- For MR triggered by LTM2, if measurements for LTM candidates is configured to be included in the MR MAC CE, which LTM candidate beam should be included: Only RS(s) associated with the report ID for LTM2.

- The triggered MR is not cancelled when transmitting truncated MAC CE. The triggered MR is cancelled if regular MAC CE is reported, as in current MAC running CR.

- When a beam (or multiple beam) meets the entry condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the leaving condition, then, the corresponding MR should be cancelled.

- When a beam (or multiple beam) meets the leaving condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the entry condition, then, the corresponding MR should be cancelled.

- When MR is triggered, if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, UE will trigger RACH procedure, as in legacy.

- Support the co-existence of CLTM and (e)RedCap.

- Support the co-existence of CLTM and CovEnh.

- (RRC-2) The appliedLTM-CandidateId is included in both MN and SN RRCReconfigurationComplete message (Rel18 specification should be okay).

- For RRC-3, keep current text in the running RRC CR.

- (RRC-6) In inter-MN LTM failure (LTM cell switch failure) the UE fall back to the previous source PCell configuration before selecting a cell (same as legacy procedure).

- (RRC-7) The legacy sk-counter is used for inter-MN LTM (this means sk-counter value can be re-used).

- (RRC-9) Use the term “LTM” for parts which are common to LTM and CLTM. Use the term “CLTM” only for parts which apply only to CLTM (but not LTM). This will be used in all RAN2 specifications.

- (RRC-10) Follow RAN1 agreement which indicate that at least one candidate RS (within the LTM CSI resource configuration) shall be configured for event LTM2.

### 8.6.2	Inter-CU LTM

Essential remaining open issues, including remaining issues for fast recovery,  stage-3 identified open issues if needed, etc.

1. RRC-8: Fast recovery to the candidate cell with different R19 ID as target/source?

R2-2503532	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1a: (RRC-8) For inter-CU LTM failure (the R19 set ID is different between the target cell and source cell), if the selected candidate cell has the different Rel-19 ID of source, the UE performs fast failure recovery.

R2-2503804	Discussion on inter-CU LTM	Qualcomm Incorporated	discussion

Proposal 1: RAN2 agree to NOT support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

[Qualcomm]: Understand the main difference between two sides is whether common NCC is applied to all candidates even with different Rel-19 ID. [Samsung]: Understand RAN3 status is not clear on that. In addition, whether new key pairs will be forwarded per candidate cell level or per CU level is also not clear. [Nokia, CATT, Huawei, Apple, LG]: Understand same NCC can be applied to other candidates. [Apple]: During CHO recovery discussion, it was clarified the same NCC can be applied. [Apple]: As WI rapporteur, we would like to have clear decision so that anything is not pending dependent on other WG in this late phase. Anyway, LTM recovery is for optimization just for the case LTM fails. [Ericsson]: Agree with Apple. Want to stick to what we agreed and no additional scenario is considered for fast recovery. [Lenovo]: Although we don’t think there is security issue with Xiaomi proposal, would like to be conservative due to lack of time for WI completion. Agree with Apple and Ericsson. [NTTDCM]: Ok with not allowing additional scenarios for fast recovery if we don’t have time. [Apple, Qualcomm, Ericsson, Lenovo, NTTDCM, ZTE, MediaTek]: Ok with not allowing additional scenarios. [Huawei]: We may consider no fast recovery at all.

RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

2. RRC-8: Fast recovery to the candidate cell with same R19 ID as source?

R2-2503617	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: (RRC-8) After inter-CU LTM failure, if the selected candidate cell has the same Rel-19 ID as the source cell, the UE cannot perform LTM fast recovery.

R2-2504145	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1b: In inter-CU LTM cell switch failure, if the selected cell for recovery has the same Rel-19 ID as the source cell, the UE performs key update and PDCP re-establishment.

Covered by the discussion above.

3. Fast recovery to the candidate cell with same R19 ID as target

R2-2504542	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 is kindly asked to agree to support the fast recovery without security key change.

R2-2503409	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3b: If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

[Apple, Nokia, CATT]: Target key should be regenerated by using the selected cell’s frequency and PCI. [Ericsson]: We can wait for further RAN3 progress until CB session, then we can see it. [Session chair]: It seems that companies’ common understanding is that if source gNB provides new key by using NCC to the target gNB per candidate cell level, then the UE needs to regenerate the key using frequency and PCI of the selected cell. Most of companies confirm that. [Ericsson, MediaTek]: Understand in order for RRC reestablishment to work, the UE needs to revert back to the source configuration upon T304 expiry. Otherwise if the UE maintains the target configuration until fast recovery fails, the UE needs to revert back to old of old configurations. We understand CATT proposal makes sense.

Working assumption: If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

Will wait further RAN3 progress until CB session. If no RAN3 decision that is not aligned with the working assumption is made, we’ll confirm the working assumption.

Confirm working assumption as agreement.

[LG]: Wonder if inter-CU LTM fails and the UE selects the failed target cell again, what will happen? If the UE reverts back the source at T304 expiry, the key stream-reuse can happen. [Apple]: We may need further discussion on that.

R2-2504168	Remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: In Rel-19 inter-CU LTM, keep the principle that LTM recovery is only attempted once (same as CHO and Rel-18 LTM).

4. MAC-18: CSI-RS based CFRA resource?

R2-2504027	Discussion on inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 10: (MAC-18) RAN2 assumes to support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE, which is supported for both intra-CU and inter-CU LTM. Further check with RAN1 and RAN3.

[Apple]: Think if we allow that, it will impact not only RAN3 but also RAN1/4. Prefer not allowing it. [Huawei, CATT]: Do not see real useful scenario for this proposal. [Vivo]: Understand RAN1/2 impact may be acceptable, but we may have a little bigger RAN3/4 impact. [MediaTek]: According to WID, think we don’t need to support this proposal.

RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE.

5. MAC-19: MAC-19 NCC optional or mandatory?

R2-2504423	Remaining open issues on inter-CU LTM	ETRI	discussion	Rel-19

Proposal 1: The Enhanced LTM CSC MAC CE always contains an NCC value field

Proposal 2: The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

The Enhanced LTM CSC MAC CE always contains an NCC value field.

The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

R2-2504027	Discussion on inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 11a	(MAC-19) If CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE is supported, a new field is introduced in Rel-19 Enhanced LTM Cell Switch Command MAC CE to indicate the CSI-RS index for CFRA.

Proposal 11b	(MAC-19) If proposal 11a is agreeable, the Rel-19 Enhanced LTM Cell Switch Command MAC CE can be used for both intra-CU and inter-CU LTM. NCC is optional present in the Rel-19 MAC CE.

Agreements on inter-CU LTM

- RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

- If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

- RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE.

- The Enhanced LTM CSC MAC CE always contains an NCC value field. The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

R2-2503381	Some issues for Inter-CU LTM	Lenovo Information Technology	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503409	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503479	Fast recovery for inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503486	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503532	Remaining issues of inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503617	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503681	Discussion on remaining issues of inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503744	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503747	Discussion on inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503804	Discussion on inter-CU LTM	Qualcomm Incorporated	discussion

R2-2503822	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503858	Inter-CU LTM and Fast Recovery	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2503898	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504027	Discussion on inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504043	Discussion LTM fast recover of inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2504048	Remaining issues on inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504137	Further remaining issues for Inter-CU LTM	Nokia	discussion

R2-2504145	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504168	Remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504194	Discussion on Inter-CU LTM fast recovery	InterDigital Washington DC	discussion	Rel-19

R2-2504262	Discussion on open issues of inter-CU LTM	Ofinno	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504330	Remaining issues of Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2504389	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504423	Remaining open issues on inter-CU LTM	ETRI	discussion	Rel-19

R2-2504516	Discussion on inter-CU LTM	NTT DOCOMO, INC.	discussion	Rel-19

R2-2504542	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

### 8.6.3	L1 event triggered measurement reporting

Essential remaining open issues, including further details of MR MAC CE, truncated MR MAC CE, SP CSI-RS activation/deactivation MAC CE, stage-3 identified open issues if needed, etc.

1. SP CSI-RS: Handling of the activated SP CSI-RS after LTM cell switch

R2-2504445	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: UE continues to measure the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM).

Proposal 3: RAN2 send LS to RAN3 for the issue of handling the activated SP CSI-RS resources after LTM cell switch.

R2-2504115	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: UE should continue to measure SP CSI-RS resources of target cell after the cell switch and stop to measure the other candidate cells if the target cell is configured with CSI acquisition (Option 3).

R2-2503812	Remaining issues of LTM measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 10: The SP CSI-RS resources is set to deactivated state after LTM cell switch.

[Qualcomm, LG, OPPO, Xiaomi, ZTE]: Support Apple proposal for simplicity. [CATT, MediaTek]: Support Samsung. Understand there is no real difference in the complication for all options. [MediaTek]: Should continue measurement to support subsequent LTM. [Huawei]: At least for the target cell, it should be continued for CSI acquisition purpose. [Session chair]: Ask if CSI report is done as part of LTM cell switch procedure? [Huawei]: Understand it is continued even after LTM switch completion. [Nokia]: Agree with Huawei. [ZTE]: Not sure if SP-CSI resource is still used for CSI report even after LTM cell switch completion.

UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch.

Comeback on target cell case in CB session.

[CB session]:

[Apple]: RAN1 has not made any decision regarding whether early CSI acquisition using SP-CSI resource is one-shot or is continued after LTM cell switch completion. We can postpone it to the next meeting. [Ericsson, ZTE]: Shouldn’t we distinguish two separate SP-CSI resources for RRM measurement and early CSI acquisition purposes? [Apple]: It is not decided in RAN1. Most likely we can use same SP-CSI resources for both purposes.

Postpone target cell case to the next meeting.

2. SP CSI-RS: Granularity for the SP CSI-RS activation/deactivation

R2-2503410	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6: Send LS to RAN1 to clarify the granularity of the SP CSI-RS activation/deactivation, with the following options:

-	Option 1: per LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. all RSs (from same candidate cell or different candidate cells) in one LTM-NZP-CSI-RS-ResourceSet-r19 are activated/deactivated by the new MAC CE).

-	Option 2: per candidate cell in one LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. all RSs of certain candiate cell in one LTM-NZP-CSI-RS-ResourceSet-r19 are activated/deactivated by the new MAC CE).

-	Option 3: per RS in one LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. a certain RS in one LTM-NZP-CSI-RS-ResourceSet-r19 is activated/deactivated by the new MAC CE).

R2-2504135	Final Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 14: Use ltm-NZP-CSI-RS-ResourceSet instead of NZP-CSI-RS-ResourceSet in the MAC CE to indicate the set of SP CSI-RSs to be activated for a candidate cell.

[Vivo]: Understand the current running MAC CE implemented as option 2. Understand Nokia’s proposal can achieve option1 that is aligned with RAN1.

Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

[POST130][116][MOB] (CATT)

Scope: Prepare LS to RAN3 to inform SP CSI-RS agreement.

Intended outcome: LS in R2-2504727.

Deadline: Short email discussion.

3. Working assumption: Four codepoints to represent one of {type1, type2, type3, type4}

R2-2503859	Discussion on L1 event-based measurements	Ericsson	discussion	NR_Mob_Ph4-Core

Proposal 1: The following beams can be included within a event-triggered L1 measurement report:

a.	Type1 beam: a beam that has satisfied the entry condition of the event for the entire TTT, triggers the MR MAC CE, and enters the beamTriggeredList for the first time.

b.	Type2 beam: a beam that has satisfied the leaving condition of the event for the entire TTT, triggers the MR MAC CE, and exit the beamTriggeredList.

c.	Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

d.	Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

Proposal 2: RAN2 to confirm the working assumption to introduce four codepoints to represent one of {type1, type2, type3, type4} beams.

[LG]: If we follow the modified type 1 beam definition, when type 1 was triggered but not reported yet, it is already included into beamTriggeredList, then when the UE gets a grant to report, it may not be included as the first priority. [Nokia]: Simple way is not to distinguish between type 1 and type 3. [Vivo]: Once we confirm the WA, the details that LG raised can be further discussed in MAC offline.

R2-2504115	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: There is no need to distinguish between Type 1 and Type 3 beams. Three codepoints should be introduced to represent the following: {Type 3, Type 2, Type 4}.

[ZTE]: In L3 measurement reporting, we already introduced the different bit for type 1 and type 3. Some NW implementation does not want to store all the received MR history. [CATT]: Agree with Huawei proposal. Before TEI18, there was no distinction between type 1 and type 3 and it worked well. [Ofinno]: We don’t see real difference between four codepoints and three codepoints. We can confirm working assumption to enable different NW implementation.

Confirm working assumption, i.e. four codepoints to represent one of {type1, type2, type3, type4}, as an agreement.

For detailed description of type 1, it can be handled as part of running CR preparation.

4. MAC-6: Truncated L1 MR MAC CE: Beams to be included

R2-2504028	Discussion on L1 event triggered measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7a: (MAC-6) For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam.

[LG]: If type 1 and type 2 meet the condition at the same time, why type 1 should be prioritized? [ZTE]: If grant is not enough to report both type 1 and type 2, consider type 1 is more important.

(MAC-6) For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam.

Proposal 7b (modified): (MAC-6) If the UE can not include all of the beams with the same priority, it’s up to the UE implementation which beams to be included in the truncated MR MAC CE.

[Nokia]: Why not making a rule based on the strongest beam? [Lenovo, ASUSTeK, Samsung]: Agree with Nokia.

If the UE cannot include all of the beams with the same priority, the beams are selected in the (truncated) MR MAC CE based on measured RS quality.

Proposal 7c (modified): (MAC-6) Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE.

(MAC-6) Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE.

5. MAC-6: Truncated L1 MR MAC CE: Handling of the current beam

R2-2503618	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: RAN2 to agree the priority to include beams in L1 MR truncated MAC CE is as follows (highest priority listed first, FFS whether to capture it in spec):

1)	Type 1 or Type 2 beam;

2)	Type 3 beam;

3)	Current serving beam (if configured to be included);

4)	Type 4 beam (if configured to be included).

R2-2503812	Remaining issues of LTM measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: Consider the above procedure to generate the truncated/regular MR MAC CE:

-	Step 1: Based on the available UL resource, select the beams to be reported according to the following orders:

o	1) Beams that met the conditions;

o	2) Beams that not met the conditions (if configured);

o	3) Current beam (if configured).

[Apple, OPPO]: We already made agreement the current beam’s measured result will be located as the last position. [Xiaomi]: Understand network can know the current beam’s situation by some other means, so it sounds natural to deprioritize it. [LG]: Wonder if the current beam is considered as type 1 or current serving beam in LTM 2 case. [Session chair]: Is the current beam considered as type 1? [Ericsson]: Understand it is still considered as the current serving beam and located in the end of MR, then we can keep the same MR format for LTM2 also. [Nokia]: Think the current beam needs prioritization rather than type 4. [Lenovo]: If the current serving beam is located in the end, how differential MR can be achieved? [Ericsson]: Differential MR is applied except the current serving beam. [Ofinno]: If the remaining grant is only for 1-byte, which beam should be reported? Understand the current beam should be able to be included in the case. [Ericsson]: If the current beam may or may not be included in the truncated MR MAC CE, how the network decodes truncated MR MAC CE correctly? Think we need fixed rule for the current serving beam, e.g. either by always including the current beam in the end or not including it at all. [Apple]: Propose not to include the current beam in the truncated MR MAC CE at all.

How to handle the current serving beam in the truncated MR MAC CE will be comeback in CB session (Ericsson will lead some unofficial offline among the interested companies)

[CB session]:

[Ericsson]: Based on offline, companies share the view that if network configures to include the current serving beam in MR, the UE always include the current serving beam in a truncated MR MAC CE. [Lenovo]: If grant is really minimum, the UE always include the current serving beam instead of triggered beam? [LG, Ericsson]: Understand the minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant cannot meet this minimum size, the UE cannot send even truncated MR MAC CE. [Samsung]: If network configures the current serving beam inclusion and the grant is only for the triggered beam, it is strange the UE cannot send a truncated MR MAC CE. [Qualcomm]: Ok with Ericsson’s suggestion.

Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

6. MAC-7: Whether one UL TB could include more than one truncated MAC CE or only one?

R2-2503435	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: (MAC-7) If more than one triggering events are pending and UL grant is not sufficient for two regular L1 MR MAC CEs, UE transmits as much information as possible in one L1 MR MAC CE for one event, and transmits truncated L1 MR MAC CE for the other event if there is space available in the UL grant.

R2-2503484	Discussion on L1 measurement	Lenovo	discussion	NR_Mob_Ph4-Core

Proposal 9: When UL grant is not sufficient to include all Full MRs (each with many Type3/ 4 beams) and therefore can only include 'm' Full MRs, UE shall rather maximize reporting of triggered beams (Type1/ 2) by including 'n' truncated MRs in the TB (n < m). Exact implementation is left to the UE implementation.

R2-2504639	Resolving MAC open issues for LTM event triggered MR	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

Proposal 9: In case N triggered LTM events are pending, if UL grant is sufficient for including some combinations of L regular LTM MR MAC CEs and M truncated MR MAC CEs with L+M≤N, 0≤L<N,0≤M≤N, but insufficient for including N regular LTM MR MAC CEs, it is up to UE implementation which combination to include.

[IDC]: Think it depends on the prioritization rule among MAC CEs, e.g. whether regular L1 MR MAC CE or L1 truncated MR MAC CE is prioritized. [Samsung]: Understand the regular L1 MR MAC CE should be prioritized than the truncated MR MAC CE. [Qualcomm]: Think it will be simpler to consider same priority between regular MR MAC CE and truncated MR MAC CE, then leave it to UE implementation to select which MR MAC CE to be sent. [Vivo]: Agree with Qualcomm.

Same priority between regular MR MAC CE and truncated MR MAC CE.

It is up to UE implementation how to handle/include if more than one triggering events are pending and UL grant is not sufficient for all regular L1 MR MAC CEs.

7. Prohibit timer for L1 event-triggered MR?

R2-2504263	Discussion on L1 event triggered measurement report	Ofinno	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: Introduce prohibit timer for event triggered L1 MR, with the following behavior:

-	The prohibit timer is started after triggering a new MR for a candidate cell satisfying an entry condition of an LTM event for TTT.

-	While the prohibit timer is running, a new MR is not triggered for the same candidate cell for the same LTM event.

-	While the prohibit timer is running, a new MR is triggered for the same candidate cell satisfying an entry condition of a different LTM event.

[Ericsson]: Mobility support is important, we don’t see a need. [Nokia]: It can be configurable, so if NW doesn’t want, it can be absent. [Apple, ZTE, CATT, Xiaomi, OPPO, Huawei, Samsung]: It will make MAC complicated. Beams can be changed fast. NW has already some tools to handle it. Do not see a need. [MediaTek]: Prohibit timer should not be applicable to the different beam(s) even though it is applied to the same event. [Nokia]: Understand w/o L2 filtering, prohibit timer will be helpful to avoid the frequent MRs.

No prohibit timer is applied to L1 event-triggered MR.

8. MAC-8: FFS the coexistence between event triggered report and mTRP (source cell)

[Nokia]: Want to suggest to have post email discussion until next meeting. [Ericsson]: Prefer not to have post email discussion considering we already have many for the running CRs. [Vivo, Huawei, Lenovo, Rakuten]: Support to have post email discussion.

[POST130][117][MOB] (Nokia)

Scope: Discuss issues of coexistence between L1 event triggered MR and mTRP (source cell).

- Whether to allow coexistence between event triggered MR and mTRP (source cell)?

- If companies agree to allow it, how to define the current beam?

Intended outcome: Summary discussion.

Deadline: Long email discussion.

[Apple]: We need to keep the current MR MAC CE format. Should not introduce a different MR MAC CE format just to support mTRP. [Ericsson, Samsung]: Understand the only issue is how to define the current beam. [Nokia]: Would like to include the discussion on the possible change of MR MAC CE format. [Ericsson, Apple]: Next meeting is the last meeting for WI completion, if we really want to introduce it, it should be simple.

Simple solution should be preferable and considering next meeting is the last meeting for WI completion, everything (including companies’ consensus, one solution, and the corresponding TP) should be ready for next meeting.

Agreements on L1 measurement triggered MR

- UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell.

- Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

- Confirm working assumption, i.e. four codepoints to represent one of {type1, type2, type3, type4}, as an agreement. For detailed description of type 1, it can be handled as part of running CR preparation.

- (MAC-6) For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam. If the UE cannot include all of the beams with the same priority, the beams are selected in the (truncated) MR MAC CE based on measured RS quality. Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE.

- Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

- Same priority between regular MR MAC CE and truncated MR MAC CE. It is up to UE implementation how to handle/include if more than one triggering events are pending and UL grant is not sufficient for all regular L1 MR MAC CEs.

- No prohibit timer is applied to L1 event-triggered MR.

R2-2503410	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503435	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503469	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503484	Discussion on L1 measurement	Lenovo	discussion	NR_Mob_Ph4-Core

R2-2503487	Open issues for L1 event triggered measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503554	Discussions on L1 event triggered measurement reporting	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503618	Discussion on LTM measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503682	Discussion on remaining issues of L1 event triggered measurement reporting	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503745	Discussion on issues for supporting L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503772	Discussion on L1 event triggered measurement reporting	Baicells Technologies Co. Ltd	discussion

R2-2503802	Discussion on event triggered L1 measurement reporting for LTM 	Qualcomm Incorporated	discussion

R2-2503812	Remaining issues of LTM measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503859	Discussion on L1 event-based measurements	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2503861	Discussion on truncated L1 measurement reporting	Panasonic	discussion

R2-2503989	Remaining issues on MR MAC CE	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504028	Discussion on L1 event triggered measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504044	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2504115	Discussion on L1 event-triggered measurement reporting	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504120	Discussion on the support of mTRP in event-triggered LTM	Huawei, HiSilicon, Nokia, NTT Docomo, OPPO, ZTE	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504135	Final Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504181	LCP for LTM measurement report MAC CE	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504263	Discussion on L1 event triggered measurement report	Ofinno	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504331	Remaining issues of L1 event triggered measurement reporting	Rakuten Mobile, Inc	discussion	Rel-19

R2-2504402	Discussion on L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504408	Remaining issues of event-triggered L1 measurement reporting for LTM 	Kyocera 	discussion

R2-2504422	Discussion on L1 event triggered measurement reporting for LTM	KDDI Corporation	discussion	Rel-19

R2-2504445	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504472	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504495	Discussion on event-triggered L1 MR MAC CE	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504610	Discussion on LTM event-triggered measurement reporting 	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2504639	Resolving MAC open issues for LTM event triggered MR	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

### 8.6.4	Conditional intra-CU LTM

Essential remaining open issues, including further details of L3 measurement based C-LTM (e.g. beam selection, cell selection), whether to support fallback from RACH-less CLTM CS to RACH-based CLTM CS, stage-3 identified open issues if needed, etc.

1a. MAC-9: How UE selects the valid CG resource for L3 based CLTM?

1b. MAC-10: How UE selects the valid CG resource for L3 based CLTM?

R2-2503619	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2 (modified): (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one.

[Ericsson]: Is the threshold new one or legacy one? [Vivo, Apple]: Reuse the existing threshold. [Samsung]: For CG resource, we don’t have any threshold that is associated with CG resource. In legacy, there is CG resource associated threshold in terms of CG-SDT. [Qualcomm]: Prefer leaving it to UE implementation. [Samsung]: On CG resource selection, we have clearly specified UE behaviour in the past. [Qualcomm]: If the concerned beam is more than one, how to handle it? [Vivo]: In the case, it is up to UE implementation. [ASUSTeK]: How to handle 2 TA case in association with this proposal? [Vivo]: It will be discussed as part of running CR preparation.

(MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one.

Proposal 3: (MAC-9) If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

(MAC-9) If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

2. MAC-11: UE behavior if TA is valid but UE couldn’t obtain valid CG resource

R2-2504049	Remaining issues on CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4 (modified). For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant.

[Ericsson]: What “no valid configured uplink grant” means? [Apple]: We can remove “valid”. [Huawei]: What’s difference compared to the previously agreed proposal? [Samsung]: The new one is “if there is no configured uplink grant”. [Vivo]: For L1 based CLTM, it was already agreed.

For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant.

3. How to handle if TAT of the target cell expires during RAHC-less CLTM execution?

R2-2503931	Remaining Open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2 (modified). In case of timeAlignmentTimer associated with PTAG expiry during RACH-less LTM cell switch, discuss the following options:

-	Opt1) the UE terminates the CLTM cell switch procedure

-	Opt2) the UE fallbacks from RACH-less LTM to RACH-based C-LTM

-	Opt3) the UE does nothing (i.e. count on T304).

[LG, CATT, Apple, Lenovo, Huawei]: Understand this issue can also happen in R18 LTM and we count on T304. Support option 3. [Samsung, Qualcomm, Ericsson, MediaTek, Nokia, Rakuten, Ofinno, IDC]: There is difference between R18 LTM and R19 C-LTM. In R18 LTM, TAT timer starts upon the reception of LTM cell switch command while in R19, TAT timer starts much earlier so there would be more chance it expires early. Also note the current running MAC CR is already somewhat different compared to R18 MAC. [Vivo]: To the current running CR, the UE checks based on whether TAT runs when the first CG resource is available, which is different in R18. [Samsung]: Note in R18 discussion, some companies understand it can be controlled by NW, e.g. TAT is set as similar value of T304, however it cannot be applied to CLTM case.

Comeback in CB session (Vivo wants to have more time for offline discussion).

[CB session]:

R2-2504921	Offline summary on the expire of target cell TAT during RAHC-less CLTM	Vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1 (modified): Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

[Huawei]: Seems “the first available CG occasion” definition is not clear. [Vivo]: CG occasion associated with the selected beam was already captured in the running CR. [Lenovo]: How P1 works in the case that the UE has two TAs (one is PDCCH order-based TA and the other one is UE based TA). [Ericsson]: It is not R19 CLTM specific issue. [Vivo]: This proposal 1 is about PDCCH order-based TA. Will update the running CR accordingly.

Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

Proposal 2: RAN2 to discuss the below options:

-	Option 2 (Fallback): During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

-	Option 3 (HO failure): During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, there is no fallback to RACH-based CLTM. The UE waits for T304 expires.

Rapporteur suggests to try option 2 during CB session.

During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

4. RACH-less C-LTM: CG resource selection when CSI-RS is used for measurement?

R2-2503931	Remaining Open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1. In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS.

In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS.

5. RACH-less C-LTM: TCI state

R2-2504146	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 3: For CG-based RACH-less cell switch, if separate TCI mode is used, the UE indicates a suitable DL TCI state in the initial UL transmission (details FFS).

RAN2 will not do anything for the separate TCI mode optimization.

R2-2503411	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 9: Send LS to RAN1 to confirm whether UE needs to determine the target TCI state upon the RACH-less conditional LTM execution and indicate it to lower layer.

Can be treated as part of running CR discussion.

[Ericsson]: For CATT issue, do not think it is an issue because the UE can know which TCI state based on association between SSB and CSI-RS. For Huawei issue, network can send a response based on the received UL transmission, which would be probably more wide-beam. [Huawei]: Wonder how the UE does? Don’t we need to specify UE behaviour for the reception of NW response. [MediaTek]: Understand there would be complication to support separate TCI state. Also think in R18 LTM, details are not crystal clear. Propose not to optimize anything for separate TCI state. [Samsung]: Do not think it is an issue. For C-LTM, the UE will attempt to receive NW response according to the SSB that is associated with the selected CG resource. [LG]: Agree with MediaTek. [Apple]: In addition, think the definition of separate TCI state is more for RAN1.

6. Others

R2-2504283	Discussion on conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

Proposal 1: In FR2, after TCI state activation for a candidate cell, the UE shall stop L1 based condition evaluation for candidate cells without active TCI states.

[MediaTek]: Support the proposal. [Ericsson]: It is clearly specified in RAN4. Do not see a need to specify anything in RAN2 spec.

Comeback in CB session.

Noted.

Agreements on C-LTM

- (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

- For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant.

- Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

- In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS.

- RAN2 will not do anything for the separate TCI mode optimization.

R2-2503382	CLTM Scenarios and remaining points	Lenovo Information Technology	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503411	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503455	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2503470	Further discussion on conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503488	Open issues for conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503533	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503555	Discussion on conditional Intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503619	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503683	Discussion on remaining issues of CLTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503746	Discussion on issues for supporting conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503773	Discussion on Conditional intra-CU LTM remaining issues	Baicells Technologies Co. Ltd	discussion

R2-2503803	Discussion on intra-CU conditional LTM 	Qualcomm Incorporated	discussion

R2-2503813	Remaining issues of conditional LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503860	Open issues for Conditional LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2503931	Remaining Open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2503956	Discussion on conditional LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2504029	Discussion on conditional intra-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504045	Discussion on remaining issues of conditional intra-CU LTM	Transsion Holdings	discussion	Rel-19

R2-2504049	Remaining issues on CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504146	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504182	Conditional LTM.	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504269	Discussion on open issues of CLTM	Ofinno	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504283	Discussion on conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2504329	Remaining issues of Conditional LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2504390	Discussion on conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504412	Remaining issue of Conditional intra-CU LTM 	Kyocera 	discussion	Rel-19

R2-2504424	Remaining open issues on conditional LTM	ETRI	discussion	Rel-19

R2-2504446	Remaining Issues of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504473	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504496	Discussion on TA handling for C-LTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2504531	Discussion on Conditional intra-CU LTM	ITL	discussion	Rel-19