3GPP TSG-RAN WG2 Meeting #130	R2-2504796

# St.Julians, Malta, May 19th – 23th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0040 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on terminology of local ID pair list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes to replace sl-LocalID-PairToAddModList with sl-LocalID-PairList for alignment with RRC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace sl-LocalID-PairToAddModList with sl-LocalID-PairList in the spec. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: L2 U2U SL relay operation Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue, as the changes are to terminology modification. - If the U2U Remote UE is implemented according to the CR and the Relay UE is not, or vice versa, there is no inter-operability issue, as the changes are to terminology modification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec still remains the terminology of sl-LocalID-PairToAddModList which is not aligned with RRC spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3a.3.2, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CR 5375 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

BEGIN OF CHANGES

5.3a.3.2	Egress link determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If there is an entry in sl-LocalID-PairList, in which the sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity match the UE ID fields in the U2U SRAP Data PDU:

-	Determine the egress link on PC5 interface towards the peer U2U remote UE identified by sl-PeerRemoteUE-L2Identity configured for the concerned sl-PeerRemoteUE-LocalIdentity as specified in TS 38.331 [3].

NEXT CHANGE

5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Remote UE, if sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity are both configured, when an SRAP Data PDU with SRAP header that contains UE ID fields which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairList is received or when an SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or any of the 5 bits LSBs of slrb-PC5-ConfigIndex used on the end-to-end PC5 connection indicated by sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Relay UE, when an SRAP Data PDU with SRAP header that contains a UE ID (for DST) field which does not match sl-PeerRemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairList is received, or when an SRAP Data PDU that contains a UE ID (for SRC) field which does not match the concerned sl-RemoteUE-LocalIdentity corresponding to sl-RemoteUE-L2Identity of the ingress link in any one of the entries in sl-LocalID-PairList with matched sl-PeerRemoteUE-LocalIdentity and sl-PeerRemoteUE-L2Identity is received, or when an SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or any of the 5 bits LSBs of slrb-PC5-ConfigIndex used on the end-to-end PC5 connection indicated by sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity is received by U2U Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

When any of the U2N Remote UE, the U2N Relay UE, the U2U Remote UE or the U2U Relay UE receives an SRAP Data PDU with invalid or reserved values, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

END OF CHANGES