3GPP TSG RAN WG2 #130	R2-2504776

St Julian's, Malta, 19th May – 23rd May, 2025

Title:	[DRAFT] LS on CB-msg3-EDT

Response to:	-

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	Huawei [to be RAN2]

To:	RAN1, RAN4

Cc:	-

Contact Person:

Name:	Xubin

E-mail Address:	xubin10@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- 1. Overall Description:

During RAN2#130 meeting, RAN2 discussed CB-msg3-EDT and achieved the following agreements regarding HARQ feedback for MSG4:

- Agreements regarding HARQ feedback for MSG4 (i.e., CB-msg3 response): - The HARQ feedback resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE. RAN2 could revisit this proposal if RAN1 has some concern. - Whether to send the HARQ feedback for CB-Msg4 can be controlled by NW. UE does not send HARQ NACK. - For NB-IoT, the SubCarrierSpacing of the HARQ feedback for CB-Msg4 is same as the CB-Msg3. - Reuse the existing format of HARQ ACK allocation signalling in the DCI. There is 2-bit HARQ ACK resource for eMTC and 4-bit HARQ ACK resource for NB-IoT. Reuse the meaning of DCI field in R1 SPEC. - Introduce a new CB-Msg3 Response (CBR) MAC sub-header in CB-Msg4. It has 1bit E for sub-header/payload indication, 2 bits T for sub-header type, 1bit T2 for HARQ ACK resource present, 1 bit T3 for TAC present, 1 bit T4 for C-RNTI present and 2bit R for reservation

Besides, RAN2 also made the following agreements regarding TAC in MSG4:

- Agreements regarding TAC in MSG4 (i.e., CB-msg3 response): - The TAC is optionally used in the CB-Msg3 response. - RAN2 assumes that NTA=0 for initial CB-msg3 transmission. - RAN2 assumes the length of the TAC field is 6 bits (we can revisit this if there is major R1 impact on TA calculation)

RAN2 would like to check following:

- whether there is any concern from RAN1 on the agreements regarding HARQ feedback for MSG4

- whether there is any concern from RAN1 and RAN4 on the agreements regarding TAC in MSG4

- 2. Actions:

- To RAN1 and RAN4:

ACTION: RAN2 respectfully asks RAN1 and RAN4 to provide feedback if any and take the above information into account in the specification work if necessary.

- 3. Date of Next RAN2 Meetings:

RAN2#131	25th August – 29th August 2025		Bangalore, India

RAN2#131bis	13th October – 17th October 2025		Prague, Czech