3GPP TSG RAN WG2#130	R2-2504965

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG RAN WG1 #121	R1-2504936

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	LS on Msg4 PDSCH repetition

Release:	Rel-19

Work Item:	NR_NTN_Ph3

Agenda Item:	9.11

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Mohamed EL JAAFARI

E-mail Address:	mohamed.el-jaafari@thalesaleniaspace.com

Send any reply LS to:	3GPP Liaisons Coordinator:  mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

As part of the WI on NR NTN enhancement phase 3, RAN1 has agreed to support for Msg4 PDSCH repetition. The following agreement has been made regarding the activation/deactivation of Msg4 PDSCH repetition.

Agreement For the activation/deactivation of Msg4 PDSCH repetition: - Alt 1: UE specific PDSCH with Msg4 repetition activation indicated via DCI Format 1_0: - Signaling uses re-interpretation of 1 MSB in MCS field in DCI. - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH. - Note: RAN1 considers there is no difference between capability and request - FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs. - The aggregation factor is configured in SIB1, with possible value 2 or 4 - When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled.

RAN1 kindly requests RAN2 to consider the above agreement when designing the report included in Msg3 PUSCH.

# 2. Actions:

Action to TSG RAN WG2:

RAN1 respectfully requests RAN2 to take the above into account and provide feedback if needed.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #122	25 Aug – 29 Aug 2025	Bengaluru, IN

TSG-RAN WG1 Meeting #122-bis	 13 Oct – 17 Oct 2025 	Prague, CZ