3GPP TSG-RAN WG2 Meeting #131	R2-2505118

Bengaluru, India, 25-29 August, 2025

Agenda item:	8.7.6

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on XR rate control

Document for:	Discussion and Decision

1. Introduction

RAN2 has agreed to support per QoS flow rate control via MAC CE. The gNB can indicate recommended bitrate to the UE while the UE can indicate desired bitrate to the gNB. Some details remain for further discussion, such as how to indicate the QoS flows in the MAC CE and how to set the prohibit timer for bitrate query. In this contribution, we will give our views on the remaining issues.

2. Discussion

2.1 MAC CE formats

During RAN2#130 meeting, it was agreed that XR UL Rate Control MAC CE can comprise multiple QoS flows. It was excluded to explicitly signal PDU session ID and QFI in the MAC CE. RAN2 should further consider the overhead and the number of QoS flows to down-select from the options listed in the following agreement.

RAN2#130 agreements: (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. We rule out explicit signalling of PDU session ID + QFI We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping

From the perspective of overhead, using explicit DRB ID and QFI ID should be excluded first. For each QoS flow, it will consume 11bits to indicate the corresponding DRB ID and QFI, which is even more than the number of bits used to indicate the recommended bitrate (8bits). When there are multiple QoS flows contained in a MAC CE, the overhead will be larger and it is quite inefficient.

To reduce the overhead, some people proposed to indicate the QFI implicitly, e.g., using a bitmap to identify the QoS flows associated with the DRB, while the DRB ID is still signalled explicitly. In our view, this has very limited effectiveness. The intention to comprise multiple QoS flows within one MAC CE is to perform rate control for multiple QoS flows associated with a multi-modal service at the same time. However, it is highly possible that QoS flows associated with a multi-modal service are mapped to different DRBs since their QoS requirements are different. In this case, to indicate the multiple QoS flows in the same MAC CE, multiple DRB IDs should be included, leading to more overhead.

Observation 1: Explicit DRB ID should not be included in UL Rate Control MAC CE.

Thus, more implicit way to indicate the QoS flows is preferred. Specifically, the gNB can map the QoS flows subject to UL rate control to some short indexes or a bitmap, and provide the mapping relationship to the UE in advance. When the gNB wants to perform UL rate control for one or more QoS flows, it can indicate the QoS flows using the pre-configured indexes or bitmap in the MAC CE, which has less overhead compared to explicit DRB ID or QFI. To minimize the overhead, a bitmap should be used in the MAC CE since it only requires 1bit for each QoS flow.

Proposal 1: [MAC-3, 4] Use a bitmap to indicate the QoS flows configured for rate control in UL Rate Control MAC CE.

Each bit in the bitmap would correspond to a QoS flow which is pre-configured for UL rate control. During the pre-configuration, the gNB can indicate the QoS flows using QFI combined with PDU session ID or DRB ID. Both options can work, but PDU session ID is more stable since DRB can be released and remapped, leading to the change of DRB ID.

When the UE receives the MAC CE, it can be specified that the i-th bit in the bitmap corresponds to the i-th QoS flow among the configured QoS flows based on ascending order of the PDU session ID and QFI. When the bit is set to 1, the recommended bit rate for the corresponding QoS flow is included the in the MAC CE. Otherwise, there is no recommended bit rate for the QoS flow. This would be similar, for example, to a way DRB ID is currently specified in TS 38.321 for Duplication Activation/Deactivation MAC CE.

Proposal 2: [MAC-3, 4] The bitmap in UL Rate Control MAC CE corresponds to the list of QoS flows configured for bit rate control.

It was also agreed to support bit rate query procedure for XR. Similar to the legacy ANBR mechanism, a prohibit timer s configured for the QoS flow which is subject to rate query. The Rate Query MAC CE should follow the same format as the Rate Control MAC CE. Multiple QoS flows can be included in the MAC CE with the same bitmap as in UL Rate Control MAC CE. When there is UL resource allocated for a new transmission, the MAC entity can generate a single MAC CE to include the Rate Query information of all the QoS flows which triggered the rate query procedure and for which the corresponding prohibit timer is not running.

Proposal 3: [MAC-3, 4] The Rate Query MAC CE has the same format as the Rate Control MAC CE.

It may happen that there is no enough UL resource to accommodate all the pending bit rate queries in a single UL Rate Query MAC CE. In this case, the UE can send a MAC CE with fewer QoS flows, and remain the bit rate queries for the other QoS flows until more UL resource becomes available. It can be left to the UE implementation to decide which QoS flows to be included in the MAC CE and they can be indicated via the bitmap. There is no need to define additional MAC CE formats for this.

Proposal 4: [MAC-5] It is up to UE implementation on which QoS flows to be included in the MAC CE when the UL-SCH resource cannot accommodate all the pending bit rate queries.

To show the specification impacts, we provide the following TP which defines the configuration and the MAC CE for UL rate control and query procedure.

------ Change to TS 38.331 ------ MAC-CellGroupConfig ::= SEQUENCE { ------ unchanged part skipped ------ [[ rateControlQoSFlowList-r19 SetupRelease {RateControlQoSFlowList-r19} OPTIONAL, -- Need M ]] } RateControlQoSFlowList-r19 ::= SEQUENCE (SIZE(1..maxNrofRateControlQoSflow-r19)) OF RateControlQoSFlow-r19 RateControlQoSFlow-r19 ::= SEQUENCE { pdu-Session PDU-SessionID, qfi QFI RateQueryProhibitTimer ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30} OPTIONAL, -- Need R } RateControlQoSFlowList A list of QoS flows for which the uplink rate control, as specified in TS 38.321 [3], is configured.. RateQueryProhibitTimer The timer is used for UL rate query in TS 38.321 [3], in seconds. Value s0 means 0 s, s0dot4 means 0.4 s and so on..

------ Change to TS 38.321 (based on running CR [1])------ 6.1.3.x Uplink Rate Control MAC CE The Uplink Rate Control MAC CE is identified by a MAC subheader with an eLCID as specified in Table 6.2.1-1 and Table 6.2.1-2 for available bit rate recommendation from the serving gNB and bit rate query from the UE, respectively. The fields in the Uplink Rate Control MAC CE are defined as follows: - Qi: In bit rate recommendation from the serving gNB and bit rate query from the UE, this field indicates the presence of the Bit Rate field for the QoS flow i, where i is the ascending order of the PDU session ID and then the QFI among the QoS flows configured for uplink rate control and associated with this MAC entity. The Qi field set to 1 indicates that the Bit Rate field for the QoS flow i is present. The Qi field set to 0 indicates that the Bit Rate field for the QoS flow i is not present. - Bit rate: In bit rate recommendation from the serving gNB, this field indicates a recommended bit rate for the indicated QoS flow. In bit rate query from the UE, this field indicates a desired bit rate. The MAC entity shall use the bit rates specified in Table 6.1.3.x-1 to set the value of this field. This field is indicated in kbits/s. The length of this field is 8 bits. Editor’s note: The length of the Bit rate field may change, depending on the final format of the MAC CE. Q7; Q6; Q5; Q4; Q3; Q2; Q1; Q0; Oct 1. Bit Rate 1; Oct 2. …. Bit Rate m; Oct m+1. Figure 6.1.3.x-1: UL Rate Control MAC CE

2.2 UE capability

According to the latest running CR [2], there are two UE capabilities defined for UL rate control, corresponding to the functionality of rate control and rate query, respectively. Some companies argue that the maximum number of QoS flows subject to rate control should be a new UE capability since there can be up to 960 QoS flows in a UE and it is impossible to consider so many QoS flows for rate control. Hence, a UE capability for the maximum number of QoS flows that can be rate controlled should be introduced. However, considering the network situation and the service requirements and traffic amounts, there should not be many QoS flows suitable for rate control, because it is usually a small number of QoS flows that contribute the majority of the traffic, and rate control is useful only when it is applied to these QoS flows. Thus, the maximum number of QoS flows subject to rate control should not be very large and it can be constantly defined as a number acceptable to most UEs. There is no necessity to let the UE report the supported number of QoS flows additionally.

Proposal 5: [UE capability-05] There is no need to introduce additional UE capabilities for maximum number of QoS flows.

Another issue related to UE capability raised during the e-mail discussion was whether CN should be aware of UE’s capability on rate control. It was claimed that if the CN is unaware of the UE capability, it may result that the information provided to the RAN is ignored when the UE does not support UL rate control. However, it is not a critical issue in our view. Actually, even if the CN is aware of the UE capability, it is still possible that the gNB does not configure any rate adaptation for the UE since there is no congestion or for some other reasons, which also makes the provided information to be ignored. Considering that this information is provided over the network interface and does not add much additional overhead, it is not a big issue if it is occasionally provided but not used by the gNB eventually.

Proposal 6: There is no need to let CN know the UE's capability for UL rate control.

3. Conclusion

In this contribution, we discussed the remaining issues for XR rate control. The following observations and proposals were made:

MAC CE formats:

Proposal 1: [MAC-3, 4] Use a bitmap to indicate the QoS flows configured for rate control in UL Rate Control MAC CE.

Proposal 2: [MAC-3, 4] The bitmap in UL Rate Control MAC CE corresponds to the list of QoS flows configured for bit rate control.

Proposal 3: [MAC-3,4] The Rate Query MAC CE has the same format as the Rate Control MAC CE.

Proposal 4: [MAC-5] It is up to UE implementation on which QoS flows to be included in the MAC CE when the UL-SCH resource cannot accommodate all the pending bit rate queries.

UE capability:

Proposal 5: [UE capability-05] There is no need to introduce additional UE capabilities for maximum number of QoS flows and CN awareness for UL rate control.

Proposal 6: There is no need to let CN know the UE's capability for UL rate control.

4. Reference

[1]. [POST130][505][XR] MAC running CR and open issues, https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23130%5D/%5BPOST130%5D%5B505%5D%5BXR%5D%20MAC%20running%20CR%20and%20open%20issues%20(Qualcomm)/Running%20CR

[2]. [POST130][509][XR] UE capabilities CRs, https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23130%5D/%5BPOST130%5D%5B509%5D%5BXR%5D%20UE%20capabilities%20CRs%20(Xiaomi)