3GPP TSG-RAN WG2 Meeting #131	R2-2505125

Bangalore, India, 25-29 Aug 2025

Agenda Item:	8.1.4

Source:	NEC

Title:	Analysis of solutions for UE side model data collection

Document for:	Discussion, Decision

# Introduction

Extension discussions were taken during RAN#106 on the way forward for UE side model data collection. A way forward was endorsed in RP-243307.

RAN2 will focus on the discuss on the following aspects, according to the endorsed way forward:

o	Study RAN aspects related to data transfer over UP

o	Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o	Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o	Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o	Discuss on the scalability aspects of CP

At RAN2#129 meeting, the following agreements were reached for UE-side data collection

- Agreements

- Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration:

- Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).  FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

This paper provides further analysis on the solutions identified for UE side model data collection

# Discussion

During the email discussion in [POST130][033][AI PHY] UE Side data collection, the following were suggested to capture in TR38.843 for CP based solution to support the data collection for UE-side model training:

- UE memory allocation.

The collected data may need to be stored in the access stratum (AS) buffer for the control plane-based data transfer via AS.

- Segmentation for UE side data collection.

A single RRC message can contain maximum about 9KB data. Thus RRC or higher layer segmentation is needed to transfer more data that can fit in a single RRC message. The existing RRC message only supports up to 16 segments in UL, which amounts to a maximum of 144KB of collected data that can be transmitted.

- Continuity of the collected data reporting.

In case the collected data are not yet fully transferred before a handover, Xn / NG-AP signaling enhancements may be required for the continuity of the data reporting. Similarly, it could be further discussed the case of collected data not yet fully transmitted before a radio link failure, or before transitions to IDLE/INACTIVE mode.

During Rel-19 time frame, RAN2 may be not able to assess the above issues and conclude a CP based solution to support the data collection for UE-side model training. However, we think the above analysis does not discuss a new protocol layer above RRC layer to handle the data collection including the data segmentation for the recorded data within the UE, which may reduce the requirements to segment the data at RRC layer and avoid the specification impact for RRC layer. Hence we have the following proposal:

Proposal-1: The study of CP based solution to support the data collection for UE-side model training  is postponed to the next release.

According to the analysis of the different options identified for UE side model data collection, the UP-based data transfer can apply to all of the options (including option 1a/1b, option 2 and option 3). As supported by legacy approach, for the data transfer over UP, it is the AMF/SMF in 5GC that control the establishment of the PDU session and the corresponding tunnel between UE and UPF. On top of the current system architecture in 5G CN, the existing NWDAF (Network Data Analytics Function) entity in the current 5G CN domain may be used for Data Collection Function purpose for AIML model training for option-2. As such the UP-based data transfer may be the better option.

Proposal-2: Data transfer over UP for UE side model data collection is prioritized for Option-2.

Proposal-2a: The existing NWDAF (Network Data Analytics Function) entity in the current 5G CN domain may be used for Data Collection Function purpose for AIML model training for Option-2.

In case of UP based data transfer, it is not clear, how the UE is configured to collect and report the data collected at the UE to the 5G CN via UP tunnel. One possibility is to allow the AMF to configure the UE via NAS layer signalling the data collection configuration and data report criteria. However, it should be noted that traditionally, the UE is configured by RRC signalling for any type of data report. In addition, the 5G CN may be not able to know which type of data the UE should collect and report. It is not clear if the NWDAF can be assumed to hold these type of information.

Proposal-2b: To support Data transfer over UP for UE side model data collection, NAS layer may be employed to configure the data collection configuration and data report criteria to the UE for Option-2.

From the data collection procedure perspective, for Option-2, it would be essential if it is UE initiated procedure or network initiated procedure. If it is UE initiated procedure, it is like the traditional procedure for UE triggered uplink data transfer to the network. The signalling procedure should be governed by NAS layer between the UE and AMF/SMF. If it is network initiated procedure, a paging alike procedure may be used by 5GC to request the NG-RAN node to page the UE to trigger the report for its collected data. Then a special type of paging may be sent from NG-RAN node to the UE. The same principle may apply to Option-3 also.

Proposal-3: RAN2 to discuss if it is UE initiated procedure or network initiated procedure to trigger the data report from the UE to the network.

Proposal-3a: NG-RAN may be involved in the paging procedure to page the UE to trigger the report for its collected data in case of network initiated procedure.

Based on the discussion above, a TP is drafted for UP based solution.

Proposal-4: Agree the TP in the Annex

# Conclusion and Proposal

We have the following observations and proposals:

Proposal-1: The study of CP based solution to support the data collection for UE-side model training  is postponed to the next release.

Proposal-2: Data transfer over UP for UE side model data collection is prioritized for Option-2.

Proposal-2a: The existing NWDAF (Network Data Analytics Function) entity in the current 5G CN domain may be used for Data Collection Function purpose for AIML model training for Option-2.

Proposal-2b: To support Data transfer over UP for UE side model data collection, NAS layer may be employed to configure the data collection configuration and data report criteria to the UE for Option-2.

Proposal-3: RAN2 to discuss if it is UE initiated procedure or network initiated procedure to trigger the data report from the UE to the network.

Proposal-3a: NG-RAN may be involved in the paging procedure to page the UE to trigger the report for its collected data in case of network initiated procedure.

Proposal-4: Agree the TP in the Annex

# Reference

- RP-243307

- RP-242850

- Email discussion summary, [POST130][033][AI PHY] UE Side data collection

# Annex: TP to be included in 7.2.1.3.2 of TR38.843

7.2.1.3.2.X	Data collection for UE-side model training – UP solutions analysis

To support data transfer over UP for UE side model data collection (Option-2), NAS layer may be employed to configure the data collection configuration and data report criteria to the UE. The existing NWDAF (Network Data Analytics Function) entity in the current 5G CN domain may be used for Data Collection Function purpose for AIML model training. Either UE initiated procedure or network initiated procedure may to used to trigger the data report from the UE to the network. In case of network initiated procedure, a paging alike procedure may be used by 5GC to request the NG-RAN node to page the UE to trigger the report for its collected data. A special type of paging may be sent from NG-RAN node to the UE for such purpose.