3GPP TSG-RAN WG2 Meeting #131	R2-2505135

# Bangalore, India, 25 – 29 Augugst 2025

Agenda item:		8.6.3

Source:	Xiaomi

Title:	Remaining issues of L1 event triggered measurement reporting

Document for:		Discussion and Decision

# Introduction

In RAN2#130 meeting, following was agreed regarding L1 event triggered measurement reporting [1]:

Agreements on L1 event-triggered MR

- UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell.

- Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

- Confirm working assumption, i.e. four codepoints to represent one of {type1, type2, type3, type4}, as an agreement. For detailed description of type 1, it can be handled as part of running CR preparation.

- (MAC-6) For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam. If the UE cannot include all of the beams with the same priority, the beams are selected in the (truncated) MR MAC CE based on measured RS quality. Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE.

- Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

- Same priority between regular MR MAC CE and truncated MR MAC CE. It is up to UE implementation how to handle/include if more than one triggering events are pending and UL grant is not sufficient for all regular L1 MR MAC CEs.

- No prohibit timer is applied to L1 event-triggered MR.

- 16 beams in total including the current beam in MR MAC CE.

- The beam offset for current beam of serving cell, i.e. Obs in the event evaluation inequality, is needed for the LTM event evaluation. Obs offset is configured per-cell, while applied at the level of indidual beam. Obs offset is applied to LTM3.

- The Semi-persistent CSI-RS resource set for candidate cell MAC CE is initially deactiated upon (re-)configuration by upper layer.

- RAN2 assumes there is no L1-SINR quantity for L1 event triggered MR.

- It is up to the NW configuration whether the UE can include measurements for LTM candidates in the MR MAC CE triggered by LTM2, i.e. based on the current configuration in RRC on the number of beams for LTM2 included in the MAC CE.

- For MR triggered by LTM2, if measurements for LTM candidates is configured to be included in the MR MAC CE, which LTM candidate beam should be included: Only RS(s) associated with the report ID for LTM2.

- The triggered MR is not cancelled when transmitting truncated MAC CE. The triggered MR is cancelled if regular MAC CE is reported, as in current MAC running CR.

- When a beam (or multiple beam) meets the entry condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the leaving condition, then, the corresponding MR should be cancelled.

- When a beam (or multiple beam) meets the leaving condition for TTT, and before the UL grant occasion, this beam (or all these beams) meets the entry condition, then, the corresponding MR should be cancelled.

- When MR is triggered, if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, UE will trigger RACH procedure, as in legacy.

- (RRC-10) Follow RAN1 agreement which indicate that at least one candidate RS (within the LTM CSI resource configuration) shall be configured for event LTM2.

In this contribution, we discuss the remaining issues of the L1 event triggered measurement reporting.

# Discussion

## (MAC-27) Event Triggered L1 MR and beam failure

In email discussion “[POST130][111][MOB] (Vivo)” [6], open issue MAC-27 is as follows:

| MAC-27 | Whether/how to cancel the triggered Event Triggered L1 MR, and whether/how to reset TTT upon beam failure [Rapp]: New added based on Anil’s comments. Sine the event triggered MR procedure is triggered based on the current beam, the procedure may be cancelled upon the beam failure on the SpCell. The TTT also may be stopped. i.e. the following may be needed. Upon beam failure on the SpCell: 1>cancel, if any, triggered Event Triggered L1 Measurement Report; 1>reset TTT for event triggered L1 measurement report triggering condition evaluation; Since it is not discussed an FFS may be added and this issue can be added in open issue list. | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

Currently upon beam failure on SpCell, BFR procedure or random access procedure is triggered, according to TS 38.321 clause 5.17:

The MAC entity shall for each Serving Cell configured for beam failure detection:

1>	if the Serving Cell is configured with two BFD-RS sets:

2>	if beam failure instance indication for a BFD-RS set has been received from lower layers:

…

3>	if BFI_COUNTER of the BFD-RS set >= beamFailureInstanceMaxCount:

4>	trigger a BFR for this BFD-RS set of the Serving Cell;

2>	if BFR is triggered for both BFD-RS sets of the SpCell and the Beam Failure Recovery procedure is not successfully completed for any of the BFD-RS sets:

3>	initiate a Random Access procedure (see clause 5.1) on the SpCell;

…

1>	else:

2>	if beam failure instance indication has been received from lower layers:

…

3>	if BFI_COUNTER >= beamFailureInstanceMaxCount:

…

4>	else:

5>	initiate a Random Access procedure (see clause 5.1) on the SpCell;

Regarding cancelling the triggered Event Triggered L1 Measurement Report, given that there is beam failure, such L1 MR report can be helpful for the network to make decision on whether UE should stay in current cell or switch to another cell. L1 MR MAC CE can be sent after BFR or RACH procedure is completed.

With BFR or random access procedure, either a new serving beam is selected or the old serving beam is kept for use. No matter the serving beam changes or not, the note below in MAC running CR [5] still applies and there seems no strong motivation to stop timeToTrigger:

NOTE x:	timeToTrigger is not restarted if the current beam of serving cell changes and the entry condition is still met with the new current beam.

Proposal 1: (MAC-27) Specification changes are not needed for event triggered L1 MR upon beam failure on SpCell.

## Event configuration

SpCell or serving cell in event LTM2/3/5

In RRC running CR [3], the definition of event LTM2/3/5 are as follows:

Event LTM2:	Beam of SpCell becomes worse than absolute threshold;

Event LTM3:	Beam of candidate cell becomes amount of offset better than the beam of SpCell;

Event LTM5:	Beam of SpCell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In MAC running CR [5], the definition of event LTM2/3/5 are as follows:

Event LTM2 (Beam of serving cell becomes worse than threshold)

Event LTM3 (Beam of candidate cell becomes offset better than beam of serving cell)

Event LTM5 (Beam of serving cell becomes worse than threshold1 and Beam of candidate cell becomes better than threshold2)

It can be seen that SpCell and “serving cell” are used in RRC CR and MAC CR, respectively. Layer 3 event A3 and A5 are both defined for SpCell. Although layer 3 event A2 is defined for serving cell, the main purpose of event A2 is for SCell management, which is not the case for Event LTM 2. It is suggested that RAN2 to clarify whether serving cell in Event LTM 2/3/5 refers to SpCell only.

Proposal 2: RAN2 to clarify whether serving cell in Event LTM 2/3/5 refers to SpCell only.

## SR of Event Triggered L1 MR MAC CE

In the RAN2#127bis meeting, RAN2 made the following agreements for the SR used for the Event Triggered L1 MR MAC CE:

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In RAN#129 meeting, RAN2 agreed that “The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.”

In general, we think that the triggering/cancellation/priority of the dedicated SR for the Event Triggered L1 MR MAC CE can reuse the procedure defined for the dedicated SR for BFR MAC CE. This means that the dedicated SR is triggered when a L1 measurement report is triggered. When the L1 measurement report is cancelled (e.g. due to the report/cancellation of the Event Triggered L1 MR MAC CE), the dedicated SR is also cancelled. The related note in TS 38.321 is copied below:

NOTE 3:	When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

According to TS 38.321 as quoted above, the BFR SR has higher priority than the BSR SR, because the beam failure recovery is more urgent than the data transmission. From our understanding, the transmission of Event Triggered L1 MR MAC CE should also have higher priority, so that the LTM cell switch can be triggered in time. This means that the priority of the SR for Event Triggered MR MAC CE should be higher than that of the SR for BSR.

Proposal 3: The dedicated SR for Event Triggered L1 Measurement Report MAC CE has higher priority than the SR for BSR.

## UE capability Issue 8

In “[POST130][120][MOB] (CATT)” [2], open issue 8 is: “Whether to define a capability for reporting the beam not satisfying the event condition in the MR MAC CE?” In RRC running CR [3], field allowReportAnyBeam can be configured so that UE can report the beams not satisfying the event condition, as below:

allowReportAnyBeam Indicates whether the UE can report the measurement results for the beams not satisfying the conditions of the events at least for the time duration configured by timeToTrigger as specified in TS 38.321 [3].

Reporting beams not satisfying event condition is an optimization to provide network additional information. As this is not an essential feature, it is preferrable to define a separate capability instead of mandating that any UE supporting LTM events should support reporting beams not satisfying event condition.

Proposal 4: (UE capability-8) Per UE capability is defined for reporting the beams not satisfying the event condition in the MR MAC CE. A UE supporting this feature shall also indicate the support of LTM events.

## Coexistence between event-triggered L1 MR and mTRP

In email discussion “[POST130][117][MOB] (Nokia)” [4], rapporteur proposed that the coexistence between event-triggered L1 measurement reporting and mTRP for the source cell is supported. One open issue is whether the network needs to know the exact TRP and thus MR MAC CE shall indicate the TCI state associated with the reported RSRP value for the serving/current beam.

The purpose of L1 MR MAC CE is mainly to provide the radio information of the candidate cells, so the network can perform related actions, e.g. instruct UE to perform early DL/UL sync or execute cell switch. The provisioning of the serving beam information is best efforts and network can obtain such information (e.g. which is the best serving beam) via other means e.g. CSI report of the serving cell / beam as such information is anyway needed for scheduling. Therefore there is no need to provide the TCI state associated with the reported RSRP of the serving beam.

Proposal 5: For coexistence between event-triggered L1 MR and mTRP, there is no need to provide the TCI state associated with the reported RSRP of the serving beam (i.e. the serving beam for event evaluation) in MR MAC CE.

# Conclusion

In this contribution, we discuss the remaining issues of the L1 event triggered measurement reporting, and propose the following:

Proposal 1: (MAC-27) Specification changes are not needed for event triggered L1 MR upon beam failure on SpCell.

Proposal 2: RAN2 to clarify whether serving cell in Event LTM 2/3/5 refers to SpCell only.

Proposal 3: The dedicated SR for Event Triggered L1 Measurement Report MAC CE has higher priority than the SR for BSR.

Proposal 4: (UE capability-8) Per UE capability is defined for reporting the beams not satisfying the event condition in the MR MAC CE. A UE supporting this feature shall also indicate the support of LTM events.

Proposal 5: For coexistence between event-triggered L1 MR and mTRP, there is no need to provide the TCI state associated with the reported RSRP of the serving beam (i.e. the serving beam for event evaluation) in MR MAC CE.

# References

[1] R2-2504671, Vice Chairman (Samsung), "Report from session on R18 SL, R18/19 MOB and R19 NES"

[2] R2-2505164, CATT, "Report of [POST130][120][MOB]"

[3] R2-2505815, Ericsson, Huawei, "Introduction of RRC changes for mobility enhancements phase 4"

[4] R2-2505719, Nokia, "Report from [POST130][117][MOB] (Nokia)"

[5] R2-2505397, vivo (Rapporteur), "MAC CR for Mob Ph4"

[6] R2-2505398, vivo, "Discussion summary and list of MAC open issue for Mob Ph4"