3GPP TSG-RAN WG2 #131	R2-2505158

# Bengaluru, India, 25 - 29 August 2025

Agenda Item:	8.6.2

Source:	MediaTek Inc.

Title:	UE capability for fast recovery for inter-CU LTM

Document for:	Discussion, Decision

1	Introduction

In RAN2#130, RAN2 agreed that fast recovery after inter-CU MCG LTM cell switch failure is supported towards LTM candidate cells which have the same Rel-19 ID (i.e., ltm-NoSecurityChangeID) as the target cell of the failed inter-CU MCG LTM cell switch.

Agreements on inter-CU LTM

- RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

- If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

- RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE.

- The Enhanced LTM CSC MAC CE always contains an NCC value field. The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

In the post meeting email discussion [POST130][120][Mob], the Question12 asked whether a separate UE capability is needed for inter-CU LTM fast recovery, and the companies gives diverse answers.

In this paper, we discuss about the new UE capability for this functionality.

2	Discussion

Fast recovery after intra-CU MCG LTM cell switch failure and RLF is supported already in Rel-18. In such scenario, the fast recovery is possible towards LTM candidate which belongs to the same gNB-CU as the original source PCell. Therefore, this fast recovery procedure is an intra-CU LTM cell switch and the UE performs following steps during the procedure:

- Keeps current AS security configuration associated with the master key;

- Keeps PDCP and SDAP entities, their state variables, buffers and timers of the radio bearers using the master key;

- Re-establishes RLC entities of the MCG RLC bearers according to explicit network indication (ltm-NoResetID);

- Performs PDCP data recovery for the AM DRBs according to explicit network indication (ltm-NoResetID);

- Resets MAC;

- Configures lower layers according to the LTM candidate configuration.

The UE indicates support for this functionality via UE capability ltm-Recovery.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ltm-Recovery-r18 Indicates whether the UE supports recovery procedure for MCG LTM execution when the selected cell in RRC re-establishment procedure is a LTM candidate as specified in TS 38.331 [9]. UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18 for at least one band. | UE | No | No | No |

During Rel-19 fast recovery procedure after inter-CU MCG LTM cell switch failure, the UE performs following steps:

- Generates a new master key using NCC in the LTM cell switch command;

- Re-establishes RLC entities of the MCG RLC bearers;

- Re-establishes PDCP entities of the radio bearers using the master key;

- Reset MAC;

- Configure lower layers according to the LTM candidate configuration.

- The Rel-19 fast recovery after inter-CU LTM cell switch failure is very different procedure from UE perspective compared to the Rel-18 fast recovery after intra-CU LTM cell switch failure and RLF.

- The problem is that the UE capability ltm-Recovery-r18 currently indicates UE support for both Rel-18 fast recovery procedure and Rel-19 fast recovery procedure. It is true that it is up to UE to select the candidate cell for fast recovery, according to the capability it supports. However, this means that the UE which indicates support for Rel-19 inter-CU LTM and ltm-Recovery-r18 is required to have also Rel-19 fast recovery procedure implemented and tested. However, depending on infra vendors' support, there might not be enough verification possibilities for Rel-19 fast recovery procedure. This can in practice prevent Rel-19 UEs from indicating support for ltm-Recovery-r18.

- Rel-19 UEs will not be able to indicate support for ltm-Recovery-r18 even if they support Rel-18 fast recovery procedure, if there are no verification possibilities for Rel-19 fast recovery procedure.

To prevent this issue from happening, we propose to define a new UE capability to indicate the UE support for Rel-19 fast recovery procedure.

New UE capability is defined to indicate the UE support for Rel-19 fast recovery procedure.

3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	New UE capability is defined to indicate the UE support for Rel-19 fast recovery procedure.