3GPP TSG-RAN WG2 Meeting #131                                                               R2-2505168

Bangalore, India, 25th – 29th Aug, 2025

Source:	CATT

Title:	Consideration on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

In last RAN2#130 meeting [1], on-demand SSB was discussed with the following agreements:

Agreements on OD-SSB

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

- The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed.

- When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

• OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

- For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

In this contribution, we share our view on the remaining issues of OD-SSB. Our proposals are summarized in Section 3.

Discussion

Clarification of od-ssb-nrofBurst activation by RRC

In the last RAN1 #121 meeting, following agreements have been achieved:

Agreement (RAN1 #121) For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

According to the above agreement of RAN1, OD-SSB transmission configuring od-ssb-nrofBurst can only be activated by MAC CE.

In the current running CR, od-ssb-nrofBurst is configured in parallel with od-ssb-ActivationStatus in OD-SSB-Config-r19. If the conclusion of RAN1 is supported, a restriction needs to be added that od-ssb-ActivationStatus should be absent when od-ssb-nrofBurst is configured. From the perspective of RAN2, there is no clear motivation to prevent od-ssb-nrofBurst activation by RRC, especially in Case#2, where OD-SSB can be configured to be implicitly deactivated in the initial configuration. Therefore, we propose:

Proposal 1: Support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst.

Reference cell for od-ssb-sfn-Offset

Based on the current running CR, the od-ssb-sfn-Offset is used to calculate the time domain location of OD-SSB. According to the field description, the od-ssb-sfn-Offset indicates OD-SSB SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0.

od-ssb-sfn-Offset Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

However, we think that the description in current spec is still a little unclear. It is possible that the SFN across carriers in a cell group is not unaligned, so the time domain location of OD-SSB based on the SFN of different cells as a reference time is different. For example, in Case#1, where UE does not measure AO-SSB, it is unclear whether UE uses the SFN of SCell as the reference time or the SFN of SpCell as the reference time. In order to avoid confusion about the understanding of SFN for those who are not familiar with OD-SSB SCell operation, we propose to add the clarification of the SFN in the filed description of od-ssb-sfn-Offset.

Proposal 2: SFN of SCell serves as the timing reference of the offset indicated by od-ssb-sfn-Offset, and a clarification for this is to be added in the filed description of od-ssb-sfn-Offset.

Configuration of servingCellMO-r19

In the last post email, the issue with the configuration of servingCellMO-r19 was proposed.

| Fujitsu 006 | – OD-SSB-Config RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.; Field servingCellMO is introduced in IE OD-SSB-Config. In this way it is possible to keep the same field name an hence all the procedural text intact.. [Fujitsu] RAN2 agreed to introduce a new servingCellMO in ServingCellConfig in RAN2#130. Although the current CR may work, we think the original agreement is simpler for implementation, because it can keep the same message structure. We also think even if it is introcuded in ServingCellConfig, the current procedural text is not changed (similar to servingCellMO-r17). We would like to add it into open issue list and ask other company’s views. |
| --- | --- |

We support the proposal of Fujitsu, servingCellMO-r19 is configured per cell, which should be configured in ServingCellConfig in the same way as legacy.

Proposal 3: Include servingCellMO-r19 in ServingCellConfig, instead of in OD-SSB-Config-r19 as specified in the current RRC running CR.

Configuration of od-smtc-r19

According to the RAN1 agreement, the OD-SSB configuration can be updated by MAC CE when SCell is activated.

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Based on the above agreement, even when AO-SSB and OD-SSB are in different frequencies, the transmission parameter of OD-SSB, e.g., OD-SSB periodicity, can still be updated through MAC CE.

In the RAN2#129bis meeting, an OD-SSB specific SMTC was introduced when AO-SSB and OD-SSB in the same frequency, with the following agreements:

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

In order to meet the requirements that the OD-SSB configuration can be updated after SCell activation in RAN1 agreement, the OD-SSB specific SMTC should also be applied when AO-SSB and OD-SSB are in different frequencies as well. Therefore, it should be clarified that OD-SSB specific SMTC is applied when the OD-SSB is activated and SCell is activated, regardless if AO-SSB and OD-SSB are in the same frequency or different frequencies. And if it is agreed, the condition of od-smtc-r19, i.e., Cond IntraFreq, should be removed.

Proposal 4: The od-smtc-r19 is also applied if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell, i.e., Conditional Presence of IntraFreq should be removed for od-smtc-r19.

Handling of OD-SSB measurement after SCell deactivation

In the previous RAN1 meeting, it was discussed whether OD-SSB still transmits when SCell is deactivated, but no consensus was reached.

Agreement RAN1#118b For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

Regardless of whether OD-SSB is still transmitted, it is meaningless for UE to measure OD-SSB after SCell deactivation. Therefore, UE should stop measuring OD-SSB after SCell deactivation.

Proposal 5: After SCell deactivation, UE stops the OD-SSB measurement and considers the OD-SSB as deactivated.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Clarification of od-ssb-nrofBurst activation by RRC

Proposal 1: Support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst.

Reference cell for od-ssb-sfn-Offset

Proposal 2: SFN of SCell serves as the timing reference of the offset indicated by od-ssb-sfn-Offset, and a clarification for this is to be added in the filed description of od-ssb-sfn-Offset.

Configuration of servingCellMO-r19

Proposal 3: Include servingCellMO-r19 in ServingCellConfig, instead of in OD-SSB-Config-r19 as specified in the current RRC running CR.

Configuration of od-smtc-r19

Proposal 4: The od-smtc-r19 is also applied if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell, i.e., Conditional Presence of IntraFreq should be removed for od-smtc-r19.

Handling of OD-SSB measurement after SCell deactivation

Proposal 5: After SCell deactivation, UE stops the OD-SSB measurement and considers the OD-SSB as deactivated.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #RAN2 130