3GPP TSG-RAN WG2 Meeting #131	R2-2505186

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.3.2.1

Source:	OPPO

Title:	Discussion on functionality management for RRM measurement prediction

Document for:	Discussion, Decision

Introduction

This paper focuses on following functionality management aspects for RRM measurement prediction use case:

1, Applicability procedure

2, Inference configuration and reporting procedure

3, Performance monitoring

In general, RAN2 made some progress in those aspects at last RAN2 meeting with some left open issues. Meanwhile work item NR_AIML_air also achieved some agreements, which could also help to make progress for AI mobility. In this paper progress from both sides are examined for potential proposals.

Applicability procedure

Applicability reporting

Following agreements are made at RAN2#130 meeting for applicability procedure:

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

In RAN2#130 meeting, RAN2 agreed following inference parameters:

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

Table 2.1-1

Those inference parameters together with optional associated ID can be used by UE to assess when an inference configuration can be applicable or not. In this paper if a UE is only configured with those key inference parameters without detailed measurement task configuration, it is called “partial configuration”. If a UE is configured with those key inference parameters together with a detailed measurement task, it is called “full configuration”. A measurement task is configured with a measurement object and a measurement configuration, which is usually based on UE’s radio capability. Since UE’s radio capability can be known before the whole applicability procedure, so the essential part of the applicability procedure for AI mobility is to check whether partial inference configuration is applicable or not.

Observation 1: For AI Mobility, the essential part of the applicability procedure is to check the applicability of inference parameters in partial configuration.

Without knowing the applicability of partial configuration, gNB can also configure those inference parameters together with a measurement task. But the main problem is that such blind configuration will be wasted when UE response that configured inference configuration is not applicable. To avoid such problem, gNB can configure partial configuration, which could contain a range of inference parameters. For example, for temporal domain case A, gNB can configure a range of prediction window length as inference parameter. If the whole range of prediction window length is applicable, UE can response they are applicable. If not, UE can answer they are not applicable. In addition, UE should be allowed to response more information i.e. applicable range of prediction window.

In NR_AIML_air item, it is agreed in RAN2#129bis about release cause when UE answering inapplicability:

Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

While in RAN2#130 meeting, RAN2 further agreed that:

1 Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified)

As the agreement in RAN2#130 indicated the “release” flag is useful when inapplicability is “due to model in availability in the local device”. If the inapplicability is not due to such internal reason but due to inference parameter from gNB, then UE should response more useful information to gNB apart from “release” flag. “Release” flag only doesn’t provide useful information for gNB and what gNB can do is to further guess. Another approach is that gNB can configure a “high level” parameter so that UE can response with detail supported inference parameter together with applicability. Here is the summary of 3 options:

- Option 1: UE response yes (applicable) or no (inapplicable + optional release flag)

- Option 2: UE response yes (applicable) or no (inapplicable + supported inference parameters)

- Option 3: UE response to a high-level parameter yes (applicable + supported inference parameters) or no (inapplicable)

Taking temporal domain case A as example, here are the potential message content in step 3 and step 4:

| Index | Prediction window in step 3 | Message in step 4 |
| --- | --- | --- |
| Option 1 | Configuration 1 : 80ms Configuration 2 : 160ms Configuration 3 : 320ms | Configuration 1: applicable Configuration 2: inapplicable (+ release flag) Configuration 3: appliable |
| Option 2 | Configuration 1: 80ms, 160ms, 320ms | Configuration 1: inapplicable + applicable window: 80ms, 320ms |
| Option 3 | Configuration 1: support of temporal domain case A | Configuration 1: applicable + applicable window 80ms, 320ms |

Table 2.1-2

Assuming the inference parameters has wide range, option 1 is least efficient way for gNB to get useful information about inference parameter because of more parameter configurations. If gNB has already known UE’s capability on AI mobility, option 2 is more efficient because it is also possible for UE to simply answer yes(applicable) for all listed inference parameters. If gNB has least AIML capability information, option 3 can be also an efficient way.

Furthermore, even for full configuration, same idea is also feasible and helpful. In this case only 1st full configuration message may take risk of blind configuration.

Proposal 1: When reporting applicability, UE can response applicable inference parameters

For partial configuration approach, if Proposal 1 is applied, it means gNB can know which inference parameters and corresponding functionality is supported by UE. gNB then can configure a full configuration for UE. In this case it most likely that UE will response such full configuration as applicable.

Observation 2: A full configuration after partial configuration will be most likely applicable

UE may be able to support multiple AI mobility functionalities. For same functionality it doesn’t make sense for gNB to configuration partial configuration and full configuration in the same message. But for different functionality it should be allowed. For example, in one round gNB may configure a full configuration for functionality A but partial configuration for functionality B. After getting response from UE about functionality A and B, gNB can further configuration full configuration for functionality B. There is also a relevant agreement in RAN2#130 as following:

3 (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

This agreement can be further adjusted for AI mobility:

Proposal 2: Partial configuration and full configuration for different AI mobility functionality/scenario can be configured in the same RRCReconfiguration message

For full configuration, when an inference configuration is not applicable, RAN2 agreed to keep it. And it is up to network’s implementation to release it. When an applicable full configuration becomes inapplicable, UE’s behaviour is agreed in RAN2#130 for NR_AIML_air:

1. When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). 2. The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable. 3. The UE shall report when CSI-ReportConfig becomes not applicable

We think similar scheme can be applied for AI mobility use case. For full configuration UE continue to perform the inference and reporting. UE shall report when one inference configuration becomes not applicable. For partial configuration since inference is not started yet, so it should be fine. For full configuration gNB can de-configure those key parameters so that UE can start legacy RRM measurement after the procedure. For inter-frequency prediction network control is necessary because measurement on additional frequency carrier usually demands measurement gap configuration. For temporal domain and spatial domain, fallback to legacy RRM measurement is possible. But the handshake between UE and gNB is relative short considering the measurement period is at least 200ms and 400ms for FR1 and FR2.

Proposal 3: When an inference configuration become inapplicable, UE shall report it to network via applicability update procedure. Autonomous release is not supported.

Proposal 4: For a full configuration, UE continues to perform the inference and reporting until the configuration is released after the UE reports it is inapplicable.

In RAN2#129bis meeting, it is agreed:

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required.    FFS when the UE can perform inference

When an inference configuration is applicable, UE will apply it immediately. UE should also activate it immediately since no further L1/L2 signaling is required. However, it doesn’t mean UE can start perform inference immediately. For partial configuration approach, it is obviously since UE can’t even be able to perform measurement without detail measurement task. For full configuration, there are two options:

- Option 1: UE start inference immediately

- Option 2: Inference is only triggered when some condition is met.

Some company believe that for temporal domain case A it is not necessary to do inference until when measurement event supposes to happen soon e.g. at cell edge. We think it is kind of optimization. Network can also configure another measurement event with relaxed threshold so that gNB can learn the proper time to configure UE to start inference based on temporal domain case A. For other cases like temporal domain case B, frequency domain prediction etc. the main motivation is to save UE’s efforts on RRM measurement and hence option 2 is more reasonable. On the other hand, it makes no sense to do any inference if UE has not start measuring at all. The natural way is to align inference with RRM measurement itself i.e. whenever UE start RRM measurement UE should start inference for AIML functionality corresponding to applicable inference configuration. It basically means any relaxing scheme e.g. s-measure is also applied for inference operation automatically.

Proposal 5: UE starts/stops inference for AIML functionality corresponding to applicable inference configuration whenever UE starts/stops RRM measurement as in legacy.

For full configuration, RAN2 agreed in RAN2#130:

Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly

In AI Mobility all the inference configuration is configured via RRC signalling. For partial configuration, UE can keep the inference configuration if it applicable. If one inference configuration is not applicable, there is no harm for UE to keep it since no inference will be performed. To make the whole scheme simple, the agreed principle for full configuration could be also applied for partial configuration.

Proposal 6: Upon receiving one or more partial inference configuration(s) via RRCReconfiguration message, UE shall maintain all the partial inference configuration(s) no matter the partial inference configuration is applicable or inapplicable until the network releases it explicitly

For full configuration, RAN2 agreed in RAN2#130:

Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes

Full configuration and partial configuration have both initial report and update of applicability. But it is not clear what message will be used for partial configuration. In RAN2#130 meeting in NR_AIML_air, it is confirmed that same messages are used for option A and option B:

1 (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

We think same principle can be also applied by partial configuration in AI mobility.

Proposal 7: For partial configuration UE reports initial applicability in RRCReconfigurationComplete message and UE updates applicability via UAI message.

Applicability report during mobility

One more issue is applicability report during mobility. There is agreement for BM use case in RAN2#128 meeting:

Agreements (RAN2#128)

Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Table 2.2-1

Our understanding is that target gNB may be interested in different functionality from source gNB but with same knowledge of UE context after signalling exchanging over Xn interface. So logically target gNB can check with UE via handover command directly and UE need answer in RRCReconfigurationComplete (i.e. upon handover complete message). For AI mobility use case, the logic is the same.

Proposal 8: During mobility, target gNB can check applicability via handover command message i.e. RRCReconfiguration and UE can response with RRCReconfigurationComplete.

Inference configuration and reporting

For UE sided model, RAN2 concluded briefly on what and how to report to gNB at RAN2#129bis meeting:

Agreements (RAN2#129bis)

5	For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

6	For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

7	Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

Table 3-1

For RRM measurement prediction on cell level result, UE should report only L3 measurement result. For sub-use case 1, although the outcome of the model is L1 beam level measurement result. But UE should still report L3 result to network since the L3 result instead of L1 result is wanted by network. Plus, reporting L1 beam level measurement results could be a big burden for Uu signalling since most likely there are multiple L1 beams need be reported.

Observation 3: For RRM measurement prediction, Reporting L1 beam level measurement is not necessary

Proposal 9: For RRM measurement prediction, only L3 cell level measurement results are reported

As for what to report depends on detailed scenarios. For temporal domain case A, UE can report at least one actual measurement result since no measurement will be saved. Then the most interested part is predicted measurement result(s) in PW. For both sliding approach or non-sliding approach with PW>=measurement period, there are at least one measurement results in PW.

Proposal 10: For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

Considering the intention is to report actual and/or predicted measurement result is to help network to predict a potential future measurement event most likely, it could be also beneficial to report more than one actual measurement results in OW. During evaluation phase it seems more actual measurement results in OW are used to predicted measurement event, the more accurate is the prediction.

Proposal 11: For temporal domain case A, one or more instances of measurement results in OW per one cell are reported in one measurementReport message

For temporal domain case B, eventually UE will get both actual and predicted measurement results in OW and/or PW. Since the intention for this scenario is to save measurement in temporal domain, one measurement result for one cell or one beam should be sufficient. No doubt this could be a predicted measurement result. On the other hand, it is also possible there is actual measurement result available before the reporting opportunity regardless it is pure periodic, or event triggered periodic procedure. Because of this one indication should be introduced to indicate whether a measurement result is actual or predicted measurement.

Proposal 12: For temporal domain case B, one measurement result per one cell or beam is reported in one measurementReport message.

Proposal 13: For temporal domain case B, one indication marking actual or predicted measurement result should be also reported along with the measurement result

For frequency domain prediction, for frequency to be predicted, all the measurement results are predicted, and it should be sufficient to just follow legacy report procedure i.e. one measurement result per cell or per cell are sufficient. For spatial domain, the reported measurement result should be also a predicted measurement result because only partial L1 beams participate the L3 filtering to form L3 cell level measurement result. Assuming the configuration for frequency and spatial domain prediction can be distinguished from other scenarios, there is no spec impact at all.

Proposal 14: For frequency domain prediction and spatial domain prediction, one predicted measurement result per one cell is reported in one measurementReport message.

Performance monitoring

Procedure

At RAN2#130s meeting, RAN2 agreed that for UE sided model:

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

Our understanding of the scheme in yellow is that it is like UE-sided monitoring i.e. UE can generate monitoring result i.e. RSRP difference based on network’s configuration. In addition, network further configure some trigger for UE to use e.g. event like trigger. The intention of such event like trigger could be as following:

- If monitoring metric is good enough, UE can skip reporting monitoring metrics until network request it

- If monitoring metric is not good enough, UE can be triggered to report monitoring metrics

- If monitoring metric is bad, UE can report related inference configuration is not applicable.

For the 1st and 2nd intention, they can be combined with UE-sided monitoring. It means after UE generating monitoring metric results UE can either report them only once, or periodically or based on configured trigger. The 3rd intention is different in the sense no monitoring metric result is reported. Instead, UE report that one related inference configuration is not applicable. Such procedure is already covered by applicability procedure. While the event like trigger need be supported.

Proposal 15: For UE sided monitoring, UE can be configured to report monitoring metric result only once, or periodically or based on configured trigger

Proposal 16: UE can be configured to assess whether a full configuration is applicable or not based on monitoring metric results and network’s configuration. And it can be applied for all scenarios in the SID scope.

To get ground truth measurement result, UE need perform measurement by following legacy measurement procedure. But for scenarios to save measurement e.g. temporal domain case B, it matters because UE supposes to skip performing some of the measurement in frequency/temporal/spatial domain.

In addition, UE need provide either ground truth measurement results or calculated metric to gNB. So even for temporal domain case A, this limited duration is also applicable for the sake of signalling overhead unless UE is configured to assess applicability based on calculated metric. To make it flexible, gNB need configure such parameter.

Proposal 17: The performance monitoring duration for UE sided model should be limited and configurable.

If RRM measurement prediction occurs within same frequency layer, UE can get ground truth measurement result by following legacy measurement procedure while running the model. For inter-frequency prediction, the story is bit different. UE should be configured with another legacy measurement task for the predicted frequency layer temporarily.

Observation 4: For inter-frequency prediction, a temporal measurement task for predicted frequency layer should be configured temporarily for monitoring purpose

For metrics, RAN2 agreed RSRP difference is the metrics. To avoid any ambiguity between gNB and UE, it is necessary to discuss further detail of the meaning of the metric.

For same scenario e.g. temporal domain case A, UE should be allowed to configure more than one measurement task to perform inference. The question is whether different measurement task associated with different measId should be monitored separately or not. During evaluation phase we can observe that prediction accuracy is quite different at least between FR1 and FR2. For frequency within same range, the difference could be smaller. But it will be difficult for gNB to differentiate the performance between two measurement tasks. In addition, measurement task could be the granularity of functionality management. The granularity for monitoring should be at least finer or equal to granularity of functionality management.

The next question is whether finer monitoring granularity is necessary e.g. among cells belonging to same frequency layer. Our view is that prediction accuracy among cells belonging to same frequency is quite similar. In addition, even there is slight difference observed among intra-frequency cells, it will be also difficult for gNB to configure UE perform inference only for partial cells.

Proposal 18: For UE side model, the metric is calculated per frequency for L3 cell level prediction i.e. intra-frequency cells should be calculated together.

For temporal domain case A, it is possible that there are more than one predicted measurement results are available in PW. During evaluation phase, 3 kinds of L3 RSRP difference are reported by company:

- Type 1: L3 RSRP difference of each predicted measurement result within PW

- Type 2: Average L3 RSRP difference of all predicted measurement results within PW

- Type 3: L3 RSRP difference of last predicted measurement result within PW

Obviously, the outcome among 3 types is different. RAN2 need decide which one should be used for performance monitoring. If more than one type is specified, then UE should be configured which one is required. Our view is that type 2 should be sufficient. The prediction accuracy usually decreases from the 1st predicted measurement result to the last one in the PW. On the other hand, the tolerance of prediction accuracy also increases because UE is getting close to candidate cell as illustrated in our contribution [2].

Figure 3.1-1 Illustration of impact of prediction accuracy to measurement event prediction

For temporal domain case B, the PW is usually much smaller than temporal domain case B. Our view is that average L3 RSRP within PW should be sufficient.

Proposal 19: For temporal domain case A and case B, average L3 RSRP difference within PW is monitoring metric

For all above cases, the calculated L3 RSRP difference should be further averaged within configured monitoring duration.

Proposal 20: L3 RSRP difference should be further averaged within configured monitoring duration

Conclusion

Here are the observations and proposals:

Observation 1: For AI Mobility, the essential part of the applicability procedure is to check the applicability of inference parameters in partial configuration.

Proposal 1: When reporting applicability, UE can response applicable inference parameters

Observation 2: A full configuration after partial configuration will be most likely applicable

Proposal 2: Partial configuration and full configuration for different AI mobility functionality/scenario can be configured in the same RRCReconfiguration message

Proposal 3: When an inference configuration become inapplicable, UE shall report it to network via applicability update procedure. Autonomous release is not supported.

Proposal 4: For a full configuration, UE continues to perform the inference and reporting until the configuration is released after the UE reports it is inapplicable.

Proposal 5: UE starts/stops inference for AIML functionality corresponding to applicable inference configuration whenever UE starts/stops RRM measurement as in legacy.

Proposal 6: Upon receiving one or more partial inference configuration(s) via RRCReconfiguration message, UE shall maintain all the partial inference configuration(s) no matter the partial inference configuration is applicable or inapplicable until the network releases it explicitly

Proposal 7: For partial configuration UE reports initial applicability in RRCReconfigurationComplete message and UE updates applicability via UAI message.

Proposal 8: During mobility, target gNB can check applicability via handover command message i.e. RRCReconfiguration and UE can response with RRCReconfigurationComplete.

Observation 3: For RRM measurement prediction, Reporting L1 beam level measurement is not necessary

Proposal 9: For RRM measurement prediction, only L3 cell level measurement results are reported

Proposal 10: For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

Proposal 11: For temporal domain case A, one or more instances of measurement results in OW per one cell are reported in one measurementReport message

Proposal 12: For temporal domain case B, one measurement result per one cell or beam is reported in one measurementReport message.

Proposal 13: For temporal domain case B, one indication marking actual or predicted measurement result should be also reported along with the measurement result

Proposal 14: For frequency domain prediction and spatial domain prediction, one predicted measurement result per one cell is reported in one measurementReport message.

Proposal 15: For UE sided monitoring, UE can be configured to report monitoring metric result only once, or periodically or based on configured trigger

Proposal 16: UE can be configured to assess whether a full configuration is applicable or not based on monitoring metric results and network’s configuration. And it can be applied for all scenarios in the SID scope.

Proposal 17: The performance monitoring duration for UE sided model should be limited and configurable.

Observation 4: For inter-frequency prediction, a temporal measurement task for predicted frequency layer should be configured temporarily for monitoring purpose

Proposal 18: For UE side model, the metric is calculated per frequency for L3 cell level prediction i.e. intra-frequency cells should be calculated together.

Proposal 19: For temporal domain case A and case B, average L3 RSRP difference within PW is monitoring metric

Proposal 20: L3 RSRP difference should be further averaged within configured monitoring duration