3GPP TSG-RAN WG2 Meeting #131	R2-2505188

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.3.2.3

Source:	OPPO

Title:	Discussion on data collection for UE sided model

Document for:	Discussion, Decision

Introduction

In RAN2#129bis, we agreed that for data collection to take NR_AIML_air related procedures as baseline for further discussion.

In RAN2#130, RAN2 made limited progress and draw following conclusion:

5  Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

In this contribution, we would like to look at the different specification impact of RRM measurement prediction and measurement event prediction on data collection for AI mobility.

Discussion

RRM measurement prediction

Data collection procedure

For data collection of UE-side model, RAN2#129 to RAN2#130 on AI PHY reach some agreements:

- RAN2#129 Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

- RAN2#129bis Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

and

RAN2#130 Agreements on UE data collection request 3 The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs.

We understand that procedure wise it is similar between AI mobility and NR_AIML_air in terms of data collection since they are more use case agonistic. The spirit of listed agreements is as following:

1, Network control the whole data collection procedure including when to start and stop, what to configure, when to release configuration etc. on its own.

2, UE is allowed to request data collection via UAI message. Network can configure whether UE is allowed to request. And it is up to network’s implementation how to treat such request.

For the listed spirits, they can be adopted for AI mobility. So, we propose:

Proposal 1: For data collection of UE sided model, network control the whole data collection procedure including when to start and stop, what to configure, when to release configuration etc. on its own

Proposal 2: UE is allowed to request data collection via UAI. Network can configure whether UE is allowed to request. And it is up to network’s implementation how to treat such request.

In addition, RAN2 also agreed that associated ID can be optional configured as inference parameter. So naturally associated ID should be also part of the data collection configuration for training.

Proposal 3: Data collection related configuration(s) and associated ID(s) (if needed) can be included in training data collection configuration

For UE sided model, data collection can be triggered either by OTT server or server in core network. In this case, gNB has no idea when a UE can start or stop data collection. The situation is same for AI mobility use case. Furthermore, gNB also doesn’t know the targeted scenario of the data collection for AI mobility. One example is that only one frequency carrier need be configured for data collection for all intra-frequency prediction. But for inter-frequency prediction two frequency carriers need be configured. Without any UE assistant information, gNB can’t configure the data collection properly.

Proposal 4: UE is allowed to request start or stop data collection.

Proposal 5: Some assistant information from UE is necessary for gNB to configure data collection properly.

The data collection configuration for AI mobility will be very similar to current measurement configuration for RRM purpose. It contains at least following aspect:

- Frequency carrier to be measured

- What to log

- How to log

The 3rd aspect i.e. how to log supposes to agnostic among scenarios. It could be either periodically, or event triggered or both. It means it has nothing to do any assistant information from UE side.

As for frequency carrier to be measured, for both RRM measurement prediction and measurement event prediction, one frequency carrier need be configured for all intra-frequency scenarios and two frequency carriers for inter-frequency carrier.

Observation 1: For frequency domain prediction, network need know whether two corelated frequency carriers to be configured

As for frequency carrier to be measured, gNB can rely on UE’s radio capability as usual. However, some UE assistant information may be still needed. Considering models for FR1 and FR2 is most likely not generalized, so at least the information of frequency range is needed.

Observation 2: Considering model generalization between FR1 and FR2 is likely not feasible, frequency range information from UE can help network to decide on frequency carrier to be configured

One issue is whether UE should provide detail information like ARFCN of the frequency carrier? For intra-frequency case, dataset of same frequency is sufficient. It doesn’t really matter data is collected on which frequency carrier. If one OTT server collected data from different frequency carrier and OTT server want to train the model as such that generalization across frequency carriers may be possible. Still, it could be up to OTT server’s implementation. For inter-frequency prediction gNB should know which combination is valid in the sense inter-frequency prediction can be performed well because otherwise gNB will never configure UE to do inter-frequency prediction for that combination. And if UEs trained by same OTT server want to work well in the same operator, model trained with different frequency carrier combinations is necessary. In short how to configure detail carrier frequency can be left to gNB’s implementation.

Proposal 6: How to configure detail carrier frequency is up to gNB’s implementation.

As for 2nd item i.e. what to log, for RRM measurement prediction, what is agreed for network sided model can be taken as reference.

Agreements For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed)

Table 2.1-1

Among the logged content, cell identification and timing info (if any) has nothing to do with scenarios i.e. they should be applied commonly for all scenarios. What matters for scenarios L1 filtered beam level measurement, L3 beam level measurement result and L3 cell level measurement. Let’s take scenarios on L3 cell level measurement prediction as example, following tables show the relationship between supported scenarios and 1st &2nd aspects.

| Scenario and/or sub-cases | What to log |
| --- | --- |
| RRM sub-case 1 | L1 beam level measurement result only |
| RRM sub-case 2 of temporal domain case A | L3 cell level measurement result only |
| RRM sub-case 2 (apart from temporal domain case A) RRM sub-case 3 | L1 beam level measurement result L3 cell level measurement result |

Table 2.1-2

What matters is signalling overhead. If we take existing measurementReport message as reference, there is one cell level measurement result per reported cell and up to 96 measurement results per reported cell. So one simple way is that UE is always required to log L3 cell level measurement result due to its limited signaling overhead and the fact that it could be not so useful for RRM sub-case 1 only. But UE need indicate whether L1 beam level measurement need be collected. Note measuring and storing L1 beam level measurement result is something new for UE. In RRM framework, L1 beam level measurement result is intermediate result i.e. they will be dropped sooner or later during measurement procedure.

Observation 3: UE can provide indication whether L1 beam level measurement result need be logged.

Based on listed observations, we propose:

Proposal 7: Potential assistant information from UE for data collection could be:

| Potential assistant information | Applicable scenarios |
| --- | --- |
| Frequency range information | All scenarios (temporal domain, frequency domain, spatial domain) |
| Whether data collection is for frequency domain prediction | Frequency domain prediction |
| Whether L1 beam level measurement results need collected | RRM sub-case 2 of temporal domain case A |

Table 2.1-3

Data collection configuration

Data collection configuration for UE sided model could be quite similar for network sided model The agreement for network sided model from RAN2#130 are listed here:

Agreements For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

Table 2.3-1

In general, current RRM measurement framework should be baseline. And it is likely that there is not much difference regarding measurement object while it is possible some enhancement on reportConfig is needed. The enhanced reprotConfig will configure UE at least what to log. The relationship between data collection configuration and RRM measurement configuration can be figured out in work phase.

As discussed in previous section, at least following information should be logged:

- L1-filtered beam level measurement results

- L3 cell level measurement results

- Cell ID information

Proposal 8: For UE sided model, existing RRM measurement framework is baseline.

Proposal 9: At least following information should be logged:

- L1-filtered beam level measurement results

- L3 cell level measurement results

- Cell ID information

Report of collected data

For UE-side model, the data collection configuration, e.g., filtering parameters, is typically set by the gNB. However, the logged data report can be transferred to other network nodes (e.g., NWDAF) and may become transparent to the gNB. In such cases, the receiving node would require access to the data collection configuration to perform model training effectively.

Even when the logged data report is not transparent to the gNB, the time interval between the configuration setup and the actual reporting of logged data can be significant. The network may not retain the corresponding data collection configuration over such an extended period. Therefore, it would be beneficial for the UE to include relevant configuration details in its logged data report to ensure consistency and enable accurate data usage.

Proposal 10: For UE-side model, the logged data report includes related data collection configuration

Measurement event prediction

As for what to log, the difference between RRM measurement and measurement event prediction is quite clear. For measurement event prediction, obviously measurement event should be logged as label information. Network doesn’t know it unless UE indicate relevant information in request message. For indirect prediction method event type itself maybe not necessary since by nature it can be generalized among measurement events.

Proposal 11: For measurement event prediction, UE need indicate that data collection is for measurement event prediction.

Proposal 12: Measurement event should be logged if the target use case is measurement event prediction

Like RRM measurement prediction, measurement event related parameters should be part of the data collection configuration. And those parameters should be also reported along with logged data. Therefore, Proposal 10 is also applied for measurement event prediction.

Proposal 13: Proposal 10 is also applicable for Measurement event prediction

RRC state when collecting data

The SID explicitly states that the study focuses on RRC_CONNECTED mode, and all the studied RRM measurement prediction and measurement event prediction happen at RRC_CONNECTED state. The RRC_IDLE and RRC_INACTIVE states are fundamentally designed to minimize UE power consumption and reduce measurement by limiting UE activity and maintaining minimal connectivity. Enforcing data collection in these states or mandating transitions to RRC_CONNECTED sorely for data collection purpose would directly contradict their core objectives.

Proposal 14: UE-side data collection only applies for RRC_CONNECTED UE. A UE in RRC_IDLE or RRC_INACTIVE state should not be required to transition to RRC_CONNECTED solely for data collection purposes.

Conclusion