3GPP TSG-RAN WG2 Meeting #131		     R2-2505199

Bengaluru, IN: 25th August – 29th August 2025

Source:	Qualcomm Incorporated

Title:	Open Issues on LCM for UE-sided Models

Agenda Item:	8.1.2.2 LCM for UE-sided model for Beam Management use case

Document for:	Discussion and Decision

# Introduction & Background

This paper discusses open issues on LCM of UE-sided models for the beam management and CSI prediction use case, considering recent RAN1/RAN2 agreements.

# Discussion on LCM of UE-sided Models for Beam Management

## 2.1 Details on the Training Data Collection Configuration

In RAN2#129bis meeting [1], RAN2 agreed that the UE can indicate preferred configurations for data collection from the list of candidate configurations provided by the network.

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In RAN2#130 meeting [2], RAN2 further agreed on the following

The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

In the post POST130][037][AI PHY] UE candidate data collection (Xiaomi_Ericsson) [3], the majority of companies supported that UE should be able to indicate one or more preferred configuration.

The running CR for 38.331 defined an extension to UEAssistanceInformation containing dataCollectionPreference-r19, which is currently defined as shown below.

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionStartStop-r19                      	ENUMERATED {start, stop}                   OPTIONAL,

dataCollectionPreferredConfiguration-r19      FFS                                       			 OPTIONAL,

...

}

The DataCollectionPreference-r19 includes the fields dataCollectionStartStop-r19 to initiate or terminate a preference for a candidate for UE-side data collection and dataCollectionPreferredConfiguration-r19. The UE can indicate multiple preferred data collection configurations by the following options:

- UE indicates a list of preferred data collection configurations without a START / STOP indication. When the network receives the list of preferred data collection configuration from the UE, the network takes the following action

  - If a candidate data collection configuration is configured for data collection at the UE, but that configuration is not indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may release the data collection configuration.

  - If a candidate data collection configuration is configured for data collection at the UE and that configuration is indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may keep the data collection configuration at the UE. No action.

  - If a candidate data collection configuration is not configured for data collection at the UE and indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may choose to configure that configuration for data collection.

- The UE indicate a list of preferred data collection configurations. Within the list for each data collection configuration ID the UE indicates preferred action (START / STOP indication).

- The UE indicates a list of preferred data collection configuration with START indication and another list of preferred data collection configuration with STOP indication.

In all of the above options, the network can determine the data collection configuration the UE prefers to be configured with, maintained, or released. Therefore, to reduce signaling overhead, the UE indicates the preferred data collection configurations without a START / STOP indication.

Observation 1: The UE can indicate multiple preferred data collection configurations by the following options

- UE indicates a list of preferred data collection configurations without a START / STOP indication. When the network receives the list of preferred data collection configuration from the UE, the network takes the following action

  - If a candidate data collection configuration is configured for data collection at the UE, but that configuration is not indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may release the data collection configuration.

  - If a candidate data collection configuration is configured for data collection at the UE and that configuration is indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may keep the data collection configuration at the UE. No action.

  - If a candidate data collection configuration is not configured for data collection at the UE and indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may choose to configure that configuration for data collection.

- The UE indicate a list of preferred data collection configurations. Within the list for each data collection configuration ID the UE indicates preferred action (START / STOP indication).

- The UE indicates a list of preferred data collection configuration with START indication and another list of preferred data collection configuration with STOP indication.

Proposal 1: To reduce signaling overhead, the UE should indicate preferred data collection configurations without a START / STOP indication.

Proposal 2: Upon the reception of one or more preferred configurations from the UE, the network  takes the following action

- If a candidate data collection configuration is configured for data collection at the UE, but that configuration is not indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may release the data collection configuration.

- If a candidate data collection configuration is configured for data collection at the UE and that configuration is indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may keep the data collection configuration at the UE. No action.

- If a candidate data collection configuration is not configured for data collection at the UE and indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may choose to configure that configuration for data collection.

## 2.2 Further Discussion on Applicability Reporting

### 2.2.1 Applicability Reporting for Option B

In the RAN1#121 [4], to support option B for applicability reporting, the RAN1 agreed on the parameters that are fully contained within the existing CSI-ReportConfig.

Agreement For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused: - For both BM-Case 1 and BM-Case 2: - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19 - For BM-Case 2: - TimeGap-r19, nroftimeinstance-r19, - Note: this doesn’t imply the associated ID is always present

In option B of applicability reporting, once the UE indicates an applicable inference-related parameter set, the network provides the full inference configuration to the UE. However, note that after the UE is configured with full inference configuration, the applicability of the full inference configuration may change due to additional conditions on the UE side or model availability (if the UE needs to release the model due to a memory issue). The applicability of the inference-related parameter set and the full inference configuration (configured based on the applicability of the inference-related parameter set) would be correlated, as the inference configuration is derived from the inference-related parameter.

Observation 2: In option B, after the UE is configured with full inference configuration for applicable inference-related parameter sets

- The full inference configuration applicability may change due to UE-side additional conditions or model availability (if the UE needs to release the model due to a memory issue).

Observation 3: The applicability of the inference-related parameter set and the full inference configuration (configured based on the applicability of the inference-related parameter set) would be correlated, as the inference configuration is derived from the inference-related parameter.

There can exist the following two options for inference configuration based on the inference configuration (as depicted in Fig. 2.2.1-1 and Fig. 2.2.1-2)

- A single inference configuration is derived from the inference-related parameter set, or

- One or more inference configurations are derived from the inference-related parameter set.

Observation 4: There can exist the following two options for inference configuration based on the inference configuration (as depicted in Fig. 2.2.1-1 and Fig. 2.2.1-2)

- A single inference configuration is derived from the inference-related parameter set, or

- One or more inference configurations are derived from the inference-related parameter set.

For simplicity of signaling and processing at the UE, RAN2 should assume that a single inference configuration is configured based on the applicability of the inference-related parameter. Furthermore, after the UE is configured with an inference configuration based on the applicability of the inference-related parameter set, the UE only indicates the applicability of the inference configuration. The network considers both the inference configuration and the inference-related parameter set applicable if the UE indicates the inference configuration as applicable; otherwise, inapplicable. If the inference configuration is released, the UE starts indicating the applicability of the inference-related parameter set.

Proposal 3: For simplicity of signaling and processing at the UE, the network provides a single inference configuration for an applicable inference-related parameter set.

Proposal 4: After the UE is configured with an inference configuration based on the applicability of the inference-related parameter set, the UE only indicates the applicability of the inference configuration.

Proposal 5: If the inference configuration is released, the UE starts indicating the applicability of the inference-related parameter set.

Fig. 2.2.1-1: A single inference configuration based on the applicability of the inference-related parameter set.

Fig. 2.2.1-2: more than one inference configuration based on the applicability of a single inference-related parameter set.

### 2.2.2 Further Signaling Enhancement for Applicability Reporting

- In the current implementation of the applicability reporting in the running CR for 38.331

- The UE is expected to send full applicability/inapplicability information of the inference configuration configured by the previous RRCReconfiguration message (note that the UE has already provided the complete applicability/inapplicability information when configured by previous RRCReconfiguration(s) messages).

- Observation 5: In the current implementation of the applicability reporting in the running CR for 38.331

- The UE is expected to send full applicability/inapplicability information of the inference configuration configured by the previous RRCReconfiguration message (note that the UE has already provided the complete applicability/inapplicability information when configured by previous RRCReconfiguration(s) messages).

While full applicability/inapplicability information can be provided for the inference configuration provided in the last RRCReconfiguration, there is no need to provide the full applicability/inapplicability information provided by previous RRCReconfiguration(s) and for which applicability is already reported in the RRCReconfigurationComplete or UAI.

Proposal 6: The following applicability/inapplicability information is provided in the RRCReconfigurationComplete message

- Full applicability/inapplicability information for the inference configuration provided by the latest RRCReconfiguration message, and

- Change in applicability/inapplicability information for the inference configuration provided by the previous RRCReconfiguration message, and for which applicability is already reported in the RRCReconfigurationComplete or UAI.

## 2.3 Indication of the Cause of an Inference Configuration Deactivation

The network can deactivate the inference configuration for different reasons, for example, due to

- Poor inference performance,

- The inference result is not desired by the network anymore (for example, semi-persistent RS), or

- Network overloading.

At the UE side, if the UE inference performance is poor, the UE side would retrain the model to achieve the desired performance. Therefore, the network can indicate whether the deactivation signal is due to poor inference performance, which will be useful to the UE side for making retraining / fine-tuning decisions.

Observation 6: The network can deactivate the inference configuration for different reasons, for example, due to

- Poor inference performance,

- The inference result is not desired by the network anymore (for example, semi-persistent RS), or

- Network overloading.

Proposal 7: The network should indicate whether the deactivation of an inference configuration is due to poor inference performance, which will be useful to the UE side for making retraining / fine-tuning decisions.

## 2.4 UE Behavior when the Associated ID is not Provided

For UE to accurately determine the applicability of the inference-related information set and the inference configuration, the associated ID is highly desired. During Rel-19, RAN1 extensively discussed how UE can determine the applicability when the associated ID is not provided in the inference configuration or the inference-related parameter set. The following cases were discussed in RAN1,

- The associated IDs are indicated during the data collection, but not during the inference

  - The network implementation is unclear, as to why the network can indicate the associated ID during the data collection but not during inference

- The associated IDs are neither indicated during the data collection nor during the inference

  - The UE is not aware of the associated IDs at the cell

Therefore, in the absence of the associated ID, how the UE determines the applicability of an inference configuration or inference-related parameter set is left up to the UE implementation.

Observation 7: For UE to accurately determine the applicability of the inference-related information set and the inference configuration, the associated ID is highly desired.

Proposal 8: In the absence of the associated ID, it is left upto the UE implementaiton how to determine the applicability of an inference configuration or inference-related parameter set.

## 2.5 Time Duration for an AI Functionality to become Available for Inference

In RAN1#121 [4], RAN1 agreed on the following, considering different timelines

Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively. - Detailed values of d and d’ can be further discussed in UE feature.

Proposal 9: No further RAN2 discussion is required on the time duration for an AI functionality to become available for inference when the UE reports applicability.

## 2.6 Support for CA and NR-DC

During the normative work, specification work is not limited to PCell, but rather CA and NR-DC scenarios are also in the scope. Therefore, both CA and NR-DC should be considered for applicability reporting. Send LS to RAN3 to specify any standardization required for supporting NR-DC.

Proposal 10: CA and NR-DC are considered for the specification work. Send LS to RAN3 to specify any standardization work required for supporting NR-DC.

# Discussion on LCM of UE-sided Models for CSI Prediction

## 3.1	Applicability Reporting for CSI Prediction Use Case

For the CSI prediction use case, there was no discussion on applicability reporting for option B. Therefore, for CSI prediction applicability reporting, only option A is supported.

Proposal 11: For CSI prediction applicability reporting, only option A is supported.

# Conclusion

LCM for Beam Management

Observation 1: The UE can indicate multiple preferred data collection configurations by the following options

- UE indicates a list of preferred data collection configurations without a START / STOP indication. When the network receives the list of preferred data collection configuration from the UE, the network takes the following action

  - If a candidate data collection configuration is configured for data collection at the UE, but that configuration is not indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may release the data collection configuration.

  - If a candidate data collection configuration is configured for data collection at the UE and that configuration is indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may keep the data collection configuration at the UE. No action.

  - If a candidate data collection configuration is not configured for data collection at the UE and indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may choose to configure that configuration for data collection.

- The UE indicate a list of preferred data collection configurations. Within the list for each data collection configuration ID the UE indicates preferred action (START / STOP indication).

- The UE indicates a list of preferred data collection configuration with START indication and another list of preferred data collection configuration with STOP indication.

Proposal 1: To reduce signaling overhead, the UE should indicate preferred data collection configurations without a START / STOP indication.

Proposal 2: Upon the reception of one or more preferred configurations from the UE, the network  takes the following action

- If a candidate data collection configuration is configured for data collection at the UE, but that configuration is not indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may release the data collection configuration.

- If a candidate data collection configuration is configured for data collection at the UE and that configuration is indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may keep the data collection configuration at the UE. No action.

- If a candidate data collection configuration is not configured for data collection at the UE and indicated as the preferred configuration in the last indicated list of preferred data collection configuration, the network may choose to configure that configuration for data collection.

Observation 2: In option B, after the UE is configured with full inference configuration for applicable inference-related parameter sets

- The full inference configuration applicability may change due to UE-side additional conditions or model availability (if the UE needs to release the model due to a memory issue).

Observation 3: The applicability of the inference-related parameter set and the full inference configuration (configured based on the applicability of the inference-related parameter set) would be correlated, as the inference configuration is derived from the inference-related parameter.

Observation 4: There can exist the following two options for inference configuration based on the inference configuration (as depicted in Fig. 2.2.1-1 and Fig. 2.2.1-2)

- A single inference configuration is derived from the inference-related parameter set, or

- One or more inference configurations are derived from the inference-related parameter set.

Proposal 3: For simplicity of signaling and processing at the UE, the network provides a single inference configuration for an applicable inference-related parameter set.

Proposal 4: After the UE is configured with an inference configuration based on the applicability of the inference-related parameter set, the UE only indicates the applicability of the inference configuration.

- Observation 5: In the current implementation of the applicability reporting in the running CR for 38.331

- The UE is expected to send full applicability/inapplicability information of the inference configuration configured by the previous RRCReconfiguration message (note that the UE has already provided the complete applicability/inapplicability information when configured by previous RRCReconfiguration(s) messages).

Proposal 6: The following applicability/inapplicability information is provided in the RRCReconfigurationComplete message

- Full applicability/inapplicability information for the inference configuration provided by the latest RRCReconfiguration message, and

- Change in applicability/inapplicability information for the inference configuration provided by the previous RRCReconfiguration message, and for which applicability is already reported in the RRCReconfigurationComplete or UAI.

Observation 6: The network can deactivate the inference configuration for different reasons, for example, due to

- Poor inference performance,

- The inference result is not desired by the network anymore (for example, semi-persistent RS), or

- Network overloading.

Proposal 7: The network can indicate whether the deactivation signal is due to poor inference performance, which will be useful to the UE side for making retraining / fine-tuning decisions.

Observation 7: For UE to accurately determine the applicability of the inference-related information set and the inference configuration, the associated ID is highly desired.

Proposal 8: In the absence of the associated ID, it is left upto the UE implementaiton how to determine the applicability of an inference configuration or inference-related parameter set.

Proposal 9: No further RAN2 discussion is required on the time duration for an AI functionality to become available for inference when the UE reports applicability.

Proposal 10: CA and NR-DC are considered for the specification work. Send LS to RAN3 to specify any standardization work required for supporting NR-DC.

LCM for CSI Prediction

Proposal 11: For CSI prediction applicability reporting, only option A is supported.

# References

- RAN2#129b meeting report

- RAN2#130 meeting report

- POST130][037][AI PHY] UE candidate data collection (Xiaomi_Ericsson)

- RAN1#121 meeting report