3GPP TSG-RAN WG2 Meeting #131	R2-2505227

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.8.4

Source: 	CATT

Title: 	Further discussion on support of broadcast service in NR NTN

Document for: 	Discussion and Decision

Introduction

In RAN2#129, we sent an LS [1] to SA4 asking whether the inclusion of the ISA in the Service Announcement can be supported within the Rel-19 timeframe.

# 1. Overall Description: RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell an MBS broadcast service is applicable. This area is called the “intended service area” (ISA) whose geographical definition is provided to the UE via SIB and is associated with an identifier (ISA ID). The attached CR includes the changes introduced by SA2 to adopt the ISA for NTN access. # RAN2 would like to ask whether including the ISA in the Service Announcement would impact in SA4 specifications and would kindly request SA4 to confirm this can be supported within the Rel-19 timeframe. # 2. Actions: # To SA WG 4: # RAN2 kindly request SA4 to confirm whether the inclusion of the ISA in the Service Announcement can be supported within the Rel-19 timeframe.

Based on SA4’s reply LS [2] and CRs achieved in SA4#133-e [3][4], this contribution discusses potential RAN2 issue on supporting the service announcement way of providing service area for MBS broadcast service.

Discussion

SA4 replied our LS in [2], indicating that:

# 1. Overall Description: SA4 thanks RAN2 for bringing up the topic of “MBS via NTN”. SA4 is still studying the realization for including an Intended Service Area into Service announcement, leveraging the RAN2 Work Item code NR_NTN_Ph3-Core. SA4 has agreed to extend the Service Announcement and add a targeted service area in form of a list of polygon or circle shapes. However, SA4 is still discussing the technical realization and possibly a generalization, so that the target service area can be extended to represents the intended service area in case of NR NTN. # SA4 confirms, that inclusion of the geographical information, i.e. in form of list of polygon or circle shapes in the Service Announcement can be supported within the Rel-19 timeframe. # 2. Actions: # To RAN WG2: # Please take the above provided information into account.

And in SA4#133-e, SA4 agreed two CRs on their specs to support polygon and circle format service area in the service announcement. An example is excerpted in the following box.

[Excerpt from S4-251482] ### 5.2.9 Availability Information data type The AvailabilityInformation data type provides additional information pertaining to the availability of the MBS Distribution Session within the 5G Network: - The targetServiceAreas property declares the one or more service areas in which the MBS Session corresponding to this MBS Distribution Session is currently available. Each target service area is expressed as one of the following: - A list of NR Cell Identifiers. - A list of Tracking Area Identifiers. - An area specified as a list of polygon or circle shapes. - In the case of a broadcast MBS Session corresponding to this MBS Distribution Session: - The mbsFSAId property identifies a preconfigured area within which, and in proximity to, the cell(s) announce the MBS Frequency Selection Area (FSA) ID and its associated frequency. - The nrRedCapUEInfo property indicates which classes of UE the MBS Distribution Session is suitable for consumption by. NOTE 1: This is used to guide frequency selection by the UE for a broadcast MBS Session. - In the case of a broadcast MBS Session corresponding to this MBS Distribution Session, the nrParameters property indicates the one or more radio frequencies in the NG-RAN downlink on which the MBS Session is transmitted in the service area(s) identified by the serviceArea property. NOTE 2: The radio frequencies may be obtained by interrogating the OAM using the value of the mbsFSAId property as a lookup key. Table 5.2.9-1 provides the detailed semantics for the AvailabilityInformation data type. - Table 5.2.9-1: Semantics of AvailabilityInformation data type Property name; Type; P; Cardinality; Description. mbsService‌Areas; array(Mbs‌Service‌Area); O; 1..N; The -This property is deprecated. The property shall be absent in this release.. target‌Service‌Areas; array(Target‌Service‌Area); O; 1..N; The Target service areas of this MBS Distribution Session, as defined in table 4.5.8-1 of TS 26.502 [6]..

Note that SA4 introduces the service area in format of circle and polygon rather than service IDs (as proposed by company in last RAN2 meetings). Providing the circle or polygon format service area via service announcement can work in parallel with the “SIB+MCCH” approach as defined by RAN2. Only minimal adaptations in TS 38.331 are required, as shown in the box below.

#### 5.9.2.2 Initiation A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s). NOTE 1: It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification. NOTE 2: It is up to UE implementation to use the cell/tracking area/polygon/circle list in the USD or the polygon/circle list in SIBXX to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. [Next change] #### 5.9.3.2 Initiation The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in, upon entering a polygon/circle area providing an MBS broadcast service the UE is interested in. The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated e.g. upon stop of the MBS session, upon leaving the cell broadcasting the MBS service the UE is interested in, upon losing interest in the MBS service, when capability limitations start inhibiting reception of the concerned service, upon leaving a polygon/circle area providing an MBS broadcast service the UE is interested in.

Observation 1: SA4 supports service area types of polygon and circle in service announcement, offering a standalone/parallel alternative to the Rel-19 RAN2-defined intended service area solution. Only minimal adaptations in TS 38.331 are required.

However, if both the service announcement and “SIB+MCCH” approaches on provide service area are supported in the NTN system, we need to discuss whether to specify UE behaviour when these two approaches provide different service area for the same MBS broadcast service. This issue may occur in the following scenarios:

- The circles in service announcement use the PointUncertaintyCircle format, whereas those in the SIBXX are expressed as reference location with radius. Because these two formats have different quantisation steps, the resulting service areas may not be perfectly aligned.

- After the UE acquires service area via service announcement in RRC_CONNECTED state, it may enter RRC_IDLE state and try to receive the MBS broadcast service using the received service area before ahead. In an NTN cell, the UE could also acquire the service area by reading the SIB. When the service area of an MBS broadcast service is changed (after the UE enters RRC_IDLE state), the UE may acquire the updated service area by reading the SIB and it is different from the service area stored in the USD.

Company may assume that network implementation should ensure alignment between the service areas provided by the two approaches. However, it is hard to guarantee that all the time. Considering we already specified the SIB approach for NTN cells, which could provide more granular and up-to-date service area to UEs in any RRC states, it is proposed that if the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD.

Proposal 1: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD.

Conclusion

In this contribution, support of broadcast service via NR NTN is discussed with the observations and proposals listed as follows:

Observation 1: SA4 supports service area types of polygon and circle in service announcement, offering a standalone/parallel alternative to the Rel-19 RAN2-defined intended service area solution. Only minimal adaptations in TS 38.331 are required.

Proposal 1: If the service area information is broadcast in an NTN cell, the UE ignores the service area information in USD.

Reference

- R2-2501583	LS on Inclusion of NTN intended service area in the Service Announcement	Ericsson	LS out	To:SA4, Cc:SA2	Rel-19	NR_NTN_Ph3-Core.

- S4-251099	Reply on Inclusion of NTN intended service area in the Service Announcement.

- S4-251354	Aligning Stage 2 Session Announcement with Stage 3 on target service area.

- S4-251482	Target Service Area Data Type in Service Announcement.

- 3GPP TS 38.331: "NR: Radio Resource Control (RRC) protocol specification".