3GPP TSG RAN WG2 Meeting #131                                                            R2-2505260

# Bangalore, India Aug 25th – 29th, 2025

Agenda item:	8.7.4

Source:	Ofinno

Title:	Scheduling Enhancements for XR

Document for:	Discussion and decision

# Introduction

In this contribution, we would like to discuss the following three remaining open issues about DSR:

- [MAC-1] Discuss whether a pending DSR should be cancelled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

- [MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

- (UE capability-01) Discuss whether a UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18.

# Discussion

## DSR Cancellation for Multiple Entry DSR

TS 38.323 [1] Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

TS 38.321 [2] A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain any DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

In Rel-18, TS 38.323 [1] defines a “delay-critical PDCP SDU.” TS 38.321 [2] then specifies that a PDCP SDU is associated with a DSR if it is a delay-critical PDCP SDU associated with the LCH that triggered the DSR, and it has not been transmitted in any MAC PDU. This association is mainly used for determining whether a pending DSR can be cancelled. The MAC entity cancels a pending DSR under one of these three conditions:

- (1) All PDCP SDUs associated with the DSR have been discarded.

- (2) A MAC PDU that is transmitted includes a DSR MAC CE that contains the delay information for all PDCP SDUs associated with the DSR.

- (3) A MAC PDU that is transmitted includes all PDCP SDUs associated with the DSR.

The rationale is that the DSR procedure can be cancelled when the DSR procedure’s purpose has been achieved, i.e., if (1) all delay-critical PDCP SDUs are discarded, (2) the delay information of all delay-critical PDCP SDUs is reported via the DSR MAC CE, or (3) all delay-critical PDCP SDUs have been transmitted.

TS 38.323 [1] Delay-reporting PDCP SDU: if pdu-SetDiscard is not configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU for which the remaining time till discardTimer expiry is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1). If pdu-SetDiscard is configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU belonging to a PDU Set of which the PDU Set remaining time is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1).

In Rel-19, TS 38.321 [2] introduces the Multiple Entry DSR MAC CE. This enhances the Rel-18 Single Entry DSR MAC CE by allowing the reporting of multiple delay statuses within each remaining time range, based on multiple reporting thresholds. This provides the network with more granular delay information. However, since delay-critical PDCP SDUs only satisfy the remaining time threshold condition for triggering a DSR and cannot reflect all the PDCP SDUs for multiple remaining time thresholds for reporting DSR via a Multiple Entry DSR MAC CE, TS 38.323 further defines “delay-reporting PDCP SDU.” The delay-reporting PDCP SDU is a PDCP SDU associated with different remaining time ranges, corresponding to the multiple remaining time thresholds used for reporting DSR via the Multiple Entry DSR MAC CE. In essence, delay-reporting PDCP SDUs are those whose delay status satisfies the thresholds and should be reported in the Multiple Entry DSR MAC CE.

However, the current MAC entity does not consider delay-reporting PDCP SDUs to be associated with a DSR. Consequently, even when at least one LCG is configured with dsr-ReportingThresList (i.e., enabling the generation of Multiple Entry DSR MAC CE), the current MAC entity cancels the pending DSR based solely on delay-critical PDCP SDUs, ignoring delay-reporting PDCP SDUs. This can lead to premature DSR cancellation, e.g., even if delay-reporting PDCP SDUs have not been discarded/transmitted, or their delay status has not been reported. This undermines the benefits of the Multiple Entry DSR MAC CE. Furthermore, the enablement of the Multiple Entry DSR MAC CE (i.e., when at least one LCG has dsr-ReportingThresList configured) indicates the network’s preference for receiving finer-grained delay status information. Therefore, the UE should not arbitrarily cancel the DSR if delay-reporting PDCP SDUs are still waiting for transmission or reporting the delay status. We believe the cancellation conditions should be based on delay-reporting PDCP SDUs if the UE is configured to report the Multiple Entry DSR MAC CE. This ensures that the enhanced DSR capability is fully utilized.

[MAC-1] For the cancellation of a pending DSR, if at least one LCG is configured with dsr-ReportingThresList (i.e., Multiple Entry DSR MAC CE is enabled), a delay-reporting PDCP SDU is considered to be associated with the DSR (if it is associated with the LCH which triggered the DSR and it has not been transmitted in any MAC PDU).

## BSR Cancellation based on DSR

Figure 1: BSR MAC CE vs. DSR MAC CE

As illustrated in Figure 1, the BSR MAC CE provides buffer size information for one or more LCGs, while the DSR MAC CE provides both buffer size information and remaining time information for one or more LCGs. Furthermore, the DSR MAC CE can provide finer granularity in buffer size value through a new buffer size table defined in Rel-18. The Multiple Entry DSR MAC CE in Rel-19 can even provide multiple buffer size values among different remaining time ranges.

Basically, the information (i.e., buffer size for LCG(s)) provided by the BSR MAC CE can also be conveyed by the DSR MAC CE. The DSR MAC CE, however, offers additional information and potentially greater granularity. This leads to an issue: whether the UE can cancel all triggered BSRs if the buffer size information that would have been reported in the BSR MAC CE is included in the DSR MAC CE.

TS 38.321 [1] All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a Long, Refined Long, Extended Long, Short, or Extended Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly.

To address this, we should consider the advantages and disadvantages of these two options:

- Option 1: BSR cancellation based on DSR

  - UE cancels all the pending BSRs if a DSR MAC CE contains the relevant buffer size information.

- Option 2: Cancellation of BSR is decoupled from DSR

  - UE cancels the pending BSR and the DSR independently

Option 1 (BSR cancellation based on DSR) presents an advantage in UL resource efficiency by eliminating the transmission of a redundant BSR MAC CE containing less precise buffer status, but the disadvantage is to introduce specification impact and may need further discussion on some potential issues. For example, it needs the discussion on whether the buffer size information in the DSR MAC CE is up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly. This is for the UE to determine whether all the triggered BSRs (that are triggered prior to MAC PDU assembly) can be cancelled. It is possible that some BSRs are triggered after the MAC PDU assembly of the DSR MAC CE. It is also possible that some BSRs triggered prior to the MAC PDU assembly of the DSR MAC CE, but the buffer status included in the DSR MAC CE does not reflect the last event that triggered a BSR prior to the MAC PDU assembly due to UE processing timeline. Moreover, it’s complicated for the UE to estimate what is the amount of data that would have been reported in the BSR MAC CE but is instead reported in the DSR MAC CE at the end.

Option 2 (Cancellation of BSR is decoupled from DSR) might incur the overhead of transmitting an additional BSR MAC CE, but this overhead is considered minimal given that there is only one additional BSR MAC CE that will be transmitted. Due to the constraint that a MAC PDU shall contain at most one BSR MAC CE, even when multiple events have triggered a BSR. More importantly, this option avoids any (1) specification changes, (2) further discussions on potential issues, and (3) UE complexity. Given this issue was already existing in Rel-18 without any enhancement and the overhead is minimum, we slightly prefer to not have any enhancement on this.

Cancellation of pending BSR based on DSR introduces (1) specification changes, (2) further discussions on potential issues, and (3) UE complexity.

[MAC-2] Cancellation of pending BSR is decoupled from DSR (i.e., no specification impact)

## UE Capability of Multiple Entry DSR and Single Entry DSR

TS 38.306 [3] delayStatusReport-r18 Indicates whether the UE supports the delay status report of the buffered data as specified in TS 38.321 [8], TS 38.331 [9], TS 38.323 [16] and TS 38.322 [36]. multipleEntryDelayStatusReport-r19 Indicates whether the UE supports the delay status report of the buffered data using multiple reporting thresholds, as specified in TS 38.321 [8], TS 38.331 [9], TS 38.323 [16] and TS 38.322 [36].

RAN2#129bis meeting discussed whether a UE supporting for Rel-19 Multiple Entry DSR should also be mandated to indicate support for Rel-18 Single Entry DSR. From our perspective, the UE capability for Rel-18 Single Entry DSR should not be a pre-requisite of the UE capability for Rel-19 Multiple Entry DSR. A UE should be able to support Multiple Entry DSR independently, without also supporting Single Entry DSR. This allows UE vendors the flexibility to choose whether to implement Single Entry DSR, Multiple Entry DSR, or both, based on their specific design considerations.

On the other hand, from a network vendor perspective, even if a UE supports only Multiple Entry DSR, the network can effectively emulate Single Entry DSR through a specific configuration. This can be achieved by configuring only one reporting threshold within the dsr-ReportingThresList for each LCG and by setting this threshold value to be identical to the triggering threshold. This configuration replicates the same outcome of Single Entry DSR.

Therefore, imposing a limitation on UE capability such that support for Rel-18 Single Entry DSR is required for supporting Rel-19 Multiple Entry DSR is deemed unnecessary and overly restrictive.

From network implementation, even if a UE only supports Multiple Entry DSR, it can emulate Single Entry DSR functionality by configuring only one reporting threshold in the dsr-ReportingThresList for each LCG, with that threshold value being equivalent to the triggering threshold.

(UE capability-01) UE Capability of Rel-19 Multiple Entry DSR is decoupled from UE Capability of Rel-18 Single Entry DSR

# Conclusion

Based on the analysis above, we have made the following proposals:

Proposal 1	[MAC-1] For the cancellation of a pending DSR, if at least one LCG is configured with dsr-ReportingThresList (i.e., Multiple Entry DSR MAC CE is enabled), a delay-reporting PDCP SDU is considered to be associated with the DSR (if it is associated with the LCH which triggered the DSR and it has not been transmitted in any MAC PDU).

Proposal 2	[MAC-2] Cancellation of pending BSR is decoupled from DSR (i.e., no specification impact)

Proposal 3	(UE capability-01) UE Capability of Rel-19 Multiple Entry DSR is decoupled from UE Capability of Rel-18 Single Entry DSR

# Reference

[POST130][507][XR] PDCP running CR and open issues (LGE)

[POST130][505][XR] MAC running CR and open issues (Qualcomm)

[POST130][509][XR] UE capabilities CRs (Xiaomi)