3GPP TSG-RAN WG2 Meeting #131	R2-2505344

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.7.5

Source:	OPPO

Title:	Discussion on RLC re-transmission related enhancements

Document for:	Discussion, Agreement

Introduction

For the RLC retransmission related enhancements, RAN2 has made some conclusions for both timely RLC retransmission and avoiding unnecessary retransmission. This paper will further discuss the left issues for each aspect.

Discussion

Open issue RLC-10: Whether/How to introduce any limitations for the configuration of Tx and Rx side discard.

How to configure the parameter is fully up to the network implementation:

- For UL, when UE as the Tx side, for a specific RLC bearer, if stopReTxDiscardedSDU is configured, network as the Rx side can maintain the t-RxDiscard to determine whether a packet should be discarded, but the t-RxDiscard doesn’t need to be configured to the UE side.

- For DL, when the UE as the Rx side, for a specific RLC bearer, if t-RxDiscard is configured, it is also network implementation on whether to apply the Tx side discard, no need to indicate such information to the UE side.

If Tx/Rx discard is configured to the UE, whether the peer-side operation is applied is fully up to the network implementation.

(RLC-10) RAN2 not pursue the limitations for the configuration of Tx and Rx side discard.

Open issue RLC-11: How to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

In RAN2 #130 meeting, the following working assumption was made

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

As discussed in RAN2 # 128 meeting, there will be no window stalling issue/the Rx side SR can solve the issue, RAN2 should keep aligned on this point.

RAN2 has discussed the window stalling issue in RAN2 #128 meeting, and concluded there is no problem.

(RLC-11) RAN2 to confirm the Working assumption “(RLC-11) No need to address window stalling issue with polling retransmission.”.

Open issue RLC-12: how to handle PDCP SN gap report handling during UE mobility

The concerned case is when PDCP layer generates a SN Gap report, but the report is not transmitted successfully by RLC, e.g., due to RLC reset during mobility. This issue was discussed for R18, but no solution was agreed since it is not considered as a critical issue.

From RAN2 point of view, for Rel-18 alternative 3 should not be supported.   RAN2 doesn’t see a big problem to solve for Rel-18.

RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements.  If it is a problem further discussions are needed on the alternatives.

In R19 context, this issue is exactly the same as R18, which means if not critical for R18, it is not critical for R19 as well.

The issue is same in R18 and R19 context, which means it is not a critical issue for R19 as well.

Besides, not only the RLC reset due to mobility but the RLC discard will cause PDCP SN gap report missing. RAN2 has discussed whether the discard of PDCP control PDU is a problem, and the conclusion is all the control PDUs are not critical and can be discarded.

Special handling to avoid PDCP control PDU discard is not needed.

During the RLC discard enhancement discussion, R2 concluded no special handling to avoid PDCP control PDU discard is needed.

Therefore, RAN2 should follow the previous agreement, which means no special handling for this issue as well.

(RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility.

Open issue RLC-14: When indicated by upper layer, whether the Tx side of an AM RLC entity shall discard the RLC SDU or the RLC SDU segment(s) has been submitted to lower layers.

In the current CR, the transmitting side will cease retransmission if stopReTxDiscardedSDU is configured and the upper layer indicates that a specific RLC SDU should be discarded. However, it remains unclear whether the AM RLC entity should discard the corresponding RLC SDU or its segments if they have already been submitted to lower layers. The reason of not discarding the RLC SDU or SDU segments is to avoid RLC SN gap, but since the RLC SDU indicated as discarded will never be retransmitted, the SN gap is inevitable.

An RLC SDU marked as discarded by the upper layer will not be transmitted by the transmitting side of the AM RLC entity, resulting in an unavoidable gap in the RLC sequence numbers.

(RLC-14) The transmitting side of the AM RLC entity can discard the RLC SDU or SDU segment(s) that have been submitted to lower layers if stopReTxDiscardedSDU is configured and discard indication is received from upper layer.

Open issue UE capability-04: The pre-requisite of Tx side stopping transmission of discarded SDU

In RAN2 #130, the following agreements are made on UE capability

- Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

Tx-side and Rx-side capability are used for different traffic directions and independent as Tx side behavior and Rx side behavior. I.e.,

- For UL bearers, when UE acts as Tx side, it supports the Tx-side capability. And network act as the Rx side based on its implementation to decide whether to proceed the Rx side discarding operation (i.e., discard timer-based retransmission discarding).

- For DL bearers, when UE act as Rx side, if it supports Rx-side capability, and such operation is enabled by the network. It can discard retransmissions based on the Rx timer. Whether network as the Tx side proceed the Tx-side discard is up to network implementation.

Therefore, there is no need to couple the Tx/Rx side UE capability, as they are for different direction bearers.

For unnecessary retransmission avoidance, Tx side and Rx side packet discarding can operate independently for different traffic directions.

(UE capability-04) rxRLC-Discard-r19 is not the pre-requisite of txRLC-StopReTxDiscarded, i.e., UE supports Tx side stopping transmission of discarded SDU doesn’t need to support Rx-side discard operation.

Conclusion

We have the following observations:

Observation 1	If Tx/Rx discard is configured to the UE, whether the peer-side operation is applied is fully up to the network implementation.

Observation 2	RAN2 has discussed the window stalling issue in RAN2 #128 meeting, and concluded there is no problem.

Observation 3	The issue is same in R18 and R19 context, which means it is not a critical issue for R19 as well.

Observation 4	During the RLC discard enhancement discussion, R2 concluded no special handling to avoid PDCP control PDU discard is needed.

Observation 5	An RLC SDU marked as discarded by the upper layer will not be transmitted by the transmitting side of the AM RLC entity, resulting in an unavoidable gap in the RLC sequence numbers.

Observation 6	For unnecessary retransmission avoidance, Tx side and Rx side packet discarding can operate independently for different traffic directions.

We have the following proposals:

Proposal 1	(RLC-10) RAN2 not pursue the limitations for the configuration of Tx and Rx side discard.

Proposal 2	(RLC-11) RAN2 to confirm the Working assumption “(RLC-11) No need to address window stalling issue with polling retransmission.”.

Proposal 3	(RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility.

Proposal 4	(RLC-14) The transmitting side of the AM RLC entity can discard the RLC SDU or SDU segment(s) that have been submitted to lower layers if stopReTxDiscardedSDU is configured and discard indication is received from upper layer.

Proposal 5	(UE capability-04) rxRLC-Discard-r19 is not the pre-requisite of txRLC-StopReTxDiscarded, i.e., UE supports Tx side stopping transmission of discarded SDU doesn’t need to support Rx-side discard operation.