3GPP TSG-RAN WG2 Meeting #131	   R2-2505372

Bengaluru, India, Aug 25th – 29th, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	Scheduling enhancements for XR

Agenda item:	8.7.4

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for RAN2 to specify enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

In this contribution we will further investigate the remaining issues related to UL scheduling enhancements per this objective.

# Discussion

About open issue of RLC-13: Whether the delay-reporting or non-delay-reporting information is visible or not to RLC.

Considering that the DSR is only performed in uplink, and PDCP entity and RLC entity are both on the UE, the interaction between RLC and PDCP entities can be left to UE implementation.

In fact, same issue exists for LCP, e.g. in TS38.321 running CR, there are the following description.

##### 5.4.3.1.3 Allocation of resources Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover. The MAC entity shall, when a new transmission is performed: 1> if a logical channel is configured with priorityAdjustmentThreshold and has a PDCP SDU available for this transmission: 2> if the PDCP entity associated with this logical channel is configured with pdu-SetDiscard, and the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold; or 2> if the PDCP entity associated with this logical channel is not configured with pdu-SetDiscard, and the remaining time of discardTimer of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold: 3> consider this PDCP SDU being priority adjustable. ...

LCP is performed in UE MAC entity, but how UE MAC entity decides that “the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold” has not been specified, we understand this is also based on UE implementation.

Observation 1: Both DSR and LCP are triggered in UE, hence, the interaction between RLC and PDCP entities can be left to UE implementation.

Proposal 1a(RLC-13): The delay-reporting or non-delay-reporting information is visible to RLC.

Proposal 1b(RLC-13): For both DSR and LCP procedure, the interaction between various protocol layers can be left to UE implementation.

About open issue of [MAC-1]: Discuss whether a pending DSR should be cancelled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

Considering that the delay critical SDU is defined in Rel-18 for delay status reporting with Single Entry DSR MAC CE, and delay reporting SDU is defined in Rel-19 for delay status reporting with Multiple Entry DSR MAC CE, the triggering threshold and reporting threshold of delay status reporting with Single Entry DSR MAC CE is independent from that with Multiple Entry DSR MAC CE, the cancellation of the delay status reporting with Single Entry DSR MAC CE should also be independent from that with Multiple Entry DSR MAC CE .

Proposal 2a(MAC-1): the cancellation of the delay status reporting with Single Entry DSR MAC CE should be independent from that with Multiple Entry DSR MAC CE.

Proposal 2b(MAC-1): a pending DSR with Multiple Entry DSR MAC CE should not be cancelled based on delay critical SDUs info(e.g. there is no delay critical SDUs or all delay critical SDUs have been reported).

About open issue of [MAC-2]: Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

Considering the BSR triggering condition is different from that of the DSR, and it has been specified in TS 38.321 that “All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader.”, same as the rel-18 mechanism, BSR cancellation should be independent from the DSR reporting.

Proposal 3 (MAC-2): BSR cancellation is independent from the DSR reporting, pending BSRs may not be cancelled by “all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE”.

About open issue of [MAC-6]: Whether any spec change is needed to capture the agreement on DSR cancellation in the DC case.

In RAN2#130 meeting, it has been agreed that

- (MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR. - (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. - (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

It means that one MAC entity in the UE can see PDCP SDU information associated with DSR in the other MAC entity.

In the TS 38.321 running CR[4], it has already captured that:

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

We think this description can cover the DC case, but the intention of the “when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR” description is that all the PDCP SDU associated with the DSR have been transmitted after this MAC PDU transmission, not that all the PDCP SDU associated with the DSR are included in this MAC PDU. So, we think it’s more accurate to say “when a MAC PDU is transmitted and all the PDCP SDUs associated with the DSR have been transmitted after this MAC PDU transmission”

Proposal 4(MAC-6) change the DSR cancellation description as following:

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and all the PDCP SDU associated with the DSR have been transmitted after this MAC PDU transmissionthis MAC PDU includes all the PDCP SDUs associated with the DSR.

About open issue of PDCP-1 (Editorial): Whether the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” needs to be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU

Considering that the definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU is used for delay-reporting PDCP data volume calculation, the SDU that will not be included in delay-reporting PDCP data volume should be excluded from the delay-reporting PDCP SDU and non-delay-reporting PDCP SDU. We think that “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” is used to describe the SDUs that are not be calculated in delay-reporting PDCP data volume, it’s better to be clarified in the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

Observation 2: The definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU is used for delay-reporting PDCP data volume calculation.

Proposal 5 (PDCP-1): The text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” should be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

# Conclusion and proposals

The following observations/proposals are made:

Observation 1: Both DSR and LCP are triggered in UE, hence, the interaction between RLC and PDCP entities can be left to UE implementation.

Proposal 1a(RLC-13): The delay-reporting or non-delay-reporting information is visible to RLC.

Proposal 1b(RLC-13): For both DSR and LCP procedure, the interaction between various protocol layers can be left to UE implementation.

Proposal 2a(MAC-1): the cancellation of the delay status reporting with Single Entry DSR MAC CE should be independent from that with Multiple Entry DSR MAC CE.

Proposal 2b(MAC-1): a pending DSR with Multiple Entry DSR MAC CE should not be cancelled based on delay critical SDUs info(e.g. there is no delay critical SDUs or all delay critical SDUs have been reported).

Proposal 3 (MAC-2): BSR cancellation is independent from the DSR reporting, pending BSRs may not be cancelled by “all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE”.

Proposal 4(MAC-6) change the DSR cancellation description as following:

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and all the PDCP SDUs associated with the DSR have been transmitted after this MAC PDU transmissionthis MAC PDU includes all the PDCP SDUs associated with the DSR.

Observation 2: The definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU is used for delay-reporting PDCP data volume calculation.

Proposal 5 (PDCP-1): The text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” should be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- Draft_RAN2_129bis_Meeting_Report_v2

- 3GPP TS 38.321 V18.4.0 (2024-12), NR;Medium Access Control (MAC) protocol specification

- R2-250xxxx R19 XR MAC running CR_v24_Rapp