3GPP TSG-RAN WG2 Meeting #131	   R2-2505373

Bengaluru, India, Aug 25th – 29th, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	RLC enhancements for XR

Agenda item:	8.7.5

Document for: Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for RAN2 to specify RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. .

In this contribution, we will further discuss the RLC re-transmission related enhancements and give our proposals.

# Discussion

About open issue of PDCP-2: Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers.

Based on the current RLC running CR for XR[3], when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if stopReTxDiscardedSDU is configured and the RLC SDU or a segment thereof has been submitted to the lower layers, the transmitting RLC entity will stop transmission or retransmission of the RLC SDU or the segment, thus the receiving PDCP entity cannot receive the discarded SDU.

Observation 1: Based on the current agreement, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if stopReTxDiscardedSDU is configured, the receiving PDCP entity cannot receive the discarded SDU.

And based on the current RLC running CR for XR[3], T-RxDiscard timer should be configured in the Rx RLC entity to avoid Rx window stalling if stopReTxDiscardedSDU is configured in the Tx RLCentity. In which case, even if the RLC SDU or a segment thereof has been submitted to the lower layers, the RX RLC entity may discard it based on the T-RxDiscard timer.

Observation 2: Based on the current agreement, even if the RLC SDU or a segment thereof has been submitted to the lower layers, the RX RLC entity may discard it based on the T-RxDiscard timer.

Based on the current specification[2], when the PDCP SDU(s) are discarded in PDCP but the discarded PDCP SDU(s) or segment(s) thereof have been submitted by RLC to lower layers, it shall not trigger a PDCP SN gap report, thus the receiving PDCP entity cannot know that the SDU(s) have been discarded and will wait for the SDU(s) until the t-Reordering Timer expiring, this is not the intention of the PDCP SN gap report.

Observation 3: Based on the current specification, when the PDCP SDU(s) are discarded in PDCP but the discarded PDCP SDU(s) have been submitted by RLC to lower layers, it shall not trigger the a PDCP SN gap report.

Since transmitting PDCP entity already considers the RLC entity information(e.g. whether the transmitting AM RLC entity has submitted the discarded RLC SDU or a segment thereof to the lower layers), it is reasonable that the transmitting PDCP entity consider the RLC parameter(e.g. stopReTxDiscardedSDU) to decide whether to trigger PDCP SN gap report.

Observation 4: Based on the current specification, transmitting PDCP entity already considers the RLC entity information, it is reasonable to also consider the RLC parameter(e.g. stopReTxDiscardedSDU) when necessary.

Based on these observations, it is reasonable that if stopReTxDiscardedSDU is configured, PDCP SN gap report can be triggered when indicating lower layer (e.g. RLC) to discard a particular RLC SDU. e.g. the TP for TS 38.323:

### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - if stopReTxDiscardedSDU is not configured, the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. ...

Proposal 1(PDCP-2): If stopReTxDiscardedSDU is configured, PDCP SN gap report can be triggered even when the transmitting AM RLC entity has submitted the discarded RLC SDU or a segment thereof to the lower layers.

About open issue of RLC-10: Whether/How to introduce any limitations or associations for the configuration of stopReTxDiscardedSDU and DL t-RxDiscard.

For SDU discard indication from upper layer (e.g. PDCP), there are two branches:

The first is [2]:

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

The second is[3]:

##### 5.2.3.1.1 General ... If stopReTxDiscardedSDU is set to enabled, when receiving a discard indication for an RLC SDU with SN = x from the upper layer (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment for transmission or retransmission. Editor’s Note: FFS on the term, whether it should be “obsolete”, or “outdated”, or “discard”. Same as below.

Based on the above description, we can conclude that: if the RLC SDU or a segment thereof has been submitted to the lower layers, the transmitting side of an AM RLC entity shall not discard the indicated RLC SDU; the transmitting side of an AM RLC entity only stops the transmission or retransmission of the indicated RLC SDU if stopReTxDiscardedSDU is set to enabled.

Observation 5: Based on the current RLC running CR, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if the RLC SDU or a segment thereof has been submitted to the lower layers, it will not be discarded, and will not be transmitted.

Then for DL, if t-RxDiscard is not configured, the RLC SDU or RLC SDU segment(s) will stay in the transmitting window and lead both the transmitting window stalling and the receiving window stalling.

Observation 6: Based on the current RLC running CR, If the (re)Rx discarded SDU stop indication (e.g. stopReTxDiscardSDU) in transmitting side is set to enabled, but the Rx discarded Timer(e.g. t-RxDiscard timer) in receiving side has not been activated, the transmitting window and receiving window maybe stalling. It is a critical issue to be solved.

To avoid transmitting window maybe stalling, the following options can be considered:

Option 1: Transmitting window stalling and receiving window stalling are avoided based on Rx discarded Timer(e.g. t-RxDiscard timer) if the stop (re)Rx discarded SDU indication (e.g. stopReTxDiscardSDU) in transmitting side is set to enabled. Considering that the UL transmitting window forwarding depends on UL receiving side timer based discarding mechanism, but t-RxDiscard is specified only for DL, the UL receiving side timer based discarding is up to network implementation. How to specify the dependency between the transmitting side unnecessary (re) transmission and receiving side timer based discarding should be considered.

Option 2: Transmitting RLC entity discards the SDU when it stops (re)transmission of the SDU indicated from upper layer (e.g. PDCP) to discard.

Proposal 2(RLC-10): RAN2 discuss which of the following option is used to deal with the Transmitting window stalling:

Option 1: Transmitting window stalling and receiving window stalling are avoided based on Rx discarded Timer(e.g. t-RxDiscard timer) if the stop (re)Rx discarded SDU indication (e.g. stopReTxDiscardSDU) in transmitting side is set to enabled.

Option 2: Transmitting RLC entity discards the SDU when it stops (re)transmission of the SDU .

If Option 1 is agreed, RAN2 specify the dependency between the transmitting side unnecessary (re) transmission and receiving side timer based discarding.

If Option 2 is agreed, RLC SN gap report should be used to avoid the receiving window stalling.

Considering that the unnecessary (re)transmission avoidance and RLC SN gap report are both determined in transmitting RLC entity, it is more easily been coordinated, we prefer option2.

Observation 7: Unnecessary (re)transmission avoidance and RLC SN gap report are both determined in transmitting RLC entity, option 2 is more easy to be specified.

To support RLC SN gap report, similar as PDCP SN gap report, the RLC control PDU including the SN(s) of Discarded RLC SDU and/or Discarded RLC SDU segments is preferred.

Proposal 3(RLC-10): If option 2 is agreed, an RLC SN gap report is supported, which uses RLC control PDU including the SN(s) of Discarded RLC SDU(s) and/or Discarded RLC SDU segment(s).

About open issue of RLC-11: FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”.

In RAN2#130 meeting, there are the following agreements:

RAN2 agreement in #130 meeting: - Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) - Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

Considering that polling is used to trigger RLC status report, which triggers re-transmission based on NACK or stop retrannmission based on ACK, this is not necessary for the case that there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP. Polling retransmission cannnot deal with the window issue for the case that there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP: e.g. in this case, NACK will be included in the RLC status report(e.g. if t-RxDiscard timer does not expire), and the RLC entity can do nothing when receiving the NACK and the retransmission has been stopped.

Observation 8: Polling retransmission cannot deal with the window issue.

To deal with the window stalling, it should depend on the agreement for proposal 2.

Proposal 4(RLC-11): RAN2 confirms the Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission.

About open issue of RLC-12: FFS how to handle PDCP SN gap report handling during UE mobility.

In LS R3-247891, RAN3 describe the following issue:

For Rel-18 XR, it is unclear how to handle the PDCP SN gap report during UE’s inter-gNB mobility. If a UE discarded some UL packets before handover, the UE reports the PDCP SN gap report to the source gNB, but the target gNB may not be aware of those discarded packets if no PDCP SN gap is reported to target gNB. This may result in unnecessary delay in the target gNB.

RAN3 then list three options for resolving this issue:

- Alternative 1: The source gNB forwards the PDCP SN gap report information to the target gNB. - Alternative 2: The source gNB indicates the discarded SDUs as successfully received via the UL received status in the SN STATUS TRANSFER sent to the target gNB. - Alternative 3: The UE sends the PDCP SN gap report again after connecting to the target gNB.

It has been agreed in RAN2#129 meeting that

- From RAN2 point of view, for Rel-18 alternative 3 should not be supported.   RAN2 doesn’t see a big problem to solve for Rel-18.

- RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements.  If it is a problem further discussions are needed on the alternatives.

Since this issue only occurs when the discarded SDU has been sent to the target gNB and then source gNB receives the PDCP SN gap report, it is a corner case. And PDCP SN gap report cannot be notified in other case when SDU are discarded, e.g. in rel-19, t-RxDiscard timer based discarding mechanism can can stop the SDU (re)transmission and PDCP SN gap report cannot be sent for the discarded SDU..

Considering that the UE does not know whether the source gNB has discarded the SDU based on PDCP SN gap report, and the whether the discarded SDU has been transmitted to the target gNB, Alt 3 will waste the radio resource if the source gNB has discard the SDU based on PDCP SN gap report, and has not transmitted to the target gNB. Since alternative 1 and alternative 2 only impact RAN3 specification, whether or which alternative is used to handle PDCP SN gap report handling during UE mobility can be left to RAN3 decision.

Observation 9: Alternative 3 will waste the radio resource if the source gNB has discarded the SDU based on PDCP SN gap report, and has not transmitted to the target gNB.

Proposal 5(RLC-12): Alternative 3 is precluded. Whether or which alternative of Alt 1 and Alt 2 is used to handle PDCP SN gap report handling during UE mobility can be left to RAN3 decision.

# Conclusion

The following proposals are made:

Observation 1: Based on the current agreement, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if stopReTxDiscardedSDU is configured, the receiving PDCP entity cannot receive the discarded SDU.

Observation 2: Based on the current agreement, even if the RLC SDU or a segment thereof has been submitted to the lower layers, the RX RLC entity may discard it based on the T-RxDiscard timer.

Observation 3: Based on the current specification, when the PDCP SDU(s) are discarded in PDCP but the discarded PDCP SDU(s) have been submitted by RLC to lower layers, it shall not trigger the a PDCP SN gap report.

Observation 4: Based on the current specification, transmitting PDCP entity already considers the RLC entity information, it is reasonable to also consider the RLC parameter(e.g. stopReTxDiscardedSDU) when necessary.

Proposal 1(PDCP-2): If stopReTxDiscardedSDU is configured, PDCP SN gap report can be triggered even when the transmitting AM RLC entity has submitted the discarded RLC SDU or a segment thereof to the lower layers.

Observation 5: Based on the current RLC running CR, when indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, if the RLC SDU or a segment thereof has been submitted to the lower layers, it will not be discarded, and will not be transmitted.

Observation 6: Based on the current RLC running CR, If the (re)Rx discarded SDU stop indication (e.g. stopReTxDiscardSDU) in transmitting side is set to enabled, but the Rx discarded Timer(e.g. t-RxDiscard timer) in receiving side has not been activated, the transmitting window and receiving window maybe stalling. It is a critical issue to be solved.

Proposal 2(RLC-10): RAN2 discuss which of the following option is used to deal with the Transmitting window stalling:

Option 1: Transmitting window stalling and receiving window stalling are avoided based on Rx discarded Timer(e.g. t-RxDiscard timer) if the stop (re)Rx discarded SDU indication (e.g. stopReTxDiscardSDU) in transmitting side is set to enabled.

Option 2: Transmitting RLC entity discards the SDU when it stops (re)transmission of the SDU .

Observation 7: Unnecessary (re)transmission avoidance and RLC SN gap report are both determined in transmitting RLC entity, option 2 is more easy to be specified.

Proposal 3(RLC-10): If option 2 is agreed, an RLC SN gap report is supported, which uses RLC control PDU including the SN(s) of Discarded RLC SDU(s) and/or Discarded RLC SDU segment(s).

Observation 8: Polling retransmission cannot deal with the window issue.

Proposal 4(RLC-11): RAN2 confirms the Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission.

Observation 9: Alternative 3 will waste the radio resource if the source gNB has discarded the SDU based on PDCP SN gap report, and has not transmitted to the target gNB.

Proposal 5(RLC-12): Alternative 3 is precluded. Whether or which alternative of Alt 1 and Alt 2 is used to handle PDCP SN gap report handling during UE mobility can be left to RAN3 decision.

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- 3GPP TS 38.321 V18.4.0 (2024-12), NR;Medium Access Control (MAC) protocol specification

- [POST129bis][505][XR] RLC running CR and open issues (vivo)