3GPP TSG-RAN WG2 Meeting #131		R2-2505399

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreements were made for inter-CU LTM [1]:

Agreements on inter-CU LTM:

- RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

- If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

- RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE.

- The Enhanced LTM CSC MAC CE always contains an NCC value field. The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

In this paper, we will discuss the remaining open issues for inter-CU LTM.

- Discussion

  - [MAC-25] handling of target cell’s SP CSI-RS resources after cell switch

RAN2 agreed in the last meeting that the activated SP CSI-RS resources for candidate cells(other than the target cell) after cell switch should be deactivated and the handling of target cell SP CSI-RS resources is FFS.

- UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch.

- Postpone target cell case to the next meeting.

According to RAN1 LS [2], the semi-persistent CSI-RS resource can be used for the purpose of CSI acquisition:

LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells

- Similarly to the semi-persistent CSI-RS resource supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, RAN1 has also agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition.

Observation 1: RAN1 agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition.

According to the RAN1#120 meeting agreement, UE may measure the CSI-RS resource(s) after the reception of cell switch command for target cell CSI acquisition.

RAN1#120 Agreement

For target cell CSI acquisition,

- After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration

Therefore, the measurement of semi-persistent CSI-RS resources of target cell need to be continued after cell switch and it is used for the target cell CSI acquisition.

Proposal 1: [MAC-25] UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch.

  - [MAC-26] RACH-less intra-/inter-CU LTM and Conditional intra-CU LTM with two TAGs

According to the current MAC running CR [3], the procedure for setting the PREAMBLE_POWER_RAMPING_COUNTER to 1 during the early RACH for LTM candidate cells is defined as follows [3]:

TS 38.321 Section 5.1.1

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	flush the MSGA buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	if the Random Access procedure is initiated on a Serving Cell; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell and the PDCCH order indicates preamble initial transmission; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell, which is different from the cell to which the UE performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission:

2>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

Editor’s NOTE: In RACH-less (C)LTM, for an (C)LTM candidate cell which is configured with two TAGs, TAG ID is not considered when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1. Rapporteur realized the same issue exists for Rel-18 LTM, which was missing.

As noted by the editor, the (C)LTM candidate cell which is configured with two TAGs is not considered when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1. For example, assuming LTM candidate cell#1 is configured with two TAGs (e.g., TAG#1 and TAG#2), the issue is illustrated in Figure 1.

Figure 1. Powering ramping issue for early-RACH

Step 1: The NW indicates UE to perform random access with preamble initial transmission associated with TAG#1 of LTM candidate cell#1, with the current PREAMBLE_POWER_RAMPING_COUNTER set to 1.

Step 2: The NW indicates UE to perform random access with preamble re-transmission associated with TAG#1  of LTM candidate cell#1, with the current PREAMBLE_POWER_RAMPING_COUNTER set to 2;

Step 3: The NW indicates UE to perform Random Access with preamble initial transmission associated with TAG#2 of LTM candidate cell#1, but the PDCCH order is not received by UE.

Step 4: The NW indicates UE to perform Random Access with preamble re-transmission associated with TAG#2 of LTM candidate cell#1. The current PREAMBLE_POWER_RAMPING_COUNTER remains at 2 and will not be set to 1 according to the current specification, as the cell for the random access preamble transmission has not changed. However, the PREAMBLE_POWER_RAMPING_COUNTER should be set to 1 instead of 2, as this is the initial preamble transmission for the UE associated with TAG#1 of LTM candidate cell #1.

Observation 2: According to the current MAC running CR, for the same (C)LTM candidate cell, if the PDCCH order indicates preamble re-transmission associated with a TAG different from the TAG to which the UE performed the last Random Access Preamble transmission, the PREAMBLE_POWER_RAMPING_COUNTER will not be reset to 1.

To address this issue, for the same (C)LTM candidate cell, the PREAMBLE_POWER_RAMPING_COUNTER should be set to 1 if PDCCH-order indicates preamble re-transmission associated with a TAG different from the TAG to which the UE performed the last Random Access Preamble transmission.

In fact, the same issue exists in the Rel-18 specification, and both Rel-18 and Rel-19 specifications need to be updated. The TP is provided in the Annex.

Proposal 2: [MAC-26] The PREAMBLE_POWER_RAMPING_COUNTER should be set to 1 if the PDCCH order indicates preamble re-transmission associated with a TAG different from the TAG within the same (C)LTM candidate cell to which the UE performed the last Random Access Preamble transmission.

Proposal 3: [MAC-26] If proposal 2 is agreed, adop the TP in Annex for both Rel-18 and Rel-19 specification.

  - [UE capability Issue-12] Whether to define a capability for inter-CU LTM recovery

In the email discussion regarding the UE capability issue for mobility [4], whether to define a separate per-UE capability for inter-CU LTM fast recovery was discussed.

In our view, a separate UE capability for inter-CU LTM fast recovery needs to be defined rather than reusing the existing capability, ltm-Recovery-r18, due to the differing behaviors of inter-CU and intra-CU LTM fast recovery. For instance, unlike the intra-CU LTM fast recovery, the UE must compare the Rel-19 ID of the selected cell with that of the LTM target cell after inter-CU LTM failure to determine whether inter-CU LTM fast recovery can be executed. Additionally, the UE is required to perform PDCP re-establishment and a security key update during inter-CU LTM fast recovery.

Proposal 4: A separate UE capability for inter-CU LTM fast recovery is defined.

If Proposal 4 is agreed, a separate indicator for inter-CU LTM fast recovery should be introduced in the RRC specification instead of reusing the existing attemptLTM-Switch IE. Intra-CU LTM fast recovery will be supported when the attemptLTM-Switch IE is configured, while inter-CU LTM fast recovery will be supported when the new indicator for inter-CU LTM fast recovery is configured.

Proposal 5: If Proposal 4 is agreed,  a separate indicator to enable inter-CU LTM fast recovery should be introduced in the RRC specification instead of reusing the existing attemptLTM-Switch IE.

  - Inter-CU LTM fast recovery after L3 HO or CHO failure

In the last meeting, RAN2 discussed the LTM fast recovery after inter-CU LTM failure and reached the following agreement:

- If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

However, the feasibility of performing inter-CU LTM fast recovery after L3 HO or CHO has not yet been addressed.

According to the agreement made in RAN2#125bis, intra-CU LTM fast recovery after any L3 HO or CHO failure is not supported in Rel-18.

RAN2#125bis Agreement

- LTM recovery is not supported for T304 expiry after HO (L3 HO) and CHO, but supported for T304 expiry after LTM cell switch (and RLF as earlier agreed). This agreement supersedes earlier agreements.

The motivation of the agreement is to avoid performing an intra-CU LTM fast recovery after an inter-CU handover. This is because, following an inter-CU handover, PDCP re-establishment and RLC re-establishment will be performed, making it difficult for a UE to revert back to the source cell configuration once buffers and state variables have been reset, and in this case the intra-CU LTM couldn’t be performed successfully.

Observation 3: Intra-CU LTM fast recovery after L3 HO or CHO failure is not supported in Rel-18.

Regarding Rel-19 inter-CU LTM fast recovery, PDCP re-establishment will be performed during the inter-CU LTM fast recovery so the revert back issue discussed in Rel-18 does not exist. However, new issues for the inter-CU LTM fast recovery need to be considered.

There are two scenarios for the inter-CU LTM fast recovery after L3 HO or CHO failure:

- Case 1: inter-CU LTM fast recovery after L3 HO or CHO failure without security key update

- Case 2: inter-CU LTM fast recovery after L3 HO or CHO failure with security key update

In Case 1, the UE has no available NCC for security key derivation for inter-CU LTM fast recovery. Therefore, Case 1 is not supported.

Observation 4: Inter-CU LTM fast recovery after intra-CU L3 HO or CHO failure cannot be supported due to no NCC can be used for security key derivation.

For case 2, whether the NCC included in the RRCReconfiguration of L3 HO or CHO can be utilized for LTM fast recovery is not clear, because the handover preparation procedure and security key distribution procedure of L3 HO/CHO and LTM are separate, the source gNB may generate security keys for the target cell of L3 HO/CHO and the LTM candidate cells based on different NCC value. Supporting case 2 needs further discussion in RAN2 and RAN3.

Observation 5: Supporting inter-CU LTM fast recovery after inter-CU L3 HO or CHO failure will need further discussion in RAN2 and RAN3.

Considering that there is limited time remaining in Rel-19, we propose to adhere to the Rel-18 principle and not support any form of LTM fast recovery following L3 HO or CHO failure.

Proposal 6: After L3 HO or CHO failure, the UE cannot perform inter-CU LTM fast recovery.

  - CFRA resource in LTM Cell Switch Command MAC CE

To support the CFRA resource configuration included in the LTM cell switch command MAC CE for inter-CU LTM, RAN3 reached the following agreement in the last meeting:

RAN3#128 Agreement

In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

However, whether the CFRA resource provided by the candidate gNB can be shared by multiple UEs or is reserved for a delicated UE has not yet been discussed.

In our view, there are two potential approaches:

Approach 1: Allocated by Target DU

- During the handover preparation phase, S-DU obtains dedicated CFRA resources per UE from T-DU;

- S-DU then allocates specific CFRA resource to the UE to perform LTM cell Switch.

Approach 2: Allocated by Source DU

- During the handover preparation phase, S-DU obtains a shared CFRA resource pool from T-DU for multiple UEs within a single DU;

- S-DU then allocates specific CFRA resource from the pool to the UE to perform LTM Cell Switch.

For approach 1 (CFRA resource allocated by Target DU), specific CFRA resources need to be reserved for each UE. This could lead to significant resource wastage when subsequent LTM is supported since CFRA resources would have to remain reserved over extended periods of time for LTM candidate cells. Additionally, if CFRA resources are allocated by Target DU, there would be no practical difference between including them in an LTM Cell Switch Command MAC CE versus including them in preconfigured candidate RRC configurations. This goes against our original motivation behind including CFRA resources within an LTM Cell Switch Command MAC CE.

For approach 2 (CFRA resource allocated by source DU), CFRA resources are shared among UEs within a single DU, which is similar to the early-RACH resources sharing mechanism supported in Rel-18 LTM. The S-DU is responsible for allocating specific CFRA resources for each UE to perform RACH-based LTM cell switch while simultaneously avoiding the CFRA resource conflicts. As a result, the resource wastage can be largely reduced. Therefore, we prefer using Approach 2.

Proposal 7: For inter-CU LTM, a target cell specific CFRA resource pool could be provided by target gNB-DU to source gNB-DU,  and the CFRA resource are assigned by S-DU in the LTM cell switch command MAC CE.

Since the CFRA resources for RACH-based LTM are assigned to UE by source DU, the source DU needs to inform the target DU about the mapping relationship between the CFRA resource information (e.g., preamble index and RA-RNTI) and the UE to perform LTM. This ensures that the target cell knows which UE is performing RACH-based LTM via the CFRA resource.

Observation 6: For inter-CU LTM, the target cell needs to identify which UE is performing CFRA-based RACH and which CFRA resources are being used when the CFRA resources are shared among multiple UEs.

Proposal 8: Send an LS to inform RAN3 to support the CFRA resources included in the LTM Cell Switch Command MAC CE assigned by S-DU for inter-CU LTM.

  - Coexistence of inter-CU LTM with MIMO 2TA

- Coexistence of inter-CU LTM with Rel-18 MIMO 2TA

In RAN2#129bis meeting, the following agreements were made for the co-existence of inter-CU LTM with Rel-18 MIMO 2TA,

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

However, whether RACH-based inter-CU LTM can coexist with MIMO 2TA has not been discussed.

As for RACH-based LTM and MIMO two TA, the same TAG mismatch issue exists as we discussed in Rel-18 [5]. Specifically, if a TCI state ID included in the LTM Cell Switch Command MAC CE is associated with TRP 1, while the TAG of TA in the RAR for RACH-based LTM is associated with TRP 2, after completing the LTM cell switch, UE shall use the TCI state in the LTM Cell Switch Command MAC CE for data transmission without having any available TA value associated to the TCI state.

Observation 7: Mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch Command MAC CE is associated with one TAG while the TA value in the RACH RAR is associated with another TAG, assuming the co-existence between RACH-based LTM and MIMO two TA is supported.

In [5], four options were proposed to resolve the problem:

- Option 1: After the completion of LTM, the target DU initiates a PDCCH order triggered RACH procedure via the indicated TCI state in LTM Cell Switch Command. The RACH procedure is performed to obtain the TA value associated with the indicated TCI state.

- Option 2: The target DU sends a new TCI state in RACH Msg4. The new TCI state is associated with the TAG indicated in the RACH RAR.

- Option 3: If the mismatch issue occurs, after the completion of RACH procedure, UE follows the TCI state associated to the RACH procedure, i.e. the DL TCI state is associated to the SSB for RACH procedure and the UL TCI state is associated with the TCI state for Msg3 transmission. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the TAG ID same as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM.

Due to time constraints, it was agreed that network implementation (e.g., option 1) would handle this issue in Rel-18.

RAN2#127 discussion for RACH based LTM and 2 TAs

- RAN2 understand NW can handle this situation, e.g. by PDCCH order. No need of new option.

Actually, initiating a PDCCH order-triggered RACH procedure via the indicated TCI state after LTM cell switch completion would result in additional latency. To ensure the UE can perform data transmission in the target cell immediately, other options need to be considered.

Proposal 9: RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

- Option 2: Target DU sends a new TCI state in RACH Msg 4.

- Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

Proposal 10: if Option 3 in Proposal 9 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

- Coexistence of inter-CU LTM with Rel-19 MIMO 2TA

In Rel-19 MIMO 2TA, UL-only TRP is introduced. When a UL-only TRP is configured, the pathlossOffset for it should be indicated so that UE can perform uplink transmission via this TRP. In Rel-19 MIMO, this pathlossOffset for the UL-only TRP is indicated in the TCI-state of the serving cell and the network will indicate the TCI-state info for acquiring the pathlossOffset in the PDCCH-order.

However, if a UL-only TRP is configured for an LTM candidate cell, it remains unclear how the UE acquires the pathlossOffset to perform early RACH procedures for this TRP. To support PDCCH-order based TA measurement for such UL-only TRPs in LTM candidate cells, potential enhancements may be required.

Therefore, we propose to send an LS to ask RAN1 whether Rel-19 MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

Proposal 11: Send an LS to ask RAN1 whether Rel-19 MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- Conclusion

In this paper, we discuss the remaining open issues for inter-CU LTM. We have the following observations and proposals:

[MAC-25] handling of target cell’s SP CSI-RS resources after cell switch

Observation 1: RAN1 agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition.

Proposal 1: [MAC-25] UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch.

[MAC-26] RACH-less intra-/inter-CU LTM and Conditional intra-CU LTM with two TAGs

Observation 2: According to the current MAC running CR, for the same (C)LTM candidate cell, if the PDCCH order indicates preamble re-transmission associated with a TAG different from the TAG to which the UE performed the last Random Access Preamble transmission, the PREAMBLE_POWER_RAMPING_COUNTER will not be reset to 1.

Proposal 2: [MAC-26] The PREAMBLE_POWER_RAMPING_COUNTER should be set to 1 if the PDCCH order indicates preamble re-transmission associated with a TAG different from the TAG within the same (C)LTM candidate cell to which the UE performed the last Random Access Preamble transmission.

Proposal 3: [MAC-26] If proposal 2 is agreed, adop the TP in Annex for both Rel-18 and Rel-19 specification.

[UE capability Issue-12] Whether to define a capability for inter-CU LTM recovery

Proposal 4: A separate UE capability for inter-CU LTM fast recovery is defined.

Proposal 5: If Proposal 4 is agreed,  a separate indicator to enable inter-CU LTM fast recovery should be introduced in the RRC specification instead of reusing the existing attemptLTM-Switch IE.

Inter-CU LTM fast recovery after L3 HO or CHO failure

Observation 3: Intra-CU LTM fast recovery after L3 HO or CHO failure is not supported in Rel-18.

Observation 4: Inter-CU LTM fast recovery after intra-CU L3 HO or CHO failure cannot be supported due to no NCC can be used for security key derivation.

Observation 5: Supporting inter-CU LTM fast recovery after inter-CU L3 HO or CHO failure will need further discussion in RAN2 and RAN3.

Proposal 6: After L3 HO or CHO failure, the UE cannot perform inter-CU LTM fast recovery.

CFRA resource in LTM Cell Switch Command MAC CE

Observation 6: For inter-CU LTM, the target cell needs to identify which UE is performing CFRA-based RACH and which CFRA resources are being used when the CFRA resources are shared among multiple UEs.

Proposal 7: For inter-CU LTM, a target cell specific CFRA resource pool could be provided by target gNB-DU to source gNB-DU,  and the CFRA resource are assigned by S-DU in the LTM cell switch command MAC CE.

Proposal 8: Send an LS to inform RAN3 to support the CFRA resources included in the LTM Cell Switch Command MAC CE assigned by S-DU for inter-CU LTM.

Coexistence of inter-CU LTM with MIMO 2TA

Observation 7: Mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch Command MAC CE is associated with one TAG while the TA value in the RACH RAR is associated with another TAG, assuming the co-existence between RACH-based LTM and MIMO two TA is supported.

Proposal 9: RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

- Option 2: Target DU sends a new TCI state in RACH Msg 4.

- Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

Proposal 10: if Option 3 in Proposal 9 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

Proposal 11: Send an LS to ask RAN1 whether Rel-19 MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- References

- RAN2#130_ChairNotes.

- R1-2503079, LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells.

- R2-2505397, Introduction of NR mobility enhancements Phase 4 in MAC

- R2-2505164, Report of [POST130][120][MOB] (CATT)

- R2-2406439, Discussion on MAC open issue for LTM. vivo.

Annex: TP for TS 38.321

5.1.1	Random Access procedure initialization

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	flush the MSGA buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	if the Random Access procedure is initiated on a Serving Cell; or

1>	if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell and the PDCCH order indicates preamble initial transmission; or

- if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell, which is different from the cell to which the UE performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission; or

- if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell which is configured with two TAGs, which is same from the cell to which the UE performed the last Random Access Preamble transmission, and the TAG id of the TCI state associated with the SSB in the PDCCH order is different from the TAG associated with the cell to which the UE performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission:

2>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;