3GPP TSG RAN WG2 #131	R2-2505428

Bengaluru, India, 25th – 29th August 2025

# Title:	Reply LS on the RAN simulation assumptions for ULBC

# Response to:	R2-2505068/ S4-251584

# Release:	Release 20

# Work Item:	FS_ULBC

Source:	RAN WG2

To:	SA WG4

Cc:	RAN WG1, SA WG2, CT WG1

Contact Person:

Name:	Bharat Shrestha

E-mail Address:	bshresth AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

RAN2 would like to thank SA4 for the LS. RAN2 would also like to provide the following response.

Question 1: the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option.

RAN2 Answer: For NB-IoT, RAN2 expects minimum overhead of 3 bytes for RoHC, 1 byte for PDCP and 1 byte for RLC UM when UP solution is used and minimum overhead of 2 bytes for RRC and 2 bytes for RLC AM when CP solution is used. RAN2 will further discuss the down selection between CP and UP solutions for voice support over NB-IoT-NTN and any AS layer optimization in the upcoming RAN2 meetings as this requires further coordination with SA2. RAN2 will also introduce SPS for voice over NB-IoT. RAN2 will provide further updates in the upcoming meetings.

Question 2: specifically, whether a packet overhead of 1 byte of MAC header is realistic.

RAN2 answer: In TS 36.321 Figure 6.1.6-2, 1 byte R/R/E/LCID MAC subheader format is already supported given the MAC header is the only MAC subheader in the PDU, i.e., field E = 0. If the MAC PDU includes only voice data and it fits in the TBS size configured for the SPS for voice over NB-IoT NTN, the total MAC header is limited to 1 byte.

# 2. Actions:

# To SA4:

ACTION:

RAN2 kindly asks SA4 to take the above response into account.

# 3. Dates of Next RAN2 Meetings:

RAN2#131bis						13th to 17th October 2025		Prague

RAN2#132						17th to 21st November 2025		Dallas