3GPP TSG-RAN WG2#131	R2-2505506

Bengaluru, India, Aug 25th – 29th 2025

Source: 	Fujitsu

Agenda item:	8.5.2

Title: 	Remaining issues on on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#130 meeting, RAN2 has discussed the on-demand SSB SCell operation, and the following agreement was made [1]:

Agreements on OD-SSB 1. (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst. 2. The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed. 3. When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated: • OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped. 4. For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE. 5. In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation: • The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation. • The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly. Spec impact can be further discussed in running CR preparation. 6. When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO. 7. The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured. 8. RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement. 9. RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported. 10. When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell. 11. RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

In this contribution, we would like to discuss the remaining issues for finalization of on-demand SSB SCell operation.

# Discussion

## On-demand SSB SCell

### OD-SSB configuration

(RRC/OD-SSB issue3) location of a new servingCellMO

In the RRC running CR [2], servingCellMO-r19 which indicates linking between OD-SSB and measurement object was implemented in the OD-SSB-Config-r19 as below:

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-19                      OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19           	ENUMERATED {activated}

od-ssb-Periodicity-r19             	ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-19                  	INTEGER (0..15) 				OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19             	ENUMERATED {zero, one}		OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19          	ARFCN-ValueNR					OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19       CHOICE {

shortBitmap                        BIT STRING (SIZE (4)),

mediumBitmap                       BIT STRING (SIZE (8)),

longBitmap                         BIT STRING (SIZE (64))

}                                      									OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19           	SubcarrierSpacing			OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19             	INTEGER (-60..50)			OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                  	INTEGER (0..8)				OPTIONAL, -- Need R

servingCellMO-r19                     	MeasObjectId					OPTIONAL, -- Cond InterFreq

smtc1-r19                           	SSB-MTC							OPTIONAL, -- Cond IntraFreq

...

}

However, the current implementation is different from the original intention in RAN2. In fact, RAN2 agreed to include a new servingCellMO in ServingCellConifg, not in OD-SSB-Config. It may work but we think it is simpler to include it in ServingCellConfig as it can reuse the same message structure in legacy. We also think even if it is included in ServingCellConfig, the current procedural text is not changed (similar to servingCellMO-r17). Hence, RAN2 should reconfirm the agreement in RAN2#130, i.e., when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Proposal 1:	(RRC/OD-SSB issue3) For Case#2, RAN2 to confirm to include servingCellMO-r19 in ServingCellConfig, when AO-SSB and OD-SSB have different center frequency.

Absence of parameters in OD-SSB configuration

The OD-SSB configuration supports both Case#1 (no always-on SSB) and Case#2 (with always-on SSB). According to the RAN1 LS [3], for Case#2, because the value of some OD-SSB parameter can be the same as that of always-on SSB, it can be absent and the value indicated in the always-on SSB configuration (i.e., SSB indicated by the ServingCellConfigCommon) can be applied to the OD-SSB. But in the current RRC running CR [2], such descriptions would be missing at least in PBCH block power and SSB sub-carrier spacing while SSB position in burst is correctly captured as follows:

od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

In order to clarify that, we suggest adding the similar descriptions for both PBCH block power and SSB sub-carrier spacing in the OD-SSB configuration.

Proposal 2:	For Case#2, if the PBCH block power or SSB sub-carrier spacing for OD-SSB is not configured, the UE applies the same value configured for the always-on SSB. RAN2 to clarify it in the field description.

od-ss-PBCH-BlockPower Indicates average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7. If absent, od-ss-PBCH-BlockPower is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

od-ssbSubcarrierSpacing Indicates subcarrier spacing of OD-SSB, for Case #1, i.e., no always-on SSB on this serving cell. If absent, od-ssbSubcarrierSpacing is the same as ssbSubcarrierSpacing provided in ServingCellConfigCommon. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

### On-demand SSB activation/deactivation

(RRC/OD-SSB issue2) Nr of OD-SSB burst configuration

In RAN1#121 meeting, RAN1 has concluded there is no RAN1 consensus to support RRC activation of OD-SSB configuration as the highlighted below [4].

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

Observation 2:	RAN1 has concluded there is no consensus to support RRC activation of OD-SSB configuration as an initial OD-SSB state.

Because the RRC processing time for activation may not be correctly estimated by the NW, then it is difficult to align the number of OD-SSB transmission between the UE and the NW. Thus, it is reasonable to follow the RAN1 conclusion however it is not reflected in the running RRC CR. To clarify this, it should be updated with conditional presence field.

Proposal 3:	(RRC/OD-SSB issue2) The od-ssb-nrofBurst-r19 can be configured only if the od-ssb-ActivationStatus-r19 is absent. To clarify that, the od-ssb-nrofBurst-r19 should be defined with conditional presence field as follows:

od-ssb-nrofBurst-r19                   INTEGER (1..8)				OPTIONAL, -- Cond DeactiveNeed R

| Conditional Presence | Explanation |
| --- | --- |
| Deactive | The field is optionally present, Need R, if od-ssb-ActivationStatus-r19 is absent. It is absent otherwise. |

Considering the above proposals, TP for OD-SSB-Config-r19 is shown in Annex A.

Proposal 4:	Adopt TP for OD-SSB-Configu-r19 shown in Annex A

# Conclusion

In this contribution, we have some discussions about on-demand SSB SCell operations, and the following proposals are made:

Proposal 1:	(RRC/OD-SSB issue3) For Case#2, RAN2 to confirm to include servingCellMO-r19 in ServingCellConfig, when AO-SSB and OD-SSB have different center frequency.

Proposal 2:	For Case#2, if the PBCH block power or SSB sub-carrier spacing for OD-SSB is not configured, the UE applies the same value configured for the always-on SSB. RAN2 to clarify it in the field description.

Proposal 3:	(RRC/OD-SSB issue2) The od-ssb-nrofBurst-r19 can be configured only if the od-ssb-ActivationStatus-r19 is absent. To clarify that, the od-ssb-nrofBurst-r19 should be defined with conditional presence field as follows:

od-ssb-nrofBurst-r19                   INTEGER (1..8)				OPTIONAL, -- Cond DeactiveNeed R

| Conditional Presence | Explanation |
| --- | --- |
| Deactive | The field is optionally present, Need R, if od-ssb-ActivationStatus-r19 is absent. It is absent otherwise. |

Proposal 4:	Adopt TP for OD-SSB-Configu-r19 shown in Annex A

# References

- R2-2504671, Report from session on R18 SL, R18/19 MOB, and R19 NES

- R2-2505708, Running RRC CR for enhancements for network energy efficiency

- R2-2505000, LS on Rel-19 higher layers parameters list Post RAN1#121

- RAN1#121 chairman’s note

# Annex A: TP for OD-SSB-Config

### 6.3.2	Radio resource control information elements

#### –	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by a MAC CE see TS 38.321 [3], clause XXX

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated},

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                  INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                          OPTIONAL, -- Cond Deactive

smtc1-r19                              SSB-MTC                                                                 OPTIONAL, -- Cond IntraFreq

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

OD-SSB-Config field descriptions

smtc1 Primary measurement timing configuration (see clause 5.5.2.10) to be used instead of smtc1 configured in servingCellMO in IE servingCellConfig when this OD-SSB is activated and the serving cell is activated

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS 38.213.

od-ssb-ActivationStatus Indicates the activation status of this OD-SSB pattern upon configuration. Only one OD-SSB pattern can be activated.

od-ssb-halfFrameIndex Indicates whether OD-SSB is in the first half or the second half of the frame. If the field is absent, the UE applies the value 0.

od-ssb-sfn-Offset Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is activated.

od-ssb-Periodicity The SSB periodicity in ms. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1).

od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

od-ss-PBCH-BlockPower Indicates average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7. If absent, od-ss-PBCH-BlockPower is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

od-ssbSubcarrierSpacing Indicates subcarrier spacing of OD-SSB, for Case #1, i.e., no always-on SSB on this serving cell. If absent, od-ssbSubcarrierSpacing is the same as ssbSubcarrierSpacing provided in ServingCellConfigCommon. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

| Conditional Presence | Explanation |  |
| --- | --- | --- |
| ODssbOnly | The field is optionally present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is absent otherwise. |  |
| ODssbAOssb | The field is mandatory present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is optionally present otherwise. |  |
| InterFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. FFS SSB-less SCell |  |
| IntraFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates same frequency as absoluteFrequencySSB of the serving cell. It is absent otherwise. FFS SSB-less SCell |  |
| Deactive | The field is optionally present, Need R, if od-ssb-ActivationStatus-r19 is absent. It is absent otherwise. |  |