3GPP TSG-RAN WG2 Meeting #131       	                      R2-2505513

Bangalore, India, 25th-29th Aug 2025

Agenda item:	8.3.2.3

Source:	CMCC

Title:	Discussion on data collection for UE-sided model

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 had the following agreements on data collection for UE-sided model:

- For UE sided data collection – use AI/ML PHY request/configuration framework as baseline.  FFS enhancements/or differences

This contribution will focus on data collection for RRM measurement prediction and measurement event prediction with UE-sided model.

- Discussion

During the discussion on UE-side data collection in AI/ML PHY, RAN2 has the following agreements on UE request:

- RAN2#129bis Agreements

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

On the other hand, RAN2 also has the following agreements on data collection configuration:

- RAN2#129 Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In last meeting, RAN2 agreed to use AI/ML PHY request/configuration framework as baseline. We believe that the above agreements can be also applied for UE side data collection for RRM measurement prediction and measurement event prediction use cases.

Proposal 1: The following agreements on request/configuration for UE side data collection in AI/ML PHY are also applied for RRM measurement prediction and measurement event prediction.

-  The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end.

-  Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

-  Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

-  The network can provide or release the data collection configuration (at any point in time), with or without UE request.

-  The following methods for network control of the initiation and configuration for data collection:

•	The network can decide when to start/stop the data collection and send configuration.

•	The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In last meeting, RAN2 agreed that the UE configuration is provided via the RRM measurement framework for NW side data collection. For UE side data collection, the RRM measurement framework can be also used to provide the data collection configuration to the UE.

Proposal 2: For UE side data collection, the RRM measurement framework is used to provide the data collection configuration to the UE.

In AI/ML PHY, RAN2 agreed that the UE doesn’t need to measure the candidate data collection configuration(s). The similar principle is also applicable for AI Mob.

Proposal 3: For AI Mob, the UE doesn’t need to measure the candidate data collection configuration(s).

For candidate data collection configuration, RAN2 needs to discuss the content of configuration for UE side data collection. In last meeting, RAN2 agreed that associated ID should be optionally configurable for training and inference.

RAN2#130 agreements Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

For AI/ML PHY, RAN2 agreed that the candidate data collection configuration includes at least CSI resource of Set A, Set B, and one/two associated IDs.

- RAN2 agreements on AI/ML PHY - For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

For AI/ML based RRM measurement prediction and indirect measurement event prediction, the configuration includes at least measurement resource of cell(s) and/or frequency(es), and (optional) associated ID.

Proposal 4: For AI/ML based RRM measurement prediction and indirect measurement event prediction, the configuration includes at least:

- measurement resource of cell(s) and/or frequency(es)

- (optional) associated ID

For AI/ML based direct measurement event prediction, the configuration includes at least measurement resource of cell(s) and/or frequency(es), measurement event configuration, and (optional) associated ID.

Proposal 5: For AI/ML based direct measurement event prediction, the configuration includes at least:

- measurement resource of cell(s) and/or frequency(es)

- measurement event configuration

- (optional) associated ID

During the discussion on beam management in AI/ML PHY, the dataCollectionPreferredConfiguration IE is introduced to indicate the preferred configuration(s) among the candidate configuration(s) for UE side data collection. For AI Mob, given the multiple different use cases, e.g. time/spatial/frequency RRM measurement prediction, L1/L3 beam/cell level measurement prediction, and direct/indirect measurement event prediction, the network may not configure the data collection configurations for all scenarios considering the complexity and signaling overhead. Therefore, there may be no UE preferred configuration among candidate configurations. We think the potential enhancement can be considered that the UE is allowed to request data collection configuration for other cells or frequencies which are not configured within the candidate configuration for UE side data collection.

Proposal 6: The dataCollectionPreferredConfiguration IE may be enhanced to allow the UE to request data collection configuration for other cells or frequencies which are not configured within the candidate configuration for UE side data collection.

# 3	Conclusion

Here are the proposals on UE side data collection for AI Mob.

Proposal 1: The following agreements on request/configuration for UE side data collection in AI/ML PHY are also applied for RRM measurement prediction and measurement event prediction.

-  The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end.

-  Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

-  Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

-  The network can provide or release the data collection configuration (at any point in time), with or without UE request.

-  The following methods for network control of the initiation and configuration for data collection:

•	The network can decide when to start/stop the data collection and send configuration.

•	The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

Proposal 2: For UE side data collection, the RRM measurement framework is used to provide the data collection configuration to the UE.

Proposal 3: For AI Mob, the UE doesn’t need to measure the candidate data collection configuration(s).

Proposal 4: For AI/ML based RRM measurement prediction and indirect measurement event prediction, the configuration includes at least:

- measurement resource of cell(s) and/or frequency(es)

- (optional) associated ID

Proposal 5: For AI/ML based direct measurement event prediction, the configuration includes at least:

- measurement resource of cell(s) and/or frequency(es)

- measurement event configuration

- (optional) associated ID

Proposal 6: The dataCollectionPreferredConfiguration IE may be enhanced to allow the UE to request data collection configuration for other cells or frequencies which are not configured within the candidate configuration for UE side data collection.

# 4	Reference

- Draft_RAN2_130_Meeting_Report_v3