3GPP TSG-RAN WG2 Meeting #131			           		R2-2505525

Bangalore, India, August 25 - 29, 2025

Agenda item:	8.0

Source:	Samsung, Xiaomi

Title:	Per band and per BC capability

Document for:	Discussion and Decision

# Introduction

We observe that RAN1 defines increasing number of per band and per BC capability signaling (more than 50 potentially in Rel-19 compared with Rel-18 which has around 30 per band and per BC capabilities). However, there was no general discussion on handling of per band and per BC capability since Rel-16 [1]. Therefore, in this contribution, we would like to discuss general understanding to ensure that the specification is clear.

# Discussion

First, we would like to clarify what is per band and per BC capability signaling because it is not indicated in the column of “per”. In our understanding, per band and per BC capability means that UE capability is defined in both per band and per BC with same feature components.

Observation 1: per band and per BC capability refers to UE capability is defined in both per band and per BC with same feature components.

According to Rel-16 discussion, per BC capability is indicated when further limitation is needed for the capability indicated in per band. Therefore, the final UE capability is determined considering both per band and per BC capability. Based on it, the following highlighted sentence is included in codebookComboParametersAdditionPerBC-r16. However, it is not described for the features introduced in the later release.

Observation 2: the relationship between per band and per BC capability is not described for all the capabilities.

| codebookComboParametersAdditionPerBC-r16 Indicates the list of supported CSI-RS resources across all bands in a band combination by referring to codebookVariantsList for the mixed codebook types. For mixed codebook types, UE reports support active CSI-RS resources and ports for up to 4 mixed codebook combinations in any slot. The following parameters are included in codebookVariantsList for each code book type: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource across all bands within a band combination; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band combination, simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band combination, simultaneously. For each band in a band combination, supported values for these three parameters are determined in conjunction with codebookComboParametersAddition-r16 reported in MIMO-ParametersPerBand. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Given that there are many per band and per BC capabilities to be introduced, it would be desirable to confirm RAN2’s understanding and if agreeable, to capture it in the spec.

Furthermore, “in conjuction with” may not be so straightforward considering the complicated characteristics of UE capabilities. Based on Rel-16 discussion, per band and per BC capabilities are introduced to have further restriction in a certain BC on top of per band capability i.e. under-reporting purpose.  With this assumption, per BC capability should not simply override per band capability.

The following is our understanding on how to interpret “in conjunction with”.

- The lower capability derived based on both per band and per BC capability is applied. That is, it doesn’t mean that per BC capability always override per band capability or vice versa.

- If per band capability is absent for a band, the feature is not supported for the band regardless of per BC capability i.e. even if per BC capability is indicated for a BC associating the band.

- Per BC capability can be indicated when at least one band in a BC indicates corresponding per band capability. Since absence of per band capability means UE cannot support the feature, if UE cannot support a feature in all bands in a BC, there is no meaning to allow per BC capability.

- If per BC capability is absent for a BC, per band capability is applied by default.

It would be helpful if RAN2 can check if our understanding is correct on per band and per BC capability.

Proposal 1: The lower capability derived based on both per band and per BC capability is applied.

Proposal 2: If per band capability is absent for a band, the feature is not supported for the band even if per BC capability is indicated for a BC associating the band.

Proposal 3: per BC capability can be indicated when at least one band in a BC indicates corresponding per band capability.

Proposal 4: If per BC capability is absent for a BC, per band capability is applied for the BC.

Proposal 5: if proposal 1-4 is agreed, RAN2 agree to add high level description for per band and per BC capabilities in TS38.306.

Proposal 6: if proposal 1-4 is agreed, RAN2 agree to send an LS to RAN1 to check if the assumptions are correct for per band and per BC capabilities that they introduced.

One more aspect to discuss is pre-requisite for per band and per BC capabilities. In the current specification, pre-requisite is defined for per band and per BC separately. For example, when feature A is pre-requisite of feature B, UE supporting feature B in per band capability shall support feature A in per band capability and UE supporting feature B in per BC capability shall support feature A in per BC capability according to the current specification. There is no critical issue with this approach but it is not clear if it is aligned with RAN1 assumption because RAN1 didn’t indicate any specific relationship between per band and per BC capabilities in terms of pre-requisite. And also, given that both per band and per BC capability indicates the same feature, combined interpretation might be possible. For example, UE could indicate per band and per BC capability for feature B even if UE indicate per band capability for feature A.

Proposal 7: RAN2 agree to ask RAN1 to confirm if it is correct to assume that pre-requisite for per band and per BC capabilities is defined for per band and per BC separately.

# Conclusion

In this contribution, we clarified the per band and per BC capability and propose the following observations and proposals.

Observation 1: per band and per BC capability refers to UE capability is defined in both per band and per BC with same feature components.

Observation 2: the relationship between per band and per BC capability is not described for all the capabilities.

Proposal 1: The lower capability derived based on both per band and per BC capability is applied.

Proposal 2: If per band capability is absent for a band, the feature is not supported for the band even if per BC capability is indicated for a BC associating the band.

Proposal 3: per BC capability can be indicated when at least one band in a BC indicates corresponding per band capability.

Proposal 4: If per BC capability is absent for a BC, per band capability is applied for the BC.

Proposal 5: if proposal 1-4 is agreed, RAN2 agree to add high level description for per band and per BC capabilities in TS38.306.

Proposal 6: if proposal 1-4 is agreed, RAN2 agree to send an LS to RAN1 to check if the assumptions are correct for per band and per BC capabilities that they introduced.

Proposal 7: RAN2 agree to ask RAN1 to confirm if it is correct to assume that pre-requisite for per band and per BC capabilities is defined for per band and per BC separately.

# Reference

[1] R2-2004983    Report of email discussion [Post109bis-e][962][TEI16] Under-reporting CSI-RS Capabilities     NTT DOCOMO, INC.         report     Rel-16    NR_newRAT-Core, TEI16  Late