3GPP TSG-RAN WG2 Meeting #131                                    	R2-2505539

Bengaluru, India, 25th - 29th August 2025

Agenda item:	8.17

Source:	Qualcomm Incorporated

Title:	Remaining issues for new NB-IoT NTN TDD mode

Document for:	Discussion and Decision

- Introduction

In RAN2#131 meeting, following agreements were made.

Agreements:

1.	The SI-message transmission can be postponed to the next valid D frame within the SI-Window

2.	It is up to NW implementation to avoid SI-window overlap

3.	SI repetitions will not overlap (in case of collision the subsequent SI repetition is postponed)

4.	In IoT-NTN TDD mode, the RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted (i.e. no spec change)

5.	For the timer of ra-ResponseWindowSize and mac-ContentionResolutionTimer, the absolute value limitation for FDD (i.e., 10.24s) is used for IoT NTN TDD.

6.	In IoT-NTN TDD mode the same formula as for RA-RNTI calculation for FDD is reused

7.	No extension is needed on the value range of timer in unit of ms or s for IoT NTN TDD

8.	The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs.

9.	It is up to network to configure the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13 (no spec impact)

10.	Introduce the following definition for IoT-NTN TDD mode in the impacted RAN2 specifications:
IoT-NTN TDD mode: allows use of NB-IoT channels with TDD mode for NTN with fixed values of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (can revisit this based on the TP being prepared by RAN1)

11.	In Rel19, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

12. In IoT_NTN_Ph3-Core, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode. RAN2 understands that, as part of the IoT_NTN_TDD WI, we can discuss on a case by case basis whether minor specific enhancements – not affecting other WGs - can be supported to ensure that (some of) the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

In this document, we provide details of remaining issues that are not resolved over email discussion.

- Discussion

In the NB-IoT NTN TDD mode, a new DL frame/UL frame periodicity of 9 SFN is used (i.e., current H-SFN duration of 1024 SFN is not multiple 9 SFN). RAN1 has agreed that the downlink subframes within D=8 are fixed to: subframes [3 4 5 6 7 8 9 0] (across two consecutive radio frames), i.e., 82ms out of 90ms is not used for downlink.

Epoch time

Followings are agreements to handle the SIB reception considering the new frame format.

1.	The SI-message transmission can be postponed to the next valid D frame within the SI-Window

2.	It is up to NW implementation to avoid SI-window overlap

3.	SI repetitions will not overlap (in case of collision the subsequent SI repetition is postponed)

When SIB31 is postponed to the next valid subframe, the question is whether the epoch time (explicit or implicit) included in SIB31 will also be postponed. For implicit case, the epoch time is the end of SI window where the message is received. Since we do not expect to move the SI window, this should be no issue.

However, when explicit epoch time is indicated in SFN and subframe, it cannot be in past for the serving cell. When the scheduled repetitions are postponed and epoch time was set at the end of the scheduled repetitions, then the epoch time could be in the past. Therefore, it should be clarified that the explicit epoch time is associated with the SFN where SIB1 was originally scheduled.

Clarify that the explicit epoch time is associated with the SFN where SIB1 was originally scheduled to be transmitted.

Collision due to postponed transmission

Repetitions may be larger than 8 DL subframes depending on configuration npdcch-NumRepetitionPaging-r13. In this case, multiple valid DL SFNs would be needed for a given UE. Due to SI message scheduling or resource unavailability in such as low duty cycle scenario, network may postpone the paging message delivery to the next PO, which is at least 1.28s later in NB-IoT as minimum paging DRX cycle is 1.28s. We think this can be discuss if the paging is postponed to the next valid DL subframe and that valid DL subframe has no PDSCH resource available (e.g., collided with scheduled downlink transmission), then how to minimize the paging delivery delay.

For example, POs are determined based on UE’s ID. When a PO is postponed to the next valid subframe, that subframe may be used for SI. This leads to further postponing to the next free DL subframe. During this process, another UE’s PO may also be postponed leading to congestion. In addition, network might have already scheduled DL transmissions occupying the resource which might lead to delaying the paging message to next PO.

One solution is to allow network to configure UE with a number of valid DL SFNs (not subframe) to monitor the paging. Suppose the UE postpones the paging occasion to next valid subframe of the SFN x, then UE may be configured to monitor paging in both valid subframe of SFN x and SFN x+1. In this case, network may choose to start the paging notification in SFN x or SFN x+1 depending on DL resource availability.

To avoid paging delay due to collision of postponed paging transmission with the scheduled DL transmission, UE is configured whether to monitor paging in next valid DL frames of next SFN X and SFN X+1 (if no paging in SFN X).

Similarly, when SPS or PUR occasion is postponed to the next valid frame, multiple UE’s UL transmission may collide. This would lead to interference and failure of the transmission. We think the simple solution is to interpret current values of the SPS and PUR periodicities as new values that are multiple of 90ms.

For example, the value sf128 of semiPersistSchedIntervalUL-r15 can be interpreted as value sf180 in NB-IoT TDD mode.

For SPS interval, the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

Measurements

During the invalid DL period (i.e., last 82ms out of 90ms), it may need to be clarified that UE is not required to perform any IDLE mode tasks such as, Paging monitoring, Serving cell measurement, Logged measurements, Neighbor cell measurements. UE’s regular serving cell measurements may need to be postponed to the next valid DL subframes. We may need to check with RAN4 for the potential serving cell or neighbor cell measurement relaxation.

First, for neighbor cell measurements, UE may need to know neighbor’s timing information such that it knows when the neighbor’s invalid DL SFN duration is. A NB-IoT NTN cell operating in TDD mode may need to be deprioritized as there may be NB-IoT cells operating in normal FDD mode. As NB-IoT does not support frequency priority, the UE may select TDD mode cell over FDD cell. Therefore, we think first a new list of NB-IoT neighbor cells operating in TDD mode should be introduced. Then we should consider additional assistance information such as neighbor’s SFN offset for the neighbor cell measurements or cell reselection.

As NB-IoT does not support frequency priority, the UE may select TDD mode cell over FDD cell.

Introduce a new list of neighbor cells operating in TDD mode for measurements and cell reselection.

Discuss how to provide the UEs with timing information of neighbor cell that is operating in NB-IoT TDD mode for measurements.

- Conclusion

Following proposals are made:

Proposal 1	Clarify that the explicit epoch time is associated with the SFN where SIB1 was originally scheduled to be transmitted.

Proposal 2	To avoid paging delay due to collision of postponed paging transmission with the scheduled DL transmission, UE is configured whether to monitor paging in next valid DL frames of next SFN X and SFN X+1 (if no paging in SFN X).

Proposal 3	For SPS interval, the values of semiPersistSchedIntervalUL-r15 are interpreted as new values that are multiple of 90ms.

Observation 1.	As NB-IoT does not support frequency priority, the UE may select TDD mode cell over FDD cell.

Proposal 4	Introduce a new list of neighbor cells operating in TDD mode for measurements and cell reselection.

Proposal 5	Discuss how to provide the UEs with timing information of neighbor cell that is operating in NB-IoT TDD mode for measurements.