3GPP TSG-RAN WG2 Meeting #131	R2-2505559

Bengaluru, India, 25 - 29 August, 2025

Agenda item:	8.11.2

Source:	Samsung

Title:	Discussion on the open issues in Random Access

Document for:	Discussion & Decision

Introduction

In this paper, we provide our input for the remaining MAC and RRC open issues.

Discussion

[MAC-2-1] Do companies think any additional MAC specification changes (compared to the latest version of the MAC running CR) are needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA?

For the CFRA operation, our understandings are:

1) When CFRA resource is provided, for each preamble transmission (re)attempt, UE will evaluate whether CFRA criteria (described in clause 5.1.2) are fulfilled or not, and if fulfilled, perform CFRA, otherwise perform CBRA fallback.

2) In this case, regarding the CFRA criteria (described in clause 5.1.2), they only pertain to which preamble index, and SSB/CSI-RS to use (belonging to CFRA resource or CBRA resource), without involving the change of the set of Random Access resources, i.e., the PRACH occasions are selected from the same set of Random Access resource (selected in clause 5.1.1b), regardless of CFRA or CBRA fallback, throughout the entire RACH procedure.

That is, the set of Random Access resources is selected in clause 5.1.1b at initial stage, and after that, there is no procedure text to mandate UE to switch the set of Random Access resources when performing CBRA fallback from CFRA.

Based on the above understanding, it seems no further MAC spec change is essential, since, by default, CFRA and CBRA fallback thereof are always using the same set of Random Access resources of the same RO type throughout the entire RACH procedure.

[MAC-2] Proposal 1: No further MAC spec change is needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

[MAC-3-1] Do companies think any additional specification changes (in MAC and/or RRC) are essential to address the issue on how to derive the SBFD RO locations for CFRA (ZTE’s concern)? Please provide your input with the rationale.

[MAC-3-2] If so, please provide your preferred solution with the rationale, including but not limited to:

- Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA (proposed by LGE).

In TS 38.213 running CR [1], the SBFD RO is referred to as second PRACH occasions, and the definition of the second PRACH occasions is captured below.

TS 38.213 running CR 8 Random access procedure … - second PRACH occasions, that are in RBs that are both in the active UL BWP and in the UL sub-band, and associated either only with SBFD symbols that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon when the UE is provided either sbfd-RACHSingleConfig or sbfd-RACHDualConfig, or start from an SBFD symbol and end in a non-SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes

Note that SBFD RO locations can be derived differently depending on whether sbfd-RACHDualConfig-ValidROAcrossSymbolTypes is configured (meaning sbfd-RACHDualConfig is configured) or not. It means that, if RO type is indicated as SBFD RO for CFRA, UE should know whether sbfd-RACHDualConfig-ValidROAcrossSymbolTypes is configured or not, for the SBFD RO location derivation. We think there are two options that can be considered for resolving how UE will know the configuration.

Option 1) Introduce a restriction that network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA, and apply the same configuration for CFRA SBFD RO location derivation.

Option 2) Introduce additional signalling (besides the RO type signalling) in CFRA configuration, to indicate which SBFD RACH configuration should be applied for SBFD RO location derivation for CFRA.

For Option 1), it may entail less spec impact, but also less flexibility for network configuration and SBFD RACH resource management. For Option 2), it may entail more spec changes, but higher flexibility for network configuration and SBFD RACH resource management. Considering the time budget before WI completion, we think Option 1 is more preferable to resolve the issue with minimal spec impact.

[MAC-3] Proposal 2: To resolve the ambiguity on SBFD RO location derivation for CFRA, RAN2 to consider one of the following two workable options, where Option 1 is more preferable from the spec impact perspective:

- Option 1: Introduce a restriction that network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA, and apply the same configuration for CFRA SBFD RO location derivation.

- Option 2: Introduce new RRC signalling to indicate which SBFD RACH configuration should be applied for SBFD RO location derivation for CFRA.

[MAC-4-1] Do companies think the parameter initialization is needed after RO type switching in the case of RACH configuration Option 2? Please provide your input with the rationale.

[MAC-4-2] If so, please further clarify whether all parameters in RACH-ConfigCommon should be considered for the initialization, or if certain parameters should be excluded from the initialization, along with the rationale.

After the change of the RA resource set, caused by RO type switching, the RA parameters should be updated based on the new RACH-ConfigCommon in the case of SBFD RACH configuration option 2 (option 1, always share the same RACH-ConfigCommon between the two RO types), since the RA parameters are configured per RO type, and possibly network may have different preferences for the parameter values between the two RO types.

While, preambleTransMax should not be updated (keep using the parameter configured for the former RO type) when performing RO type switching. Because if the number of preamble transmission attempts when performing RO type switching has already exceeded the newly updated value of the preambleTransMax (configured for the latter RO type), the UE will miss the condition of RACH failure after the RO type switching. Note that the RACH failure condition (captured below) includes only the case that the number of preamble transmission attempts is “equal to” the preambleTransMax + 1, rather than embracing the “greater than” case.

TS 38.321 (v18.6.0) - 5.1.4 Random Access Response reception … 2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> if the Random Access Preamble is transmitted on the SpCell: 4> indicate a Random Access problem to upper layers; 4> if this Random Access procedure was triggered for SI request: 5> consider the Random Access procedure unsuccessfully completed. 3> else if the Random Access Preamble is transmitted on an SCell: 4> consider the Random Access procedure unsuccessfully completed.

[MAC-4] Proposal 3: Parameter initialization should be performed after RO type switching.

[MAC-4] Proposal 4: preambleTransMax should be excluded from the parameter initialization.

[MAC-5] Do companies support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching? Please provide your input with the rationale.

In RACH configuration Option 2, power ramping steps for SBFD RO and non-SBFD RO are configured separately by two ‘RACH-ConfigCommon->RACH-ConfigGeneric’, respectively, resulting in possibly different power ramping steps for different RO types. If network has different preferences for the power ramping steps between the two RO types, a power offset introduced for compensating the power ramping difference (between two RO types) is useful to prevent sudden change of the preamble transmission power, following the same rationale behind the power offset compensation in 2-step to 4-step RA type fallback.

[MAC-5] Proposal 5: A power offset is introduced to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for the RO type switching.

[RRC-6] Whether to support SBFD for LTM.

To support SBFD RO in LTM cell switch, RAN3 specification impact should be considered, since it is more reasonable for target cell to decide which RO type the UE should use in LTM cell switch, and hence, RAN3 signalling (from target cell to source cell) for the preferred RO type is required for such purpose, which should be discussed and addressed in RAN3 (cross-WG issue), if RAN2 agrees to support SBFD RO in LTM cell switch.

Also, TCI state is provided in LTM cell switch command MAC CE, which will be used by UE in target cell after switching to the target cell. This TCI state is supposed to be determined based on L1 measurement report based on reference signal of the target cell. While, considering the reference signal is not of SBFD symbol, whether the TCI state can be used with SBFD symbols after switching to the target cell should be discussed and relevant agreement should be made in other WI (cross-WI issue).

The discussions on the issues above (involving cross-WG issue and cross-WI issue) are time-consuming. Considering the time budget before the WI completion, RAN2 should not consider integrating SBFD into LTM cell switch operations.

[RRC-6] Observation 1: Integrating SBFD with the LTM cell switch requires additional cross-WI and cross-WG discussions, for which there is insufficient time, given the timeline for WI completion.

[RRC-6] Proposal 6: RAN2 to not consider integrating SBFD into LTM cell switch operations.

[RRC-7] Whether to support SBFD with NR-DC.

The issue of whether SBFD can be supported in NR-DC entails RAN4 specification impact. In the previous discussion, RAN4 has agreed the measurement (CLI and other measurements) for SBFD. However, RAN4 has not discussed/agreed on the measurements for SBFD in PSCell or SCell. If RAN2 decides to support the simultaneous configuration of SBFD and NR-DC, further discussion and agreement on PSCell measurements (e.g., RLM, BFD) will be required in RAN4, which would necessitate sufficient time budget.

Considering the current timeline, we believe the allocated time for RAN4 is insufficient to complete all relevant discussions, especially if RAN2 sends an LS to RAN4 in the next meeting.

[RRC-7] Observation 2: Simultaneous configuration of SBFD and NR-DC requires additional RAN4 discussions on PSCell and SCell measurements for SBFD, for which there is insufficient time, given the timeline for WI completion.

[RRC-7] Proposal 7: RAN2 to not support simultaneous configuration of SBFD and NR-DC.

Conclusion

Based on the discussion above, we have the following proposals:

[MAC-2] Proposal 1: No further MAC spec change is needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

[MAC-3] Proposal 2: To resolve the ambiguity on SBFD RO location derivation for CFRA, RAN2 to consider one of the following two workable options, where Option 1 is more preferable from the spec impact perspective:

- Option 1: Introduce a restriction that network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA, and apply the same configuration for CFRA SBFD RO location derivation.

- Option 2: Introduce new RRC signalling to indicate which SBFD RACH configuration should be applied for SBFD RO location derivation for CFRA.

[MAC-4] Proposal 3: Parameter initialization should be performed after RO type switching.

[MAC-4] Proposal 4: preambleTransMax should be excluded from the parameter initialization.

[MAC-5] Proposal 5: A power offset is introduced to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for the RO type switching.

[RRC-6] Observation 1: Integrating SBFD with the LTM cell switch requires additional cross-WI and cross-WG discussions, for which there is insufficient time, given the timeline for WI completion.

[RRC-6] Proposal 6: RAN2 to not consider integrating SBFD into LTM cell switch operations.

[RRC-7] Observation 2: Simultaneous configuration of SBFD and NR-DC requires additional RAN4 discussions on PSCell and SCell measurements for SBFD, for which there is insufficient time, given the timeline for WI completion.

[RRC-7] Proposal 7: RAN2 to not support simultaneous configuration of SBFD and NR-DC.

Reference

[1] R1-2504970, Introduction of evolution of NR duplex operation: Sub-band full duplex (SBFD).