3GPP TSG-RAN WG2 meeting #131	R2-2505638

Bengaluru, India, Aug 25th – 29th, 2025

Source:		NEC

Agenda item:		8.7.4	Scheduling enhancements

Title:		Remaining issues on LCP and DSR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#130 meeting, agreements [1] on LCP and DSR enhancements for XR were made as below:

[MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization. (MAC-02) When PSI-based discarding is activated for a DRB, priority adjustment is applied only if there is at least one high importance packet in the buffer has a remaining time smaller than the threshold. (MAC spec is already aligned with this agreement). (MAC-13) When pdu-SetDiscard is configured, PDU sets should be treated as a whole in the LCP procedure with adjusted priority. Confirm the working assumption “No Bj enhancement is introduced” as an agreement. (PDCP-1, RLC-5) Both PDCP and RLC consider Control PDU and/or retransmitted data into the shortest configured reporting threshold. (PDCP-1, RLC-5) The value of the remaining time field in the enhanced DSR MAC CE is set to 0, if there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold of the LCG. (PDCP-1, RLC-5) An understanding is that there will be no DSR with no data indication (i.e. indicating only volume of C-PDU and/or retransmissions for any LCG) (RRC-7) If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold. New LCID is used for R19 DSR (MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR. (MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

In this contribution, we will provide our views on the remaining issues for LCP and DSR procedure.

# 2. Discussion

2.1 LCP enhancements for priority-adjusted data.

The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

In the legacy LCP process, logical channels with equal priority should be served equally. However, with the enhanced delay-awareness LCP, there's a need to discuss whether an LCH with priority-adjusted data should be served before one without such data when both have equal priority. To ensure the timely transmission of delay-critical data, we prefer to serve the LCH with priority-adjusted data first in this scenario.

Proposal 1: RAN2 to discuss whether an LCH with priority-adjusted data should be served before one without such data during LCP when both have equal priority.

2.2 Remaining issues of DSR.

Issue [MAC-1]: Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported.

In Rel-18, the definition of a PDCP SDU associated with a DSR was introduced. The definition utilizes the concept of a delay-critical PDCP SDU to determine whether a pending DSR should be cancelled — specifically, cancellation occurs when no delay-critical PDCP SDU remains.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

With the introduction of concept of delay-reporting PDCP SDU for Multiple Entry DSR, RAN2 should also discuss whether the cancellation condition for Multiple Entry DSR is similarly based on the absence of delay-reporting PDCP SDU — specifically, when no delay-reporting PDCP SDU remains or all related information has been reported. However, in the running MAC CR, the definition of a PDCP SDU associated with a DSR is still applied to Multiple Entry DSR. A specific scenario highlights the potential issue with using the definition: Suppose the largest configured threshold in dsr-ReportingThresList is greater than remainingTimeThreshold. In such a case, there may be data whose remaining time falls into the range above remainingTimeThreshold but below the largest configured reporting threshold. If the current definition is applied — and there is no data with a remaining time below remainingTimeThreshold — the Multiple Entry DSR will be cancelled, even though there still exist delay-reporting PDCP SDUs with remaining time above remainingTimeThreshold.

Observation 1: According to current running CR, a Multiple Entry DSR will be cancelled when no delay-critical PDCP SDUs remain, even though there still exist delay-reporting PDCP SDUs with remaining time above remainingTimeThreshold.

Therefore, the key issue here is whether the information of delay-reporting PDCP SDU with remaining time above remainingTimeThreshold should still be reported when there is no delay-critical data remaining. There are two alternatives to handle it:

- No delay-critical PDCP SDU, no Multiple Entry DSR, as specified in current running CR.

- No delay-reporting PDCP SDU, no Multiple Entry DSR. That is, no data associated with the Multiple Entry DSR left.

Regarding alternative 2, we believe it’s valuable to report this information since Multiple Entry DSR itself covers this range of data. Moreover, it is reasonable to cancel a DSR only when there is no data associated with it. Although a Multiple Entry DSR may still be triggered later when the remaining time falls below the remainingTimeThreshold, why should we postpone a highly probable DSR trigger and introduce an additional “cancellation -> trigger” process? Since the Multiple Entry DSR has already been triggered, the UE could provide this information to the network early for scheduling purposes. If alternative 2 is adopted, RAN2 should discuss whether to revise the definition of a PDCP SDU associated with a DSR, or introduce a new definition of a PDCP SDU associated with a Multiple Entry DSR. For example,

The definition of a PDCP SDU associated with a DSR may be updated as below:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU or delay-reporting PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

Or a PDCP SDU associated with a Multiple Entry DSR may be defined as below:

A PDCP SDU is considered to be associated with a Multiple Entry DSR if it has not been transmitted in any MAC PDU and is a PDCP SDU with remaining value of the running PDCP discardTimer below the largest configured reporting threshold and associated with the logical channel which triggered the DSR.

Proposal 2: [MAC-1] RAN2 discusses to revise the definition of a PDCP SDU associated with a DSR, or introduce a new definition of a PDCP SDU associated with a Multiple Entry DSR.

Issue [MAC-2]: Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

It was agreed in RAN2#129bis meeting that non-delay-reporting data ahead of delay-reporting data shall be counted in the buffer size calculation for the Multiple Entry DSR.

Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review)

Given that buffer size of both non-delay-reporting data and delay-reporting data may be reported by Multiple Entry DSR, there exists a scenario where the “delay-reporting buffer size plus non-delay-reporting buffer size” equals “total buffer size”. To avoid redundant buffer size reporting, for an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in Multiple Entry DSR equals the total buffer size. Simply adding a new BSR-cancellation condition would keep the change minimal and introduce little complexity.

Proposal 3: [MAC-2] For an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in Multiple Entry DSR equals the total buffer size (i.e., “delay-reporting buffer size + non-delay-reporting buffer size” equals “total buffer size”).

# Summary

This contribution provides our analysis on the enhancements of DSR & LCP procedure, and has the following proposals:

Proposal 1: RAN2 to discuss whether an LCH with priority-adjusted data should be served before one without such data during LCP when both have equal priority.

Observation 1: According to current running CR, a Multiple Entry DSR will be cancelled when no delay-critical PDCP SDUs remain, even though there still exist delay-reporting PDCP SDUs with remaining time above remainingTimeThreshold.

Proposal 2: [MAC-1] RAN2 discusses to revise the definition of a PDCP SDU associated with a DSR, or introduce a new definition of a PDCP SDU associated with a Multiple Entry DSR.

Proposal 3: [MAC-2] For an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in Multiple Entry DSR equals the total buffer size (i.e., “delay-reporting buffer size + non-delay-reporting buffer size” equals “total buffer size”).

# References

[1] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7th Apr – 11th Apr, 2025

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.5.0