3GPP TSG RAN WG2 Meeting #131			R2-2505655

Bengaluru, India, August 25- 29, 2025

Agenda Item: 	8.4.2

Source:		Sony

Title:			Disabling/enabling LP-WUS in RRC Idle/Inactive mode

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version for the WID for Low Power Wake-up Signal and Receiver was approved [1].

At RAN2#126 [2], the following agreements were made related to RRC Idle/Inactive mode:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

    - LP-SS configuration

    - LP-WUS configuration

    - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8

From RAN2 perspective, no new procedure is introduced for SI reception/updates.

Agreements made during RAN2#129 [3], related to Idle/Inactive:

On separate band issue RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec. Send LS to RAN1 and RAN4 to inform the agreements. On sub-grouping For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. On SA2 LS (R2-2500050/S2-2412876) RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology. Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

At RAN2#130 [4], the following agreements were made:

Agreements on RRC_IDLE INACTIVE procedure The entry/exit condition for LP-WUS monitoring is mandatory in LP-WUS configuration, if the LP-WUS configuration is provided by the NW. Full coverage for LP-WUS is not precluded, e.g., if there needs to be a threshold value so that the condition is always fulfilled for all LPWUS UEs. It is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types. RAN2 aim at supporting enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, if the solution can be concluded in the August meeting.

In this contribution, we discuss our view on some RAN2 aspects related to disabling and enabling of LP-WUS in RRC Idle/Inactive state.

# Disabling/Enabling LP-WUS in RRC Idle/Inactive

The operation of wake-up signal (WUS) and/or low power WUS (LP-WUS) is introduced in a 3GPP cellular network with the motivation to improve UE power consumption.

In the approach in the Rel-18 study and in the ongoing Rel-19 work-item, the LP-WUS signal is transmitted in a configured time occasion prior to the legacy Paging Occasion. If there is a Paging message for a certain UE, then the base-station transmits LP-WUS, and when the UE detects the LP-WUS, it will turn on its main receiver to monitor for paging reception. If no LP-WUS is detected then the UE’s main receiver remains turned off, i.e., the UE will not turn on the main receiver and does not monitor any paging occasion. Due to various reasons, there is a need to be able to turn off the usage of LP-WUS in certain areas, e,g,, for LP-WUS resource load issues or other.

At RAN2#130 meeting, it was agreed that RAN2 should aim to support enabling/disabling LP-WUS monitoring in IDLE/INACTIVE per UE.

In this paper, we propose a method for the Network to be able to both disable and enable the usage of LP-WUS without setting up a dedicated connection to the UEs.

And at RAN2#130 the following email discussion was initiated [5]:

[Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE (Huawei)

In the email discussion, different views are proposed, and mainly three directions were up for discussion.

- NAS signalling

- RRC signalling

- No NAS/RRC spec changes

  - Apple’s method

  - CATT’s method

and the following conclusions were made in, R2-250xxxx [Post130][222][LPWUS] Enabling Disabling LP-WUS per UE_v17_Rapp [6].

Observation (9/15): Many prefer to specify either NAS or RRC signalling to support enabling/disabling LP-WUS per UE.

Proposal: (RRC-12) RAN2 to down-select from NAS signalling or RRC signalling to support enabling/disabling LP-WUS per UE.

Hence, the UE is configured, while in RRC-Connected mode, by using e.g., the RRC Release message to configure the UE with LP-WUS configuration and whether the LP-WUS monitoring for that UE is enable or disabled.

The drawback with this alternative is that the UE has to be in contact with the network and has to use dedicated signalling to change its LP-WUS monitoring configuration.

Observation 1: A UE that is configured via dedicated signalling whether LP-WUS monitoring is enabled or disabled, can not alter its configuration while in Idle/Inactive mode.

Another alternative can be that the enabling/disabling LP-WUS monitoring is configured or indicated via System Information. This would save the device from entering connected mode there is a need or preference to change the monitoring configuration. There is a problem with signalling with specific UEs in Idle/Inactive mode. Basically the network would never address a specific UE via System Information, and if the network would like to reach a specific UE, it would use paging. Using paging would force the UE to enter Connected mode.

Likely the preference of using LP-WUS may change over time, and not necessarily only when the device is in Connected mode, it would be beneficial to be able to reconfigure the device while in Idle/Inactive mode.

One option would be to turn on or turn of LP-WUS usage in the entire cell for all devices, but that is a rather blunt instrument.

Observation 2 LP-WUS can easily be turned on of off for all devices in the entire cell via system information:

Since the main issue or background for being able to disable LP-WUS usage it network resources, the issues is broader than related to a single device, hence some group configuration seems more relevant.

And yes, wouldn’t  it be possible to indicate for a group of UEs to change their LP-WUS configuration., i.e., to e.g., either turn on or turn off LP-WUS monitoring.

E.g., if some groups based on UE-ID or CN subgrouping configuration could be configured via system information, to either enable or disable LP-WUS monitoring.

Proposal 1: Configure groups of devices to enable or disable LP-WUS monitoring by indicating those groups based in UE-ID or CN based subgrouping via System Information.

Another alternative of grouping devices, can be to use random number, lets say if the network likes to disable LP-WUS usage for  50% of the population, it can broadcast enable/disable by 50% and the UE just randomly calculates whether to disable LP-WUS.

Yet another option could be based on UE type or UE traffic pattern.

Proposal 2: The Network indicates via System Information a Random Number of UE´s to enable or disable LP-WUS.

Proposal 3: The Network indicates via System Information certain types of UEs or UEs with specific traffic patterns to enable or disable LP-WUS.

By making it possible to configure and reconfigure LP-WUS monitoring in Idle/Inactive, this would allow for supporting the mobility of using or supporting LP-WUS while making cell reselection, saving the UEs from entering connected mode, and allowing the network to change the LP-WUS configuration without establishing a connection to the devices.

Proposal 4: Introduce support for Idle/Inactive mode mobility using LP-WUS.

Draft Text proposal for 38.331, DownlinkConfigCommonSIB

LP-SubgroupDisableEnableConfig-r19 ::=        SEQUENCE {

lp-SubgroupsForCNDisabled-r19         INTEGER (1.. maxNrofPagingSubgroupsLP-r19),

lp-SubgroupsForUEIDDisabled-r19       INTEGER (1.. maxNrofPagingSubgroupsLP-r19)                               OPTIONAL  -- Need S

...

lp-subgroupDisableEnableConfig Configuration via system information on Disabling and Enabling of specific LP-WUS subgroups

LP-SubgroupConfig field descriptions

lp-SubgroupsForCNDisabled Indicates the subgroups configured for CN-assigned subgrouping that is/are disabled

lp-SubgroupsForUEIDDisabled Indicates the subgroups configured for UE-ID -assigned subgrouping that is/are disabled

Proposal 5: Use the text proposal above as baseline for detailed definition of disabling/enabling LP-WUS via System Information.

# Summary

In this contribution, we have discussed our views on aspects related to subgrouping, entry/exit criteria and latency aspects.

Observation 1: A UE that is configured via dedicated signalling whether LP-WUS monitoring is enabled or disabled, can not alter its configuration while in Idle/Inactive mode.

Observation 2 LP-WUS can easily be turned on of off for all devices in the entire cell via system information:

Proposal 1: Configure groups of devices to enable or disable LP-WUS monitoring by indicating those groups based in UE-ID or CN based subgrouping via System Information.

Proposal 2: The Network indicates via System Information a Random Number of UE´s to enable or disable LP-WUS.

Proposal 3: The Network indicates via System Information certain types of UEs or UEs with specific traffic patterns to enable or disable LP-WUS.

Proposal 4: Introduce support for Idle/Inactive mode mobility using LP-WUS.

Proposal 5: Use the text proposal above as baseline for detailed definition of disabling/enabling LP-WUS via System Information.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- Minutes RAN2#126, Fukuoka

- RAN2#129, Athens

- RAN2#130, St Juliens

- [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE (Huawei)

- R2-250xxxx [Post130][222][LPWUS] Enabling Disabling LP-WUS per UE_v17_Rapp