3GPP TSG-RAN WG2 Meeting #131						R2-2505680

Bangalore, India  Aug 25th – 29th , 2025

Agenda item:		8.2.3

Source:			Lenovo

Title:		Remaining issues on random access for Ambient IoT

Document for:	 	Discussion and Decision

Introduction

In RAN#107 meeting the work item for Ambient IoT was revised and approved in [1]. In last RAN2 meeting, A-IoT random access procedure was discussed and reached following agreements:

Agreements - For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952). Capture a NOTE that other implementation are allowed. X, Y will be signalled by paging message - The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages. R2D trigger message is not sent in CFRA procedure. Come back if RAN1/4 sees any issues. Send LS to RAN1/RAN4 - FFS R2D byte alignment dependent on TBS size discussion Agreements on RA - Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) - A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. - Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. Agreements on NACK reception: - After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify. Agreements on RN16/AS ID maintainance: - Confirm a device is not expected to maintain both AS ID and RN16. After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader. This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission. How to capture device behavior is FFS

In this contribution, left open issues are discussed based on the [2].

Discussion

Issue 2-3: R2D trigger message byte alignment

It has been agreed that “A new R2D message (i.e., R2D trigger message) other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. RAN2 assumes that the R2D message does not include slot number/count down number.”. However, whether the R2D trigger message is byte-alignment or not has not reached a consensus.

With the agreements and assumptions, the R2D trigger message may be designed with short size. Considering the RAN1 agreement to support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only for the R2D transmission triggering random access, when the reader wants to indicate the start of a set of MSG1 resources, e.g., the reader initiates new access occasion for D2R transmission, or transmits new paging message/subsequent paging message, there will be follow up R2D trigger message, which indicates that the R2D trigger message occurs very frequently. In addition, based on the running MAC CR the R2D trigger message (can be also called access trigger message) only includes one R2D message type filed, and the length of the field is 3 bits. If the R2D trigger message is byte-aligned, the rest 5 bits may need to be set padding or reserved bits to guarantee the alignment, which will waste the radio resource. Hence, the R2D trigger message is designed to be bit-alignment instead of byte-alignment.

Proposal 1 (Issue 2-3): RAN2 confirms that the R2D trigger message is bit-alignment instead of byte-alignment.

Issue 2-4: CBRA failure detection

In last RAN2 meeting, it was discussed CBRA failure detection issue, that a device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further down-selected between option B and C below.

  - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

  - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

The difference between option B and option C is that how many R2D trigger message is counted by the device that expecting MSG2 e.g. for option B, one R2D trigger message is counted while for option C, K R2D trigger messages are counted. Between these two options, option C provides the reader the flexibility to respond MSG2, that the next R2D trigger message can be sent without waiting for the Msg1 processing and Msg2 preparation delay in the reader. This is important to avoid unnecessary procedure hanging since during the waiting for Msg2 response, the reader cannot send R2D trigger message and other device cannot send Msg1.

Observation 1: Option C provides reader flexibility to respond MSG2 and avoid unnecessary procedure hanging.

On the other hand, from device perspective, counting K R2D trigger message does not increase the device complexity much. The only difference is counting 1 or K R2D trigger message between Option B and C.

Observation 2: Option C does not increase device complexity much.

Proposal 2 (Issue 2-4): RAN2 to pursue option C for CBRA failure detection, that a device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary is the reception of either the kth R2D trigger message or the subsequent paging message

Issue 2-5: random ID differentiation in Msg2

On the other hand, in last RAN2 meeting, it was discussed and reached the consensus about the feasibility of including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible.  FFS whether to include it. RAN2 has discussed the collision rate that in Msg2 for one R2D trigger message. However, if RAN2 agreed Kth R2D trigger message for CBRA failure detection in above, such collision rate should be calculated and discussed again.

One Msg2 message contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. These echoed random ID(s) are for those A-IoT devices that has successful contention resolution in specific Msg1 resource. And it is possible that two or more echoed random ID(s) corresponding to different access occasions/Msg1 resources are same in one Msg2 message, which means RN16 collides in the Msg2.

In last RAN2 meeting, this issue was discussed and there has comments that the probability of RN16 collision in Msg2 is very low. We calculate the collision probability in the below:

Assuming there have N RN16 that responded in one Msg2. Then there are totally (216)N different combinations since each RN16 has 216 possible values. For the first RN16, there totally have 216 possible values. For the second RN16, to different with the first RN16, there totally have 216-1 possible values. And for the Nth RN16, to different with all the other N-1 RN16, there totally have 216-(N-1) possible values. So, the probability P0 that all N RN16 are different in one Msg2 is as following

P0=216*216-1*216-2*…*(216-N+1)(216)N

Then in one Msg2, the probability P for at least two RN16 are same is as following

P=1-P0=1-216*216-1*216-2*…*(216-N+1)(216)N

Assuming time domain resource X=2 and frequency domain resource Y=8, for one R2D trigger message case, N=16 and the max collision rate P=1.8‰. On the other hand, for 4 R2D trigger message case, N=64 and the max collision rate P=3.04% which is almost 17 times than one R2D trigger message case. And this collision rate cannot be neglected anymore, and including frequency index to decrease the collision rate is necessary.

Proposal 3 (Issue 2-5): RAN2 confirms to include frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices.

Issue 2-6: number indication of echoed random IDs

As the running CR implemented, Msg2 may include multiple entries, and each entry includes random ID, AS ID indication and AS ID if assigned. The device receives Msg2, and just decodes the entry one by one till the end of the message or till its Msg2 response is found. In this case, there is no need to including additional number indication for entries.

Proposal 4 (Issue 2-6): RAN2 confirms there is no need to indicate the number of random ID entry in Msg2.

Conclusion

In this contribution we have discussed A-IoT random access procedure and made the following observations and proposals:

Proposal 1 (Issue 2-3): RAN2 confirms that the R2D trigger message is bit-alignment instead of byte-alignment.

Observation 1: Option C provides reader flexibility to respond MSG2 and avoid unnecessary procedure hanging.

Observation 2: Option C does not increase device complexity much.

Proposal 2 (Issue 2-4): RAN2 to pursue option C for CBRA failure detection, that a device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary is the reception of either the kth R2D trigger message or the subsequent paging message

Proposal 3 (Issue 2-5): RAN2 confirms to include frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices.

Proposal 4 (Issue 2-6): RAN2 confirms there is no need to indicate the number of random ID entry in Msg2.

Reference

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- [POST130][027][AIoT] MAC Running CR (Huawei)