3GPP TSG-RAN WG2 Meeting #131	R2-2505683

Bangalore, India, Aug.25th – 29th, 2025

Agenda item:		8.4.3

Source:		Lenovo

Title:	Open issues on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:		Discussion and Decision

- Introduction

In RAN#103 meeting, WID of Rel-19 Low-power wake-up signal and receiver for NR (LP-WUS/ WUR) [1] was discussed and agreed, the objectives related to RRM measurement for RRC IDLE/INACTIVE modes of the work items are given as following.

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In the previous RAN2 meetings [2]-[6], RRM measurement relaxation and offloading in RRC_INACTIVE/IDLE state has been discussed and following agreements have been reached:

RAN2#126 Agreements: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. RAN2#127 Agreements： - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis Agreements: Working assumption: - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 Agreements: - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbor cell RRM measurement relaxation. RAN2#129 Agreements： - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds. RAN2#129bis Agreements: - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progresses, if any). - RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both. - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. - The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. - How to define LP-RSRP and LP-RSRQ is up to RAN1. - The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review. - Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). RAN2#130 Agreements: - The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. - It is up to NW to configure the condition for LP-WUS monitoring and/or [R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading], as in the current RRC running CR. - The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

This contribution aims to present our views on the open issues related to RRM measurement relaxation and offloading from MR to LR for UEs in RRC_IDLE/INACTIVE mode.

- Discussion

Open issue (RRC-7/38304-3): FFS on exit condition for serving cell RRM measurement relaxation.

For serving cell measurement relaxation, MR performs relaxed measurement on serving cell and neighboring cell while LR is ON and performs the serving cell measurement, which can also be defined as the partial offloading case or relaxation case. Regarding to the entry conditions for the relaxation case, in RAN2 #129 meeting, it has been agreed that the entry conditions for serving cell RRM relaxation can include both MR and LR measurements, if LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds. While for the exit conditions for serving cell measurement relaxation, i.e., MR stops performing the measurement relaxation on serving cell and fall backs to legacy RRM measurement, LR also stops performing the measurement on serving cell. In previous meetings, companies had discussion on whether additional exit condition is needed in addition to ‘not fulfilling the entry condition’. Some companies proposed additional exit condition is needed to avoid ping pone problem. However, in our understanding, in legacy neighboring cell measurement relaxation, the ping pone issue also exits but no impacts on the measurement, even for UE capable with LP-WUR, ping-pone issue also has no impacts on the measurement since LR is always on. Additionally, different with offloading case that only LR measurement result is available, both MR and LR measurement results are available for the relaxation case, NW may also include both MR and LR measurement results into the entry conditions by NW configuration. It makes sense to follow legacy neighboring cell measurement relaxation principle, i.e., use ‘not fulfilling the entry conditions’ as the exit condition and no additional exit conditions are needed for serving cell measurement relaxation.

Proposal 1: (RRC-7/38304-3) RAN2 to agree that ‘not fulfilling the entry condition’ as the exit condition for serving cell measurement relaxation.

Open Issue (RRC-10/38304-6): FFS on low mobility criteria

As defined in RAN4#114 meeting, the scaling factor of MR RRM relaxation for serving cell/neighboring cell is 16, the requirement is quite different with Rel-16 neighboring cell measurement relaxation (e.g., the scaling factor is 2-4), which means that separate conditions for Rel-19 neighboring cell measurement relaxation for LP-WUS UE needs to be defined. In RAN2#127bis meeting, RAN2 assumed that for neighboring cell measurement relaxation for UEs capable of LP-WUS, MR based criterion on ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. In our understanding, ‘low mobility’ criterion guarantees that the UE has low possibility to perform the cell reselection, the neighboring cell measurement can then be relaxed. Same principle can still be applied in LP-WUS. Since for LP-WUS UE, potential cell switch due to high mobility may further lead to the UE to switch between MR and LR more frequently, which will reduce the power saving gain. It is suggested that ‘low mobility’ criterion can be used in Rel-19 neighboring cell measurement relaxation for UEs capable of LP-WUS in addition to ‘not at cell edge’ criterion. Network may configure the low mobility criterion optionally in addition to ‘not-at-cell-edge’ criterion.

Proposal 2: (RRC-10/38304-6) RAN2 to agree that ‘low mobility’ criterion can be used for Rel-19 neighboring cell measurement relaxation for UEs capable of LP-WUS.

Furthermore, in RAN2#129bis meeting, RAN2 has discussed and agreed to merge the entry/exit condition for serving cell RRM measurement relaxation and Rel-19 neighboring cell RRM measurement relaxation. We understood that merging those two conditions which refers to UE can trigger both serving cell measurement relaxation and neighboring cell measurement relaxation simultaneously by using one unified condition, e.g., based on the same MR and LR threshold (if configured). As further clarified during online discussions, this principle currently applied to the criterion with ‘not at cell edge’, whether and how to apply to other criterions if supported is another issue. Based on our proposal 2, if ‘low mobility’ criterion has been applied to Rel-19 neighboring cell measurement relaxation, we understood that same criterion also needs to be applied to the serving cell measurement relaxation. It is reasonable since there is a relatively high possibility for UE with high mobility to perform cell reselection while needs normal measurement for both serving cell and neighboring cell.

Proposal 3: (RRC-10/38304-6) If ‘low mobility’ criterion has been agreed to apply for Rel-19 neighboring cell measurement relaxation, same criterion also needs to be applied to the serving cell measurement relaxation.

Additionally, for serving cell measurement offloading case, current agreements defined that the entry conditions for serving cell measurement offloading are at least MR result greater than a certain RSRP threshold, and LR result could also be considered. The agreed entry condition aims to guarantee the UE with a good coverage performance for both MR and LR. However, the serving cell measurement offloading not only aims to achieve UE power saving gain but also needs to limit the impacts on mobility performances as much as possible. For this purpose, the UE needs to at least stay in a relatively stable status and have a low possibility of performing cell (re)selection, then the serving cell measurement offloading from MR to LR could be considered, since LR cannot perform neighboring cell measurement. For example, UE with high-mobility camps on small cell may not be suitable to perform offloading even with large measured RSRP value. To achieve this, the low mobility criterion may also be applied to the serving cell measurement offloading case, which can be defined as the difference of RSRP value being lower than a (pre-)configured threshold within a time duration, and MR measurements are used for the entry condition evaluation for low mobility criterion.

Proposal 4: (RRC-10/38304-6) ‘Low mobility’ criterion can be used as one of the entry conditions for serving cell measurement offloading case, which is evaluated by the MR.

- Conclusion

Proposal 1: (RRC-7/38304-3) RAN2 to agree that ‘not fulfilling the entry condition’ as the exit condition for serving cell measurement relaxation.

Proposal 2: (RRC-10/38304-6) RAN2 to agree that ‘low mobility’ criterion can be used for Rel-19 neighbouring cell measurement relaxation for UEs capable of LP-WUS.

Proposal 3: (RRC-10/38304-6) If ‘low mobility’ criterion has been agreed to apply for Rel-19 neighboring cell measurement relaxation, same criterion also needs to be applied to the serving cell measurement relaxation.

Proposal 4: (RRC-10/38304-6) ‘Low mobility’ criterion can be used as one of the entry conditions for serving cell measurement offloading case, which is evaluated by the MR.

- References

3GPP RAN#103, RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

3GPP RAN2#126, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#127/17bis, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#128, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#129/129bis, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#130, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.