3GPP TSG-RAN WG2 Meeting #131	R2-2505693

Bangalore, India  Aug 25th – 29th, 2025

Agenda item:	8.19.2

Source:	Lenovo

Title:	Discussion on PL offset in the case of LTM

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

1	Introduction

During the MAC open issues discussion, it was mentioned whether to support Rel-19 sDCI mTRP 2TA with PL offset for Rel-18 LTM cell switch. If it is supported, how to indicate the up-to-date PL offset for LTM cell switch?  And Rapporteur suggested to discuss the related issues in TEI. In this document, we discuss the remaining issues.

2	Discussion

As the sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact e.g. for the serving cell configured with 2TA, or for the candidate cell configured with two TAG IDs and one TAG ID indicated to the UE to perform a RACH-less LTM cell switch. It is natural to support asymmetric DL sTRP and UL mTRP as 2TA case. Namely, the asymmetric DL sTRP and UL mTRP with PL offset can be configured in LTM candidate cell configuration and the configuration is up to network implementation.

Proposal 1: Support sDCI mTRP 2TA with PL offset for Rel-18 LTM cell switch.

Considering the previous agreement on PRACH transmission, PL offset is applicable only to PDCCH-order CFRA. And the CFRA is supported in Rel-18 LTM. For CFRA LTM, PL offset corresponding to the target cell is needed if TCI state is associated with the TAG with PL offset.

Further, for RACH-less LTM, the UE accesses the target cell using either a configured grant or a dynamic grant via DCI. The configured grant is provided in the LTM candidate configuration, and the UE selects the configured grant occasion associated with the TCI state indicated in the cell switch command. If the TCI state is associated with TAG with PL offset, PL offset should be indicated as well.

To support the cell of asymmetric DL sTRP and UL mTRP as a target cell in the case of LTM, the corresponding PL offset should be indicated in the cell switch command.

Proposal 2: To support the cell of asymmetric DL sTRP and UL mTRP with PL offset as a target cell in the case of LTM cell switch, the PL offset value should be indicated in the LTM cell switch command.

3	Conclusion

In summary, we have following proposals for PL offset in LTM cell switch:

Proposal 1: Support sDCI mTRP 2TA with PL offset for Rel-18LTM cell switch.

Proposal 2: To support the cell of asymmetric DL sTRP and UL mTRP with PL offset as a target cell in the case of LTM cell switch, the PL offset value should be indicated in the LTM cell switch command.

4	References

[1] RAN2#130 chairman notes.

[2] [Post130][219][MIMO_Ph5] Running CR for 38.321 (Samsung).