3GPP TSG-RAN WG2#131											   R2-2505707

Bengaluru, IN, Aug. 25th – 29th, 2025

Agenda Item:	8.8.2

Source: 	NEC

Title:	        Clarification on downlink coverage enhancement

Document for:	 Discussion

# Introduction

At the last meeting [1], RAN2 made the following agreement regarding idle mode SMTC enhancement:

1. the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue) 2. To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications) 3. We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

Besides, RAN1 sent an LS to RAN2 about Msg4 PDSCH repetition as following [2].

As part of the WI on NR NTN enhancement phase 3, RAN1 has agreed to support for Msg4 PDSCH repetition. The following agreement has been made regarding the activation/deactivation of Msg4 PDSCH repetition. Agreement For the activation/deactivation of Msg4 PDSCH repetition: - Alt 1: UE specific PDSCH with Msg4 repetition activation indicated via DCI Format 1_0: - Signaling uses re-interpretation of 1 MSB in MCS field in DCI. - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH. - Note: RAN1 considers there is no difference between capability and request - FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs. - The aggregation factor is configured in SIB1, with possible value 2 or 4 When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled. RAN1 kindly requests RAN2 to consider the above agreement when designing the report included in Msg3 PUSCH.

In this document, we would like to discuss two aspects:

- Share our preference on “FFS if also an SSB-index based SMTC selection is supported” in addition to location-based SMTC selection

- Share our concern on the agreement part “ each SMTC can be associated with a reference location of the intended neighbor cells”

- Share our views on whether to introduce the report condition and how to report for Msg4 PDSCH repetition capability/request.

# Discussion

## 2.1 SMTC

[open issue]: FFS if also an SSB-index based SMTC selection is supported

(serving cell) SSB-index based SMTC selection are shown in following figure. UE choose which SMTC to measure based on the selected SSB of serving cell (i.e. best SSB).

With option 2, the network provides SMTC configuration per SSB (i.e. per serving beam) rather than per cell.

Because SSB-based solution does not work for single beam scenario, RAN2 agreed to the location-based solution. We do have sympathy with SSB-based solutions, because it simply extends per serving cell to per serving beam configuration. It has better generalization to other types of beams forming deployments and having better future compatibility. It may also be simpler for UE implementation. On the other hand, we normally do not introduce two solutions for one issue, particularly for idle mode features.

Not to introduce the SSB-index based SMTC selection in addition to location-based solution.

New issue: Clarification on “ each SMTC can be associated with a reference location of the intended neighbor cells”

We made the above agreement, but we are not sure if it is workable considering different scenarios:

As follows, in Rel-17, smtc4list-r17 is introduced for SMTC configuration in a NTN cell, where each SMTC can be linked to a PCI list. This means typically one SMTC is linked to multiple neighboring cells. This should be the same even in Rel19 with beaming hopping.

SSB-MTC ::=   		SEQUENCE {

periodicityAndOffset	CHOICE {

sf5                 	INTEGER (0..4),

sf10                   	INTEGER (0..9),

sf20                   	INTEGER (0..19),

sf40                   	INTEGER (0..39),

sf80                   	INTEGER (0..79),

sf160                  	INTEGER (0..159)

},

duration        		 ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

smtc4list-r17     	SSB-MTC4List-r17                OPTIONAL,   -- Need R

SSB-MTC4List-r17::= SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

SSB-MTC4-r17 ::=	SEQUENCE {

pci-List-r17 		SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need M

offset-r17        	INTEGER (0..159)

}

When network configure the reference location for a SMTC:

- If one SMTC is associated with only one neighboring cell as below, reference location could be the cell center.

- However, if one SMTC is associated with multiple neighboring cells, all these cells together may provide an irregular or even discontinuous coverage area, it is almost impossible to provide a proper reference location.

Where should be the Reference location of a SMTC linked to two cells with red dots?

If one SMTC is configured for TN network measurement, it may be not needed to associate this SMTC with any reference location, but rather than relying on TN coverage assistance information. In this case, UE shall not down select this SMTC based on reference location

Reference location is equal to cell centre

Network provides reference locations of configured SMTCs

RAN2 discuss if it is workable to associate only one reference location with each SMTC, considering one SMTC may be linked to multiple cells (PCI list) measurement

In our opinion, it should be possible to associate one SMTC with zero, one or multiple neighboring cells’ reference locations to cover all cases mentioned above. Because one SMTC is also associated with a list of PCIs, we can either configure one reference location per PCI or a list of reference locations per SMTC.

## 2.2 Msg4 PDSCH repetition

[New issue for Msg4 PDSCH repetition from RAN1 LS]: discussion on whether to introduce a threshold and how to report UE capability/request in Msg3 PUSCH.

According to RAN1 LS [2], a UE capable of Msg4 PDSCH repetition may report its capability/request to the network. Regarding whether to specify a condition for the UE to report its capability/request is FFS. For this issue, considering the UE can measure the DL channel quality based on the received SSB, it is better to introduce a threshold to determine whether to request Msg4 PDSCH repetition, so that the network can properly schedule Msg4. When a threshold for Msg4 PDSCH repetition is configured and the RSRP of the DL pathloss reference is less than the configured threshold, a UE capable of Msg4 PDSCH repetition can report its request. If the threshold is not configured, the UE capable of Msg4 PDSCH repetition also reports its capability, similar to PUCCH repetition of Msg4 HARQ-ACK.

A threshold for Msg4 PDSCH repetition should be configured by SIB1.

A UE capable of Msg4 PDSCH repetition should report its capability/request:

- if both repetition number and threshold are configured and the RSRP of DL pathloss reference is less than the configured threshold, or

- if repetition number is configured and threshold is not configured

Besides, a UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH. For Msg3, since there are more reserved LCIDs when LX field is set to 1, it is natural to use LCID to report UE capability/request. Then, with LCID, the following two cases may occur, considering both capability/request for PUCCH repetition of Msg4 HARQ-ACK and capability/request for Msg4 PDSCH repetition are reported by the LCID of the CCCH message:

- Case 1: The UE reports capability/request of PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition simultaneously when two conditions for reporting PUCCH repetition and Msg4 PDSCH repetition are met.

- Case 2: The UE reports capability/request Msg4 PDSCH repetition when only the condition for Msg4 PDSCH repetition is met.

For the above two cases, the different LCIDs can be defined as shown in Table 1.

Table 1: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 | 216 + 328 | CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE |
| 9 | 216 + 329 | CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE |
| 10 | 216 + 330 | CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE |
| 11 | 216 + 331 | CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE |
| 12 | 216 + 332 | CCCH of size 48 bits for Msg4 PDSCH repetition of an eRedCap UE |
| 13 | 216 + 333 | CCCH of size 64 bits for Msg4 PDSCH repetition of an eRedCap UE |
| 14 | 216 + 334 | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition, except for an (e)RedCap UE |
| 15 | 216 + 335 | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition, except for an (e)RedCap UE |
| 16 | 216 + 336 | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of a RedCap UE |
| 17 | 216 + 337 | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of a RedCap UE |
| 18 | 216 + 338 | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of an eRedCap UE |
| 19 | 216 + 339 | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of an eRedCap UE |
| 8 to 63 | (216 + 340) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

To report UE capability/request for Msg4 PDSCH repetition, the following codepoints should be defined:

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 48 bits for Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of an eRedCap UE

# Conclusion

In summary, we would like to share our view on downlink coverage enhancement with following proposals:

- Not to introduce the SSB-index based SMTC selection in addition to location-based solution.

RAN2 discuss if it is workable to associate only one reference location with each SMTC, considering one SMTC may be linked to multiple cells (PCI list) measurement

A threshold for Msg4 PDSCH repetition should be configured by SIB1.

A UE capable of Msg4 PDSCH repetition should report its capability/request:

- if both repetition number and threshold are configured and the RSRP of DL pathloss reference is less than the configured threshold, or

- if repetition number is configured and threshold is not configured

To report UE capability/request for Msg4 PDSCH repetition, the following codepoints should be defined:

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 48 bits for Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 64 bits for Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition, except for an (e)RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of a RedCap UE

- CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of an eRedCap UE

- CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK and Msg4 PDSCH repetition of an eRedCap UE

# Reference

[1] Draft_RAN2_130_Meeting_Report_v1.

[2] R2-2505024 Draft LS on Msg4 PDSCH repetition Thales.