3GPP TSG-RAN WG2 Meeting #131	R2-2505790
Bengaluru, India, August 25 – 29, 2025

Agenda Item:	8.5.2

Source:	InterDigital

Title:	Remaining issues for On demand SSB

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on Network Energy Savings for NR has an objective on on-demand SSB SCell [1].

In this contribution, remaining open issues for on-demand SSB activation/deactivation are discussed.

# Discussion

Max number of possible OD-SSB configuration

One remaining open issue is the max number of possible OD-SSB configurations that can be indicated in the MAC CE, the following editor’s note are captured in the running CRs of TS 38.321 and TS 38.331:

Editor’s note:  FFS value for maxNrofOD-SSB

Editor’s note: the size of OD-SSB ConfigIndexi is FFS

The following agreement was made in RAN1#121:

Agreement For a cell supporting on-demand SSB SCell operation, for configuring od-ssb-nrofBurst of which the value range is {N2 integer values}, - N2= 8 - Note: This is updated from the previous RAN1 agreement. - The following values for od-ssb-nrofBurst are taken as the starting point and to be confirmed in RAN1#122 - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}. - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}.

Per RAN1 agreements, the OD-SSB configuration can be used to indicate which configuration to apply, including: the OD-SSB periodicity, the SFN offset, the number of SSB bursts (od-ssb-nrofBurst), the SSB position within the burst, and the OD-SSB frequency. For the OD-SSB periodicity, RAN1 agreed to reuse legacy values, including (5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

Therefore, there is a very large number of distinct combinations of values that can be indicated for at least these five parameters. It is reasonable to support a subset of those to be indicated and configured. In our view, one out of 32 possible configurations is enough to capture enough permutations of these configuration parameters that can be indicated.

Proposal 1:	Support value 32 for maxNrofOD-SSB; i.e. corresponding to a 5 bit ConfigIndexi field indicated in the OD-SSB MAC CE.

Whether od-ssb-nrofBurst is applied upon RRC configuration

One open issue from the RRC running CR is whether od-ssb-nrofBurst is applied upon RRC configuration. RAN1 has discussed this issue and came to the following conclusion: “Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst.”

Therefore, RAN1 concluded that od-ssb-nrofBurst can only be configured by RRC and activated by MAC CE. RAN2 can follow the same conclusion.

Proposal 2:	Regarding whether od-ssb-nrofBurst is applied upon RRC configuration, follow the RAN1 conclusion: od-ssb-nrofBurst can only be configured by RRC and activated by MAC CE.

# Conclusion

The following proposals are provided for on demand SSB reception on SCells:

Proposal 1:	Support value 32 for maxNrofOD-SSB; i.e. corresponding to a 5 bit ConfigIndexi field indicated in the OD-SSB MAC CE.

Proposal 2:	Regarding whether od-ssb-nrofBurst is applied upon RRC configuration, follow the RAN1 conclusion: od-ssb-nrofBurst can only be configured by RRC and activated by MAC CE.

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson