3GPP TSG-RAN WG2 Meeting #131			     R2-2505805

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.7.6 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining open issues on rate control signaling for XR

Document for: 	Discussion and Decision

Introduction

Following remaining open issues related to UL rate control were identified during the MAC running CR discussion. This contribution discusses these open issues.

[MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE. [MAC-4] Format of the UL Rate Control MAC CE. [MAC-5] Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

Discussion

- Agreements in RAN2#130 meeting: We rule out explicit signalling of PDU session ID + QFI We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping

The MAC-3 open issue is how to identify a QoS flow in a UL rate control MAC CE and two options are remained as shown in the RAN2 agreements above.

- Option 1: Explicit DRB ID + QFI (FFS if QFI is explicit or implicit);

- Option 2: Implicit, e.g. index or mapping.

Before discussing each option, we need to understand how the UL rate control MAC CE is generated and transmitted to the UE. Considering CU-DU split architecture, the CU-UP can know which QoS flow should be throttled and how much data rate should be changed for a specific QoS flow, but the UL rate control MAC CE is generated by the DU since the MAC entity is located at the DU, not CU-UP. This means that the exact information for UL rate control (i.e., DRB ID, QoS flow ID, and required data rate) is recognized by CU-UP, but the CU-UP cannot make the UL rate control MAC CE, i.e., DU makes the UL rate control MAC CE.

Observation 1. The CU-UP can know which QoS flow should be throttled and how much data rate should be changed for a specific QoS flow, but the DU generates the UL rate control MAC CE.

In this situation, if the option 2 is used, the DU would not exactly know QoS flow configurations and DRB-to-QoS flow mapping since this information is available at the CU-UP, not DU as explained in the observation 1. Thus, if a bitmap is used to identify QoS flow among a DRB, it is doubt whether this bitmap can be filled by the DU without addition F1 signalling/information from the CU-UP.

Observation 2. If a bitmap is used for the option 2, the DU would not fill this bitmap without additional signalling/information from the CU-UP since the DU does not exactly know QoS flow configurations and DRB-to-QoS flow mapping and this information is available at the CU-UP.

Companies also suggest to have a configured bitmap or a new short identifier for DRB ID+QoS flow ID mapping by RRC, but the bitmap or a new short identifier would be updated/changed whenever QoS flow-to-DRB remapping occur, DRB established/released, or QoS flow established/released. This means that additional signalling exchange/overhead is required for the option 2.

Observation 3. The option 2 may require additional signalling exchange/overhead since the bitmap or a new short identifier would be updated/changed whenever QoS flow-to-DRB remapping occur, DRB established/released, or QoS flow established/released.

On the other hand, if the option 1 is used, the DU can easily generate a UL rate control MAC CE since the CU-UP will provide this information (i.e., DRB ID, QoS flow ID, and required data rate) to the DU and the DU can directly use this information to make the UL rate control MAC CE without additional operation to generate a bitmap. In our view, even though the option 1 requires a little more bits than the option 2 in order to identify one QoS flow, one UL rate control MAC CE may not control too many QoS flows simultaneously since the UE may not use multiple XR services at the same time.

Proposal 1. DRB ID + QoS flow ID is used to indicate a QoS flow in the UL rate control MAC CE.

For the MAC-4, Considering the RAN2 agreements so far, the UL rate control MAC CE format should be possible to have multiple QoS flows associated with one DRB or multiple DRBs and this may require additional field in the format. Thus, following fields would be considered for the UL rate control MAC CE format:

- DRB ID (5bits);

- Number of QoS flow within a DRB (3bits);

- QoS flow ID (6bits);

- Data rate (8bits);

Proposal 2. The UL rate control MAC CE includes 5bit DRB ID, 3bit number of QoS flow, 6bit QFI, and 8bit data rate field.

The example format for the proposal 2 seems like the following figure.

Figure 1. UL rate control MAC CE format

For the MAC-5, as per the MAC running CR, the priority of a rate control MAC CE for bit rate query has same priority as MAC CE for legacy Recommended bit rate query which is lower than data from any logical channels as shown in the below box. In the legacy RBR bit rate query, if available UL-SCH resources cannot accommodate all the pending bit rate queries, the RBR bit rate query is not transmitted to the network. The issue is whether to have same principle for XR UL rate control query as in legacy RBR bit rate query.

- data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query, or MAC CE for UL bit rate query;

The important point for this issue is that the target scenario of XR UL rate control query is quite different from the legacy RBR bit rate query and this principle should be changed for XR UL rate control query due to following reasons.

Legacy RBR bit rate query is for VoIP and there should be no heavy traffic when the UE uses VoIP. This means that even though the priority of RBR bit rate query is lower than data from any logical channels, the possibility of having enough UL-SCH resources for legacy RBR bit rate query would be high. However, normally XR service requires very high data rate even when the UE uses a XR service. Most UL-SCH resources would be allocated to the logical channels having available data for transmission and the possibility of having enough UL-SCH resources for XR UL rate control query would be small.

In this situation, if XR rate control query can be transmitted only when available UL-SCH resources can accommodate all the pending bit rate queries, the triggered XR rate control query would be delayed and the network cannot receive the rate query information at the time point when the network and the UE needs. This may result in data loss and increasing network congestion. Thus, the UE should transmit some of the queries, if available UL-SCH resources cannot accommodate all the pending bit rate queries.

Proposal 3. The UE transmits some of the queries, if available UL-SCH resources cannot accommodate all the pending bit rate queries.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. The CU-UP can know which QoS flow should be throttled and how much data rate should be changed for a specific QoS flow, but the DU generates the UL rate control MAC CE.

Observation 2. If a bitmap is used for the option 2, the DU would not fill this bitmap without additional signalling/information from the CU-UP since the DU does not exactly know QoS flow configurations and DRB-to-QoS flow mapping and this information is available at the CU-UP.

Observation 3. The option 2 may require additional signalling exchange/overhead since the bitmap or a new short identifier would be updated/changed whenever QoS flow-to-DRB remapping occur, DRB established/released, or QoS flow established/released.

Proposal 1. DRB ID + QoS flow ID is used to indicate a QoS flow in the UL rate control MAC CE.

Proposal 2. The UL rate control MAC CE includes 5bit DRB ID, 3bit number of QoS flow, 6bit QFI, and 8bit data rate field.

Proposal 3. The UE transmits some of the queries, if available UL-SCH resources cannot accommodate all the pending bit rate queries.