3GPP TSG-RAN WG2 #131	R2-2505827

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.8.1

Source:	Ericsson

Title:	Inclusion of the ISA in the Service Announcement

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has received a reply LS from SA4 [1] regarding the inclusion of the Intended Service Area (ISA) in the Service Announcement (also known as USD) for MBS NTN.

# 1. Overall Description:

SA4 thanks RAN2 for bringing up the topic of “MBS via NTN”. SA4 is still studying the realization for including an Intended Service Area into Service announcement, leveraging the RAN2 Work Item code NR_NTN_Ph3-Core.

SA4 has agreed to extend the Service Announcement and add a targeted service area in form of a list of polygon or circle shapes. However, SA4 is still discussing the technical realization and possibly a generalization, so that the target service area can be extended to represents the intended service area in case of NR NTN.

# SA4 confirms, that inclusion of the geographical information, i.e. in form of list of polygon or circle shapes in the Service Announcement can be supported within the Rel-19 timeframe.

# 2. Actions:

# To RAN WG2:

# Please take the above provided information into account.

SA4 has confirmed that the adoption of the ISA in the Service Announcement is possible within Rel-19 and have already starting modifying their specifications[2]. This contribution analyses the RAN2 impact and presents a way forward to follow SA4’s conclusion.

# 2	Discussion

## 2.1	ISA and MCCH re-acquisition

A UE interested in MBS must first acquire Service Announcement to become aware of the existing MBS services and their respective characteristics (e.g., start time). This may happen even before attempting to decode the MBS configuration provided in MCCH. The current RRC specification allows the UE to skip MCCH monitoring based on USD/Service Announcement:

[NOTE in TS 38.331 V18.3.0 - Section 5.9.2.2]

“It is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.”

On a related note, Stage-2 specification allows the UE to decide when to start monitoring the MCCH based on the information signalled in the Service announcement:

[NOTE in TS 38.300 V18.2.0 - Section 16.10.6.2]

It is up to UE implementation to use the start and stop times in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

The first identified impact on the inclusion of the ISA in the Service Announcement is regarding MCCH acquisition. Based on SA4 conclusion, in addition to the information present in the new SIB, the ISA present in the Service announcement may be used to relax the acquisition of MCCH. Similar to legacy, this should be based on UE implementation. We consider there is a minor specification impact and a text proposal for TS 38.300 and TS 38.331 is included in section 3.

It is up to UE implementation how to use the ISA definition in SIB/USD to skip MCCH acquisition when it is outside the ISA of the MBS broadcast service(s) it is interested in. Take the text proposals to TS 38.331 and TS 38.300 as baseline.

## 2.2	ISA and frequency prioritization

To facilitate seamless cell re-selection and optimize frequency prioritization in NGSO-based NTN scenarios, RAN2#129bis agreed that a UE may prioritize a frequency associated with an MBS session when the service is exclusively available on that frequency, provided the UE is located within the corresponding Intended Service Area (ISA). Subsequent discussions at RAN2#130 addressed the relationship between frequencies and ISAs, concluding not to introduce ISA-specific information into SIB21. Being a geo-fencing concept, it was further assumed that the ISA is applicable to all frequencies supporting the relevant MBS session. Additionally, RAN2 has recently received a LS reply from SA4 [S4-251099], confirming that the Service Announcement will contain geographical information utilizing polygon and circular shapes aligned with the RAN2 approach.

SA4 has confirmed the inclusion of the Intended Service Area defined with circles and polygon shapes in the Service Announcement.

Given this progress, we propose leveraging the established Service Announcement mechanism to convey the ISA and FSA ID directly from the Application Function (AF) to the UE. This approach ensures that the UE receives all the necessary information via the Service Announcement / User Service Description (USD) and SIB21, thereby supporting robust service continuity with minimal impact on RAN2 specifications. The only anticipated change would be the inclusion of a clarifying note in either Stage 2 or TS 38.304.

The Intended Service Area in the Service Announcement can be used for frequency prioritization in MBS NTN. Introduce the following NOTE in Stage 2 or TS 38.304.

NOTE 0x: In NTN, the UE may obtain the intended service area associated to an MBS broadcast service from SIBXX or USD. How the UE determines itself to be within the intended service area is left to the UE's implementation..

## 2.3	Stage 3 details on ISA format

In accordance with recent agreements, SA4 has specified that the ISA within the Service Announcement should be represented using either the Polygon or PointUncertaintyCircle formats as defined in TS 29.572 [2]. In parallel, RAN2 has adopted a circular representation based on referenceLocation and distanceRadius directly specified in RRC, alongside the Polygon shape defined in TS 23.032 and TS 37.355.

RAN2 and SA4 have chosen different representations of the Intended Service Area in System Information and Service Announcement, respectively.

Given that both approaches convey equivalent geospatial information, it is better to harmonize the specification by adopting the Ellipsoid-PointWithUncertaintyCircle, as defined in TS 37.355, for the circular ISA representation. Noting there would be a small difference in the range of values supported. The current distanceRadius is a 16-bit field supporting values between 0 and 3276 kilometres. In contrast, the Ellipsoid-PointWithUncertaintyCircle has an uncertainty of 7 bits with a suitably useful range between 0 and 1800 kilometres. According to TS 23.032, this uncertainty is expressed in metres and follows this formula (K=127 since 7 bits are used for the encoding):

Provided the nature of this enhancement, we understand that the difference is not significant (Ellipsoid-PointWithUncertaintyCircle consumes 9 bits less) in comparison to maintain the specifications clean and aligned across working groups. Therefore, we propose to harmonize the specification by adopting the Ellipsoid Point with Uncertainty Circle as defined in TS 37.355 for the circular ISA representation. An illustrative ASN.1 definition for this proposal is provided below.

Use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX.

SIBXX information element

-- ASN1START

-- TAG-SIBXX-START

SIBXX-r19 ::=                     SEQUENCE {

intendedServiceAreaList-r19     IntendedServiceAreaList-r19      OPTIONAL,  -- Need R

lateNonCriticalExtension        OCTET STRING                     OPTIONAL,

...

}

IntendedServiceAreaList-r19 ::=   SEQUENCE (SIZE (1.. maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19

IntendedServiceAreaInfo-r19 ::=   SEQUENCE {

intendedServiceAreaId-r19       MBS-IntendedAreaID-r19,

areaCoordinates-r19             CHOICE {

polygonArea-r19                   OCTET STRING,

circleArea-r19                    OCTET STRING

}

}

MBS-IntendedAreaID-r19  ::= INTEGER (0.. maxNrofMBS-Area-r19)

-- TAG-SIBXX-STOP

-- ASN1STOP

SIBXX field descriptions

intendedServiceAreaList Contains a list of intended service areas, each associated to one or more MBS broadcast services provided in an NTN cell.

polygonArea Contains the Polygon IE as defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

circleArea Contains the Ellipsoid-PointWithUncertaintyCircle IE as defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

# 3	Text Proposals

## 3.1	TP to TS 38.300 v18.3.0

#### 16.10.6.2	Configuration

MBS broadcast can be received by UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. A UE can receive the MBS configuration for broadcast session (e.g., parameters needed for MTCH reception) via MCCH in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. The parameters needed for the reception of MCCH are provided via System Information.

The following principles govern the MCCH structure:

-	MCCH provides the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session includes MBS session ID, associated G-RNTI scheduling information and information about neighbouring cells providing certain service on MTCH(s). MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset;

-	MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary; a notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification or stop and due to neighbouring cell information modification;

NOTE:	It is up to UE implementation to use the start and stop times and the ISA in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

-	When the UE receives an MCCH change notification, it acquires the updated MCCH in the same MCCH modification period where the change notification is sent.

## 3.2	TP to TS 38.331 v18.3.0

#### 5.9.2.2	Initiation

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s).

NOTE 1:	It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification.

NOTE 2:	It is up to UE implementation to use the cell/tracking area list and/or the ISA in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service.

Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	SA4 has confirmed the inclusion of the Intended Service Area defined with circles and polygon shapes in the Service Announcement.

Observation 2	RAN2 and SA4 have chosen different representations of the Intended Service Area in System Information and Service Announcement, respectively.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It is up to UE implementation how to use the ISA definition in SIB/USD to skip MCCH acquisition when it is outside the ISA of the MBS broadcast service(s) it is interested in. Take the text proposals to TS 38.331 and TS 38.300 as baseline.

Proposal 2	The Intended Service Area in the Service Announcement can be used for frequency prioritization in MBS NTN. Introduce the following NOTE in Stage 2 or TS 38.304.

Proposal 3	Use the Ellipsoid Point with uncertainty Circle from TS 37.355 to define the circle shape of the ISA in the new SIBXX.

# 5	References

S4-251099, Reply on Inclusion of NTN intended service area in the Service Announcement, SA4#132, Contact: Ericsson, May 2025.

S4-251482, Target Service Area Data Type in Service Announcement, Ericsson, May 2025.