3GPP TSG-RAN WG2 #131	R2-2505839

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.1.3

Source:	Ericsson

Title:	NW-side data collection

Document for:	Discussion

# 1	Introduction

In this contribution we share our views on remaining open issues that were the outcome of the email discussion [POST130][026][AI PHY] 38.331 Running CR (Ericsson).

# 2	Discussion on NW-side data collection for beam management

## 2.1 RRC open issues

### Open issue RRC-19: Reporting assistance information related to logged measurements

RAN2#129bis agreed that:

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Based on these agreements, the following configuration structure was implemented in the RRC CR to configure the three availability indication triggers via otherConfig:

LoggedDataCollectionAssistanceConfig-r19 ::= SEQUENCE { loggedDataCollectionBufferThreshold-r19 ENUMERATED {FFS} OPTIONAL, -- Need R loggedDataCollectionFullBuffer-r19 ENUMERATED {true} OPTIONAL, -- Need R loggedDataCollectionPowerLow-r19 ENUMERATED {true} OPTIONAL -- Need R }

Namely, for each of the three triggers there is a field within the IE LoggedDataCollectionAssistanceConfig. It is straightforward to have a separate optional field to configure the indication “buffer threshold reached” as in the structure above, because a specific buffer threshold value is needed by the UE to trigger the UAI indication in this case. However, for the “full buffer being reached” indication, some companies proposed an alternative to the structure above. The alternative would be to not have a dedicated configuration bit for “full buffer being reached” and instead assume that this indication is configured whenever the IE LoggedDataCollectionAssistanceConfig is configured, given that this indication is important for the NW to understand that the UE stopped logging and thus should always be configured whenever assistance information reporting for NW-side data collection is configured at the UE. This solution would also save 1 bit in the configuration. In our view, either solution works. Since there is a related discussion on the importance of these three indications in the email discussion [POST130][038][AI PHY] UE capabilities (Xiaomi), we suggest to first discuss the outcome of this email discussion.

(RRC-19) The “buffer threshold reached” indication is configured with a dedicated field in otherConfig, which gives the value of the threshold.

(RRC-19) RAN2 to discuss whether the “full buffer being reached” and “low power” indications need each a dedicated configuration bit in otherConfig, after discussing the outcome of [POST130][038][AI PHY] UE capabilities (Xiaomi).

### Open issue RRC-21: Time related content of logged data

RAN2 agreed that the logged data is grouped per configuration and that the “gap” (time interval larger than the configured logging periodicity) needs to be indicated.

The timestamp (absolute or relative) of a logged measurement is already included in the logged MDT report, as a tool to assist the network in retrieving NW operating conditions at the moment in which the measurement was taken by the UE. The same solution could be adopted in the context of AIML. We note that, if the absolute time stamp is included once in the report and the relative timestamp is reported for every taken measurement as for logged MDT, we do not foresee major overhead issues, given the UEInformationRequest/ UEInformationResponse that was agreed to be used for fetching data. This framework supports reporting amounts of data that are large enough.

However, for the beam management use case, some companies have expressed the desire to reduce the amount of time information included in the reports even further, in order to reduce the amount of signalling. This is possible given that some measurement entries will be logged at regular intervals given by the logging periodicity of the configuration. The solution could be as follows:

- Include an absolute time stamp (provided by the network at the time of configuring the UE with the NW-side data collection configuration) once in the report, as for logged MDT.

- Within the list of measurement entries for a given logging configuration:

  - For the first entry, include a relative time stamp (with respect to the absolute time stamp).

  - For the other entries, if the measurement was taken at a time difference equal to the periodicity versus the immediate previous entry, omit the time stamp; otherwise, if the measurement was taken at a “gap” interval versus the immediate previous entry, add a relative time stamp.

(RRC-21) The UE includes in the logged measurement report, an absolute time stamp (provided by the network at the time of configuring the UE with the NW-side data collection configuration), and for the first measurement entry and each measurement entry separated by a “gap” versus the immediate previous entry, a relative time stamp.

### Open issue RRC-24: Where to include the logging configuration from NW to UE

In our companion paper “Discussion on NW-side data collection framework” we provide an extensive analysis and propose to include the CSI measurement configuration for NW-side data collection within the CSI-MeasConfig.

### Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

It is useful to allow the gNB to activate the NW-side data collection based on L1 measurements received over the UCI. Especially since the NW-side data collection based on L1 events is not supported, then there is the need to give the possibility to the NW to dynamically activate the data collection based on real-time L1 measurements received from the UE. Otherwise, there is the risk with periodic resources that the gNB collects a lot of redundant data, that were already previously collected. The dynamic activation/deactivation of a logging configuration can be performed via a MAC CE and it can be further discussed if a new MAC CE is needed, or whether a legacy MAC CE can be enhanced for this, depending also on the outcome of open issue RRC-24 on where the logging configuration will be transmitted.

(RRC-25) Dynamic activation/deactivation of logging configurations for NW-side data collection is supported. FFS whether to introduce a new MAC CE or to enhance a legacy MAC CE, based on the outcome of open issue RRC-24.

### Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

In our opinion, this is an optimization and the cases in which it is beneficial are probably rare. First of all, if the UE reports low power state without any data availability indication (e.g. the UE has not yet logged anything, or the logged amount of data is very small), the NW always has the possibility to probe whether the UE logged any data in the buffer, by sending a UEInformationRequest.

The benefit of including a data availability indication (before buffer threshold is reached) together with a low power indication is only that it can save one unnecessary exchange of UEInformationRequest/ UEinformationResponse, if there is no logged data in the buffer at all. Furthermore, it is unclear how often it happens in practice that the UE reports low power state right after receiving a logging configuration and before it has time to log anything at all. Thus, the benefit of this solution seems to be questionable.

(RRC-29) UE does not send data availability indication when it sends low power indication, if the amount of data is below the configured buffer threshold.

### Open issue RRC-30: Semi-persistent resources for data collection

We do not see any reason why semi-persistent resources should not be supported for NW-side data collection. Moreover, RAN1 has not excluded semi-persistent resources for UE-side data collection, so the same type of resources should also be supported for NW-side data collection.

(RRC-30) Semi-persistent resources for NW-side data collection are supported.

### Open issue RRC-31: The release of logged AIML data in UE’s buffer

In our view, it is not needed to further specify that the UE should discard logged data upon: power off or deregistration, 48 hrs after the release of logged measurement configuration, or explicit indication from the serving gNB. For the AIML logged data, RAN2 already agreed that the logged data will be discarded in many cases, i.e. upon transition to IDLE/INACTIVE or RLF, or upon HO (unless the UE is explicitly indicated to retain the data). We note that the AIML logged data is collected in RRC_CONNECTED and is thus different than legacy MDT data, which required other conditions to discard data.

(RRC-31) RAN2 does not specify further cases when the UE shall discard logged data for NW-side data collection, beyond already agreed cases.

### Open issue RRC-33: Whether separate user consent for gNB centric training is needed

In our view, it is unclear why user consent is needed for collecting data for the considered beam management use case. For this use case (the only one relevant to NW-side data collection in Rel-19), the collected radio measurements are the same that the UE reports in L1 CSI reports based on legacy configurations. Furthermore, this issue is outside the scope of RAN2.

(RRC-33) User consent related aspects are outside RAN2 scope.

(RRC-33) RAN2 does not discuss user consent related aspects for NW-side data collection in Rel-19.

### Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

RAN2 agreed that the UE can retain logged data upon HO (if the UE is explicitly indicated to do so) and thus the target gNB can retrieve this data afterwards. Naturally, the target gNB is not the intended final destination node for the logged data, given that the logging configuration was configured by another gNB previously.

For the case where the collected logged data should be forwarded by the target gNB to the source gNB, this can be achieved via implementation. It is reasonable to assume that the most common case when the source gNB indicates to retain the logged data upon HO is when the target gNB is from the same NW vendor. Thus, an implementation-based solution for transferring data from source to target gNB would cover the typical case in practice.

Overall, this open issue does not seem critical for the completion of the WI and we thus propose to down prioritize it.

(RRC-36) RAN2 to down prioritize the discussion on how data is forwarded to OAM or source gNB after HO.

### Open issue RRC-52: NW control on retaining logged data at CHO/LTM

In our view, there is no need for further specification changes and the RAN2 agreement below applies also for CHO and LTM.

1 (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced. In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data). We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it. Notify RAN3

Also in case of CHO and LTM, the UE receives an RRCReconfiguration message (with reconfigurationWithSync), which is generated by the target cell/node. The message is received via the source cell before the execution, and stored as a CHO candidate configuration or LTM candidate configuration, respectively. When the CHO/LTM execution is triggered (fulfillment of execution condition or reception of LTM Cell Switch Command MAC CE, respectively), the UE then applies the RRCReconfiguration message (with reconfigurationWithSync), and executes 5.3.5.3 in 38.331.

Just as for normal inter-gNB HO, in case of inter-gNB CHO or LTM (introduced in Rel-19) there is/will be a HandoverPreparation message sent from the source gNB to the target gNB. This can thus indicate to the target gNB to include (in the RRCReconfiguration message) the indication to retain the logged data at the execution procedure.

We do thus not see the need for specific solutions on how to control retaining logged data at CHO/LTM.

(RRC-52) Specific solutions on how to control retaining logged data at CHO/LTM are not needed, since such control is already covered by the previous RAN2 agreement on retaining logged data upon HO.

(RRC-52) RAN2 does not specify further solutions for NW control on retaining logged data at CHO/LTM in Rel-19.

## 2.2 UE capabilities open issues

### Open issue UECap-1: whether UE can support other memory sizes and indicate to network via optional capability signaling

In case the UE has a larger memory size than the minimum 64 kB (which will likely be agreed for the baseline capability) and if this additional memory size can be used for logged data for NW-side data collection, it is beneficial if this can be signaled to the network. This allows the NW to estimate better when to retrieve the logged data so that the buffer does not become full and also to configure more suitable buffer threshold values for the data availability indication.

(UECap-1) Introduce an optional UE capability to indicate whether the UE supports larger memory sizes than 64 kB.

### Open issue UECap-2: How to define UE logged measurement of data collection for NW-side model

In our view, an overall/generic NW-side data collection capability is needed. The generic capability should imply the UE supports all of the following:

- The minimum AS memory size of 64 kB;

- Periodic and L3 event-based logging. Alternatively, at least periodic logging should be mandatorily covered by the generic NW-side data collection capability, while L3 event-based logging can be optional. We note that even for L3 event-based logging, the UE will perform logging periodically during the time that the L3 event is fulfilled, so it is reasonable to assume that at least periodic logging is mandatorily covered by the generic UE capability for NW-side data collection;

- Reporting assistance information for logging (especially data availability based on reaching the full buffer size and exceeding the configured buffer threshold). This is important so that the NW is aware of when it can start retrieving data.

(UE-Cap2) Introduce an optional generic UE capability to indicate supporting logged measurements for NW-side data collection, which implies the UE supports at least: the minimum AS layer memory size, periodic logging, reporting data availability upon reaching the full buffer size, and reporting data availability upon exceeding the configured buffer threshold.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	(RRC-33) User consent related aspects are outside RAN2 scope.

Observation 2	(RRC-52) Specific solutions on how to control retaining logged data at CHO/LTM are not needed, since such control is already covered by the previous RAN2 agreement on retaining logged data upon HO.

Based on the discussion in the previous sections we propose the following:

Proposal 1	(RRC-19) The “buffer threshold reached” indication is configured with a dedicated field in otherConfig, which gives the value of the threshold.

Proposal 2	(RRC-19) RAN2 to discuss whether the “full buffer being reached” and “low power” indications need each a dedicated configuration bit in otherConfig, after discussing the outcome of [POST130][038][AI PHY] UE capabilities (Xiaomi).

Proposal 3	(RRC-21) The UE includes in the logged measurement report, an absolute time stamp (provided by the network at the time of configuring the UE with the NW-side data collection configuration), and for the first measurement entry and each measurement entry separated by a “gap” versus the immediate previous entry, a relative time stamp.

Proposal 4	(RRC-25) Dynamic activation/deactivation of logging configurations for NW-side data collection is supported. FFS whether to introduce a new MAC CE or to enhance a legacy MAC CE, based on the outcome of open issue RRC-24.

Proposal 5	(RRC-29) UE does not send data availability indication when it sends low power indication, if the amount of data is below the configured buffer threshold.

Proposal 6	(RRC-30) Semi-persistent resources for NW-side data collection are supported.

Proposal 7	(RRC-31) RAN2 does not specify further cases when the UE shall discard logged data for NW-side data collection, beyond already agreed cases.

Proposal 8	(RRC-33) RAN2 does not discuss user consent related aspects for NW-side data collection in Rel-19.

Proposal 9	(RRC-36) RAN2 to down prioritize the discussion on how data is forwarded to OAM or source gNB after HO.

Proposal 10	(RRC-52) RAN2 does not specify further solutions for NW control on retaining logged data at CHO/LTM in Rel-19.

Proposal 11	(UECap-1) Introduce an optional UE capability to indicate whether the UE supports larger memory sizes than 64 kB.

Proposal 12	(UE-Cap2) Introduce an optional generic UE capability to indicate supporting logged measurements for NW-side data collection, which implies the UE supports at least: the minimum AS layer memory size, periodic logging, reporting data availability upon reaching the full buffer size, and reporting data availability upon exceeding the configured buffer threshold.