3GPP TSG-RAN WG2 Meeting #131	R2-2505866

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.1.2.3

Source:	Nokia

Title:	Applicability reporting for positioning use case

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As per the latest open issue LPP-19 in [POST130][025][AI PHY] 37.355 CR email discussion, companies are invited to provide contributions related to applicability reporting in this RAN2#131 meeting:

Open issue LPP-19 (new): Applicability of BM related agreements Issue description: As mentioned in [POST130][023][AI PHY] 38.305 CR (CATT), since the RAN2 intention is to have the same design for BM case and positioning case as much as possible, whether the agreements made for BM applicability reporting can also be applicable to positioning case 1. Whether the following agreements made for BM applicability reporting are also applicable to positioning case 1 and whether there are any associated LPP impacts: - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. Proposed resolution: Companies to provide contributions to the following meeting.

In this contribution, we provide our views and proposals with regards to applicability reporting, for discussion and decision.

# 2	Discussion

## 2.1	Determining and reporting of applicability

For DL AI/ML positioning (“Case 1”), UE is expected to determine applicability of its trained AI/ML model(s) to perform inference based on network (NW)-side (additional) conditions as well as any UE-side conditions. While UE-side conditions are not specified, hence not disclosed by the UE to the network, NW-side (additional) conditions are provided to the UE, mainly via the LPP Assistance Data message. These include information such as PRS configuration and TRP location information (explicit or implicit) etc.

Upon determining applicability, UE can then perform inference using the information contained in the assistance data, such as by performing necessary DL PRS measurements to provide input to its AI/ML model applicable for the given PRS configuration. The information contained in assistance data thus can be also referred to as “inference configuration”.

Observation 1: The assistance data the UE receives from LMF at least in NR-DL-AIML-ProvideAssistanceData IE are considered as inference configuration for DL AIML positioning.

Observation 2: For performing inference in DL AI/ML positioning (“Case 1”), UE determines applicability of its AI/ML model(s) at least based on the inference configurations it receives from LMF such as DL PRS configuration and TRP location information (explicit or implicit) provided in NR-DL-AIML-ProvideAssistanceData IE.

Observation 3: The first agreement from BM use case listed in the open issue LPP-19 viz. “UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device” is applicable for positioning use case also.

RAN2 had already agreed that UE indicates support for DL AIML positioning and related capabilities during the initial Capability Transfer procedure between LMF and UE. RAN2 had also agreed that whenever there is any change in applicability of DL AIML positioning, UE indicates this to the network using Capability Indication procedure by sending an unsolicited LPP ProvideCapabilities message. However, for indicating a change in applicability where the DL AIML positioning is currently not possible requires an explicit signalling bit in NR-DL-AIML-ProvideCapabilities IE because merely sending the NR-DL-AIML-ProvideCapabilities only tells the LMF that DL AIML positioning is supported.

Observation 4: The second agreement from BM use case listed in the open issue LPP-19 viz. “Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed” is applicable for positioning use case also.

Proposal 1: Add an explicit bit in NR-DL-AIML-ProvideCapabilities-r19 to indicate dynamic change in UE DL AIML positioning applicability from supported to “currently not-applicable”.

Given that applicability of DL AIML positioning in UE may dynamically change depending on the inference configuration provided in NR-DL-AIML-ProvideAssistanceData IE, the LMF may have to provide the right assistance and get feedback from UE whether DL AIML positioning is applicable or not which may lead to additional signalling overhead resulting in delays in obtaining the UE position prediction. To reduce signalling overhead and additional latency before getting the predicted UE position from the UE the LMF could proactively provide a list of candidate inference configurations (DL-PRS configurations) to the UE in advance. That is, LPP Assistance Data may contain candidate inference configurations, e.g., a list of PRS configurations, which UE may utilize to evaluate applicability and provide feedback about the inference configurations for which DL AIML positioning is applicable, whenever the UE reports DL AIML positioning capabilities. For this, the UE may provide a list of IDs of DL PRS configurations which UE has determined to be applicable.

Proposal 2: The NR-DL-AIML-ProvideAssistanceData IE signals a list of candidate DL PRS configurations which can be used by the target device to determine applicability of AI/ML positioning for the provided set of DL-PRS configurations.

Proposal 3: The UE may optionally include in NR-DL-AIML-ProvideCapabilities IE the DL-PRS configurations from the set of candidate DL-PRS configurations assistance provided by the location server for which the DL AIML positioning is currently applicable in the UE.

## 2.2	Text proposal for TS 37.355

In this clase we show what would be the impacts to ASN.1 for implementing the proposals in this paper.

#### –	NR-Candidate-DL-PRS-Configurations

The IE NR-Candidate-DL-PRS-Configurations provides a set of possible DL-PRS configurations which can be used by the target device to determine applicability of AI/ML positioning for the provided set of DL-PRS configurations.

-- ASN1START

NR-Candidate-DL-PRS-Configurations-r19 ::= SEQUENCE {

candidate-dl-prs-configuration-list-r19		SEQUENCE (SIZE (1..maxCandidate-DL-PRS-Configs-r19)) OF

Candidate-DL-PRS-Configuration-r19,

...

}

Candidate-DL-PRS-Configuration-r19 ::= SEQUENCE {

candidate-dl-prs-configuration-id-r19			Candidate-DL-PRS-Configuration-ID-r19,

nr-DL-PRS-PositioningFrequencyLayer-r19		NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-Info-r19							NR-DL-PRS-Info-r16,

...

}

Candidate-DL-PRS-Configuration-ID-r19 ::= SEQUENCE {

nr-dl-prs-configuration-id-r19				INTEGER (1.. maxCandidate-DL-PRS-Configs-r19),

...

}

-- ASN1STOP

NR-Candidate-DL-PRS-Configurations field descriptions

candidate-dl-prs-configuration-id This field identifies a candidate DL-PRS Configuration information for AI/ML positioning.

nr-DL-PRS-PositioningFrequencyLayer This field, together with nr-DL-PRS-Info, provides the candidate DL-PRS configuration information.

nr-DL-PRS-Info This field, together with nr-DL-PRS-PositioningFrequencyLayer, provides the candidate DL-PRS configuration information.

## 6.5	Positioning Method IEs

### 6.5.13	NR DL AI/ML Positioning

This clause defines the information elements for NR DL AI/ML positioning (TS 38.305 [40]).

#### 6.5.13.1	NR DL AI/ML Positioning Assistance Data

#### –	NR-DL-AIML-ProvideAssistanceData

The IE NR-DL-AIML-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based DL AI/ML positioning. It may also be used to provide NR DL AI/ML positioning specific error reason.

-- ASN1START

NR-DL-AIML-ProvideAssistanceData-r19 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r19		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r19		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-r19

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r19

NR-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17																				OPTIONAL,	-- Need ON

assistanceDataValidityArea-r19		AreaID-CellList-r17					OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r19

NR-PositionCalculationAssistance-r16

OPTIONAL,	-- Need ON

nr-DL-AIML-Positioning-Error-r19	NR-DL-AIML-Positioning-Error-r19	OPTIONAL,	-- Need ON

...,

[[

nr-Candidate-DL-PRS-Configurations-r19

NR-Candidate-DL-PRS-Configurations-r19

OPTIONAL,	-- Need ON

]]

}

-- ASN1STOP

Editor's Note: The above list of assistance data elements is FFS and depends on further RAN1/RAN2 agreements. The draft list above is the same as in IE NR-DL-TDOA-ProvideAssistanceData but without the NR-PeriodicAssistData.

NR-DL-AIML-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-DL-TDOA-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-AIML-ProvideAssistanceData message.

nr-On-Demand-DL-PRS-Configurations This field provides a set of available DL-PRS configurations which can be requested by the target device on-demand. NOTE: If this field is absent but the nr-On-Demand-DL-PRS-Configurations-Selected-IndexList is present, the nr-On-Demand-DL-PRS-Configurations may be provided in IE NR-DL-TDOA-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList This field specifies the selected available on-demand DL-PRS configurations which are applicable for this NR-DL-AIML-ProvideAssistanceData message.

assistanceDataValidityArea This field specifies the network area for which this NR-DL-AIML-ProvideAssistanceData is valid.

nr-PositionCalculationAssistance This field provides the position calculation assistance data.

nr-DL-AIML-Positioning-Error This field provides DL AI/ML positioning error reasons.

#### 6.5.13.6	NR DL AI/ML Positioning Capability Information

#### –	NR-DL-AIML-ProvideCapabilities

The IE NR-DL-AIML-ProvideCapabilities is used by the target device to indicate its capability to support NR DL AI/ML positioning and to provide its NR DL AI/ML positioning capabilities to the location server.

-- ASN1START

NR-DL-AIML-ProvideCapabilities-r19 ::= SEQUENCE {

locationCoordinateTypes-r19					LocationCoordinateTypes					OPTIONAL,

periodicalReporting-r19						ENUMERATED { supported }				OPTIONAL,

periodicReportingIntervalMsSupport-r19		PeriodicReportingIntervalMsSupport-r18	OPTIONAL,

ten-ms-unit-ResponseTime-r19				ENUMERATED { supported }				OPTIONAL,

scheduledLocationRequestSupported-r19		ScheduledLocationTimeSupport-r17		OPTIONAL,

nr-PosCalcAssistanceSupport-r19				BIT STRING {

trpLocSup			(0),

beamInfoSup			(1),

rtdInfoSup			(2),

trpTEG-InfoSup		(3),

nr-IntegritySup 		(4),

pruInfoSup			(5),

trpLoc-ImplicitSup	(6)

} (SIZE (1..8))							OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r19		SEQUENCE {

type		LOS-NLOS-IndicatorType2-r17,

granularity	LOS-NLOS-IndicatorGranularity2-r17,

...

}										OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r19		BIT STRING {

eAoD		(0),

eAoA		(1)

} (SIZE (1..8))							OPTIONAL,

nr-DL-AIML-On-Demand-DL-PRS-Support-r19		NR-On-Demand-DL-PRS-Support-r17			OPTIONAL,

nr-DL-AIML-Applicable-DL-PRS-Configurations-r19

SEQUENCE (SIZE (1..maxCandidate-DL-PRS-Configs-r19)) OF

Candidate-DL-PRS-Configuration-ID-r19		OPTIONAL,

nr-DL-AIML-On-Demand-DL-PRS-ForBWA-Support-r19

ENUMERATED { supported }				OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r19		SEQUENCE {

area-validity	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,

...

}														OPTIONAL,

multiLocationEstimateInSameMeasReport-r19	ENUMERATED { supported }				OPTIONAL,

nr-IntegrityAssistanceSupport-r19			BIT STRING {

serviceParametersSup		(0),

serviceAlertSup				(1),

riskParametersSup			(2),

integrityParaTRP-LocSup		(3),

integrityParaBeamInfoSup	(4),

integrityParaRTD-InfoSup	(5)

} (SIZE (1..8))							OPTIONAL,

...

}

-- ASN1STOP

Editor's Note: The above list of capability elements is FFS and depends on further RAN1/RAN2 agreements. The list above includes the common LPP capabilities and assistance data supported (per NR-DL-AIML-ProvideAssistanceData (NR-DL-TDOA-ProvideAssistanceData)). DL-PRS capabilities are not yet included above.

NR-DL-AIML-ProvideCapabilities field descriptions

locationCoordinateTypes This field indicates the geographical location coordinate types that a target device supports for DL AI/ML positioning. TRUE indicates that a location coordinate type is supported and FALSE that it is not.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation from the location server.

periodicReportingIntervalMsSupport This field, if present, indicates that the target device supports the reportingIntervalMs in IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation from the location server and specifies the minimum millisecond periodic reporting interval supported.

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 4 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are supported or not; bit 4 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is supported or not; bit 4 together with the bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is supported or not; - bit 5 indicates whether the field nr-PRU-DL-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 6 indicates whether the field nr-TRP-LocationInfo-Implicit in IE NR-PositionCalculationAssistance is supported or not.

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports the granularity for nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance of 'per-trp', 'per-resource', or both. Editor's Note: FFS whether the conditions defined for DL-TDOA/DL-AoD are applicable (see corresponding field description in NR-DL-TDOA-ProvideCapabilities/NR-DL-AoD-ProvideCapabilities).

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-AIML-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-DL-AIML-On-Demand-DL-PRS-ForBWA-Support This field, if present, indicates that the target device supports on-demand DL-PRS request for bandwidth aggregation.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiLocationEstimateInSameMeasReport This field, if present, indicates that the target device supports multiple location estimate instances in a single measurement report.

nr-IntegrityAssistanceSupport This field indicates the Integrity Assistance Data supported. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is supported or not.

nr-DL-AIML-Applicable-DL-PRS-Configurations This field may optionally be included by the target device to indicate the DL-PRS configurations within the set of candidate DL-PRS configurations assistance provided by the location server for which the DL AIML positioning is currently applicable in the target device.

# 3	Conclusion

This document has made the following observations:

Observation 1: The assistance data the UE receives from LMF at least in NR-DL-AIML-ProvideAssistanceData IE are considered as inference configuration for DL AIML positioning.

Observation 2: For performing inference in DL AI/ML positioning (“Case 1”), UE determines applicability of its AI/ML model(s) at least based on the inference configurations it receives from LMF such as DL PRS configuration and TRP location information (explicit or implicit) provided in NR-DL-AIML-ProvideAssistanceData IE.

Observation 3: The first agreement from BM use case listed in the open issue LPP-19 viz. “UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device” is applicable for positioning use case also.

Observation 4: The second agreement from BM use case listed in the open issue LPP-19 viz. “Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed” is applicable for positioning use case also.

And proposed the following:

Proposal 1: Add an explicit bit in NR-DL-AIML-ProvideCapabilities-r19 to indicate dynamic change in UE DL AIML positioning applicability from supported to “currently not-applicable”.

Proposal 2: The NR-DL-AIML-ProvideAssistanceData IE signals a list of candidate DL PRS configurations which can be used by the target device to determine applicability of AI/ML positioning for the provided set of DL-PRS configurations.

Proposal 3: The UE may optionally include in NR-DL-AIML-ProvideCapabilities IE the DL-PRS configurations from the set of candidate DL-PRS configurations assistance provided by the location server for which the DL AIML positioning is currently applicable in the UE.