3GPP TSG-RAN WG2 Meeting #130	R2-2505899

# Bengaluru, India, 25 – 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1921 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on inter-RAT FR2 measurement capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the description for gaplessMeas-FR2-maxCC it says that "The UE reporting this field and supporting (NG)EN-DC shall not indicate support of independentGapConfig in MeasAndMobParametersMRDC (defined in TS 38.306 [87])." The intention is that UE shall not report independentGapConfig within UE-MRDC-Capability for (NG)EN-DC when gaplessMeas-FR2-maxCC is reported in UE-EUTRA-Capability. However, UE can still report independentGapConfig in MeasAndMobParametersMRDC within UE-NR-Capability > NRDC-Parameters > measAndMobParametersNRDC for NR-DC. This should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify in the description for gaplessMeas-FR2-maxCC that UE reporting the field shall not indicate support of independentGapConfig in MeasAndMobParametersMRDC "within UE-MRDC-Capability". Impact analysis Impacted 5G architecture option: (NG)EN-DC. Impacted functionality: Inter-RAT FR2 measurements. Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no interoperability issue; however, the UE may unnecessarily restrict itself from reporting gapless FR2 inter-RAT measurement capabilities that are otherwise supported, in which case the network would configure the UE with measurement gaps. - If the UE is implemented according to the CR and the network is not, there is no interoperability issue; however, network may not consider that gapless FR2 inter-RAT measurements are supported by the UE and would configure the UE with measurement gaps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 inter-RAT measurement gaps may be configured when they are otherwise unneeded. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.52 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331 CR 5147 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 4.3.6.52	gaplessMeas-FR2-maxCC-r17

This field defines whether the UE supports inter-RAT NR FR2 measurement without measurement gap as specified in clause 9.1.2 of TS 38.133 [37] while the number of configured serving cells is less than or equal to the indicated number. This field is applicable when only E-UTRA serving cells are configured. The UE reporting this field and supporting (NG)EN-DC shall not indicate support of independentGapConfig in MeasAndMobParametersMRDC within UE-MRDC-Capability (defined in TS 38.306 [32]).

End of Changes