3GPP RAN WG2 Meeting #131	R2-2505904

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.11.2

Source:	InterDigital, Inc.

Title:	Remaining issues on RACH aspect in SBFD

# 1. Introduction

In this contribution, we discuss remaining issues and switch from SBFD-ROs to legacy-ROs considering the transmission power of PRACH, and also, how to indicate whether it’s an SBFD-aware UE during random access procedure. More details on related discussions are found in our companion contribution [6].

# 2. Discussion

RO type indication with multiple bits

In RRC open issue discussion [1], some companies mentioned that NW can configure RO type with multiple bits indicating different load values (e.g., 100%, 80%, 50%). From our view, a SBFD-aware UE behavior would not be clear upon receiving the load values. For example, how the SBFD-aware UE selects the portion of the SBFD ROs among the configured all ROs according to the load value. RAN2 needs to discuss further additional selection rule if the load values are introduced. Without this optimization, however, current specified 1 bit indication and measured RSRP UE behaviours are enough to select RO type.

Proposal 1. Do not support to configure multiple values to select RO type.

RO type indication via LTM MAC CE

In RRC open issue discussion [1], RAN2 discussed whether to support PRACH transmission using SBFD symbols with LTM cell switch procedure. From our view, the benefit of using SBFD symbol is clear that the SBFD-aware UE can reduce latency of LTM procedure using SBFD symbols during RACH procedure. Moreover, for implementing, there may be negligible RAN2 specificaiton impact on LTM cell switch command MAC CE (e.g., adding RO type indicaiton).

Proposal 2. Support PRACH transmission using SBFD symbols based on RO type indication in the LTM procedure.

Switching to legacy-ROs with transmission power

In RAN2#127bis meeting, following agreement was made [2]:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

In RAN2#129bis meeting, following agreement was made [3]:

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

In RAN1#119 meeting, following agreement was made [4]:

Agreement For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

In RAN2#127bis meeting [2], RAN2 agreed that a SBFD-aware UE is allowed to switch from SBFD-ROs (e.g., additional-ROs) to legacy-ROs after certain number of re-attempts (i.e., if the UE firstly selected SBFD-ROs). Also, RAN2 agreed [3] that the SBFD-aware UE to switch form legacy-ROs to SBFD-ROs after the number of re-attempts.

According to the legacy RACH procedure, upon detection of initial transmission failure (e.g., no reception of RA response (RAR)), a SBFD-aware UE can set increased (ramped-up) transmission power with ramping step then will perform re-attempt with the increased power. According to the power ramping step with re-attempts, the SBFD-aware UE may reach the maximum transmission power (Pcmax) of PRACH while not yet reaching the configured max number of re-attempts. Then, there is a remaining issue on whether the UE has to keep re-attempting using SBFD-ROs or do early fallback/switching to legacy-ROs (due to consistent failures even with the capped max Tx power Pcmax).

In our view, it is not desiable to keep performing re-attempts once reached the maximum power of PRACH using SBFD-ROs. The reason of continuous re-attempts being occurred may be because of PRACH reception failures at the network due to a congestion of using SBFD-ROs and higher cross-link-interference (CLI) level in symbols of SBFD-ROs, and/or RAR reception failures at the UE if received on SBFD symbols affected by UE-to-UE CLI, and so on. Also, a network cannot estimate when the maxmimum power of PRACH would be reached after number of re-attepts because each of the SBFD-aware UE may set different transmission power based on the measured path-loss value between UE and network.

To mitigate the excessive CLI level of the cell and achieve successful RACH procedures for UEs in the cell, once the power of PRACH transmission has been reached to the maximum transmission power, the SBFD-aware UE should immediately switch from SBFD-ROs to legacy-ROs while in the middle of performing re-attempts (i.e., even not yet expiry of the parameter of max number of re-attempts). In other words, reaching of max power of PRACH can indicate that the SBFD-aware UE already had attempted with the best possible ways utilizing SBFD-ROs, so that there is no reason to make the UE try more on SBFD-ROs, which rather increases undesired interference and overall congestion rates impacting to other UEs in the cell, without any benefits.

Proposal 3. Support to switch from SBFD-ROs to legacy-ROs upon reaching maximum power of PRACH transmission during performing re-attempts with power ramping, regardless of the remaining possible counts of the configured max number of re-attempts.

Early indication via MSG3

In RAN1#120bis meeting, the following agreement was made [5]:

Agreement For CBRA, if a SBFD-aware UE transmits Msg1 in legacy-RO, the UE follows the legacy behavior during the random access procedure. - FFS: Whether specification change is necessary

When a SBFD-aware UE transmits MSG1 with legacy RO, then the UE may transmit MSG3 upon reception of MSG2. When the UE transmit the MSG3, then the UE can indicate the support of SBFD operation via the MSG3 (i.e., early indication) during the random access procedure.

This approach can be beneficial for SBFD-aware UE in RRC-IDLE/INACTIVE states, where the UE can decide on whether to connect to the selected cell based on SBFD operation or connect to the selected cell based on non-SBFD operation. For example, in case the measured CLI (based on received SSB) is low, the UE could connect to the cell based on SBFD operation. Otherwise, if the measured CLI (based on received SSB) is high, the UE could connect to the cell based on non-SBFD operation.

In another example, the UE can transmit random access preamble in ROs that coincide with SBFD symbols (e.g., via additional-ROs). As such, upon reception of the preamble, the gNB will recognize that the UE is SBFD-aware, and that the UE is operating based on SBFD operation. So, gNB can further transmit RAR in SBFD symbols.

The SBFD-aware UE that has decided to perform RA based on legacy-ROs based on conditions of ROs selection, could indicate its support for SBFD operation later and as part of Msg3.

Proposal 4. Support early indication for SBFD-aware UE in RRC-Idle/Inactive states to indicate UE’s support of SBFD operation via MSG3.

# 3. Conclusion

Proposal 1. Do not support to configure multiple values to select RO type.

Proposal 2. Support PRACH transmission using SBFD symbols based on RO type indication in the LTM procedure.

Proposal 3. Support to switch from SBFD-ROs to legacy-ROs upon reaching maximum power of PRACH transmission during performing re-attempts with power ramping, regardless of the remaining possible counts of the configured max number of re-attempts.

Proposal 4. Support early indication for SBFD-aware UE in RRC-Idle/Inactive states to indicate UE’s support of SBFD operation via MSG3.

# 4. References

[1] Summary of [Post130][216][SBFD] Running CR for 38.331 (Huawei)

[2] Chairman’s notes, 3GPP TSG RAN WG2 #127bis.

[3] Chairman’s notes, 3GPP TSG RAN WG2 #129bis.

[4] Chairman’s notes, 3GPP TSG RAN WG1 #119.

[5] Chairman’s notes, 3GPP TSG RAN WG1 #120bis.

[6] R1-2505835, Discussion on SBFD operation and procedures, InterDigital, Inc.